3GPP TSG-RAN WG2 Meeting #128	R2-2409809

Orlando, USA, Nov 18th – 22nd, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5093 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on determining about to start sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
|  |  | In clause 5.9.4.3 and clause 5.9.4.4, NOTE in the RRC specification is not specifying that the UE may determine whether the session is ‘about to start’ from the start time indicated in the User Service Description (USD). Presently, the NOTE is only addressing the determination of whether the session is ‘ongoing’ from the start and stop time indicated in USD. To bring completeness and clarity, NOTE should be amended |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 5.9.4.3 and 5.9.4.4, NOTE is specified as “The UE may determine whether the session is ongoing or about to start from the start and stop time indicated in the User Service Description (USD), see TS 38.300 [2] or TS 23.247 [67]”. Impact analysis Impacted functionality: MBS broadcast Impacted 5G architecture options: NR SA Inter-operability: There is no inter-operability issue |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There may be ambiguity for determining about to start sessions for the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.4.3, 5.9.4.4; |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### <Start of Change>

#### 5.9.4.3	MBS frequencies of interest determination

The UE shall:

1>	consider a frequency to be part of the MBS frequencies of interest if the following conditions are met:

2>	at least one MBS session the UE is receiving or interested to receive via a broadcast MRB is ongoing or about to start; and

NOTE 1:	The UE may determine whether the session is ongoing or about to start from the start and stop time indicated in the User Service Description (USD), see TS 38.300 [2] or TS 23.247 [67].

2>	for at least one of these MBS sessions, SIB21 acquired from the PCell or a non-serving cell includes mapping between the concerned frequency and one or more MBS FSAIs indicated in the USD for this session, or for at least one of these MBS sessions, the concerned frequency is not included in SIB21 but is indicated in the USD for this session; and

NOTE 2:	The UE considers a frequency to be part of the MBS frequencies of interest even though NG-RAN may (temporarily) not employ a broadcast MRB for the concerned session, i.e., the UE does not verify if the session is indicated on MCCH.

2>	the supportedBandCombinationList the UE included in UE-NR-Capability contains at least one band combination including the concerned MBS frequency.

NOTE 3:	When evaluating which frequencies the UE is capable of receiving, the UE does not take into account whether they are currently configured as serving frequencies.

#### 5.9.4.4	MBS services of interest determination

The UE shall:

1>	consider an MBS service to be part of the MBS services of interest if the following conditions are met:

2>	the UE is receiving or interested to receive this service via a broadcast MRB; and

2>	the session of this service is ongoing or about to start; and

2>	one or more MBS FSAIs in the USD for this service is included in SIB21 acquired from the PCell for a frequency belonging to the set of MBS frequencies of interest, determined according to 5.9.4.3 or SIB21 acquired from the PCell does not provide the frequency mapping for the concerned service but that frequency is included in the USD for this service.

NOTE:	The UE may determine whether the session is ongoing or about to start from the start and stop time indicated in the User Service Description (USD), see TS 38.300 [2] or TS 23.247 [67].

#### <End of Change>