3GPP TSG-RAN WG2 Meeting #128                                    R2-2409834

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	8.1.2.2

Source: 	Fujitsu

Title: 	Discussion on LCM for UE-sided Model for Beam Management Use Case

Document for:	Discussion

Introduction

In previous meetings, RAN2 made several agreements on functionality identification [1][5] related issues and one LS [2] has been sent to RAN1 pending for response. In this contribution, we would like to give our views on the following aspects of LCM for UE-sided model for beam management use case：

- Performance monitoring.

- Model control.

- Functionality identification.

Discussion

Performance Monitoring

Performance monitoring of UE-side model for beam management has been continuously discussed in RAN1 with minor progress per meeting, the following agreements were made during Rel-19 [3][4]:

Agreement [RAN1 #117] For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report. - FFS on other contents. - The report is at least configured/triggered by NW. - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report. - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring. Agreement [RAN1 #118] For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resource for monitoring. - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resource for monitoring. - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam. - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g., whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

It can be seen from the above agreements that RAN1 has not determined whether the down-selection between Type I/II monitoring or between option 1/2 of Type I monitoring should be made, nor the details of reporting from UE to NW for option 2 monitoring. RAN1 will continue discussing the remaining issues for the next several consecutive meetings.

Observation 1 The details of UE-side BM performance monitoring are still pending RAN1 further conclusions.

RAN2 needs to wait for further RAN1 input for further actions. However, no matter if NW or UE derives the metrics and what will be reported from UE to NW, RAN2 can have some discussions on at least the mechanism to trigger the performance monitoring, and the potential signaling to complete the monitoring procedure.

Proposal 1 RAN2 starts at least the following discussions while waiting for further RAN1 input.

- Mechanism to trigger the performance monitoring procedure.

- Potential signaling to complete the performance monitoring procedure.

From RAN2 point of view, the performance monitoring procedure can be triggered either periodically or event-based, e.g., for option 1, NW can configure UE to periodically report L1-RSRP or BM index to assist NW derives the monitoring metrics, or UE can trigger the procedure for certain events such as UE moving or poor performance.

Proposal 2 The performance monitoring procedure can be triggered either periodically or event based.

For the signaling of the reporting, it is natural that L1 signaling to be used as baseline for L1-RSRP or BM index for option 1 as well as the listed alternatives in option 2. MAC/RRC signaling are also valid to be used regarding the near-real-time latency requirements of performance monitoring.

Proposal 3 L1 signaling can be used as the baseline for the reports of option1 and option2. MAC and/or RRC signaling can be further studied.

Model control within the functionality

For functionality-based LCM, it is considered the model activation is decided by the UE implementation. The model selection, (de)activation or switching are transparent to the network. Without the instruction from the network, it is possible that the UE wrongly activates a model which is not applicable to the current NW configuration or cannot meet the requirement of the network. To address this issue, NW can send instruction information on applicable configuration/additional condition/requirements for UE to determine the activated functionalities, then UE can do model control within the activated functionalities.

Proposal 4 The network sends the instruction information to UE for determining the activated functionalities and subsequent model control.

Functionality Identification

In previous RAN2 #127bis meeting, the following agreements were made for functionality LCM:

Agreements for BM

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality is signaled. FFS on the applicability reporting content.

- FFS if inference configuration can be signaled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g., explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality)

- Data collection initiation and configuration for data collection is under network control. FFS how the NW determines whether data collection should be initiated (e.g., via UE requests (UE directly or UE server)

- For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

- For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models. Can come back to this discussion later.

According to both the previous long offline email discussion [2] and the RAN1 conclusions [3][4], the contents of applicability reporting may vary from one simple indicator to a bunch of detailed information of the selected functionalities/FGs, anyway it is dependent on the configuration from network side and the format of associated IDs and if UE is allowed to finalize the functionality selection based on previous information exchange, e.g., if NW configures UE to down-select pre-defined associated IDs based on UE-side additional conditions, then UE may simply report one indicator as the format of one integer; if NW is willing to retain the decision then UE may need to report more information of the corresponding associated IDs to NW. From RAN2 point of view, the signaling design may need to wait for the potential down selection from the aforementioned options.

Proposal 5 The contents of applicability reporting are subject to NW configuration and details of associated ID.

Proposal 6 Both Associated IDs and detailed information (e.g., functionality/model meta information which is not reported via step 2) can be reported to determine applicability.

Aligned with our above proposals, we believe that the inference configuration can be signaled via either step 5 or step 3 and UE may respond accordingly. In some cases (e.g., functionality switching due to mediocre performance), UE may need to embark on the inference instantly as soon as possible, a pre-configuration of inference via step 3 is necessary for saving signaling and time overhead.

Proposal 7 The inference configuration can be signaled via either step 5 or step 3.

As for non-applicable functionality reporting, multiple options are available, e.g., UE may simply report an error code to reflect the fact that the previously selected functionality is not available anymore, or UE may proactively report the preferred new functionality to NW for further decision.

Proposal 8 For non-applicable functionality reporting, UE may simply report the fact that the current functionality is not available or proactively report more information such as preferred new functionality to NW.

Conclusion

Observation 1 The details of UE-side BM performance monitoring are still pending RAN1 further conclusions.

Proposal 1 RAN2 starts at least the following discussions while waiting for further RAN1 input.

- Mechanism to trigger the performance monitoring procedure.

- Potential signaling to complete the performance monitoring procedure.

Proposal 2 The performance monitoring procedure can be triggered either periodically or event based.

Proposal 3 L1 signaling can be used as the baseline for the reports of option1 and option2. MAC and/or RRC signaling can be further studied.

Proposal 4 The network sends the instruction information to UE for determining the activated functionalities and subsequent model control.

Proposal 5 The contents of applicability reporting are subject to NW configuration and details of associated ID.

Proposal 6 Both Associated IDs and detailed information (e.g., functionality/model meta information which is not reported via step 2) can be reported to determine applicability.

Proposal 7 The inference configuration can be signaled via either step 5 or step 3.

Proposal 8 For non-applicable functionality reporting, UE may simply report the fact that the current functionality is not available or proactively report more information such as preferred new functionality to NW.

References

[1] RAN2 127bis Chair Notes.

[2] R2-2407848 LS on applicable functionality reporting for beam management UE-sided model, 3GPP TSG RAN WG2#127	, Maastricht, Netherlands, Aug 19th – 23rd, 2024.

[3] Xiaodong's Session Notes RAN1#117 (9.1 AI&ML).

[4] Xiaodong's Session Notes RAN1#118 (9.1 AI&ML).

[5] RAN2 127 Chair Notes.