3GPP TSG-RAN WG2 Meeting #128                                                        R2-2409852

Orlando, USA, 18th – 22nd Nov, 2024

Source:	CATT

Title:	Further Discussion on the SN Gap Report

Agenda Item:	7.0.2.16

Document for:	Discussion and Decision

Introduction

In RAN2#127bis meeting, the following contribution was discussed and postponed:

R2-2408003 Clarification on the SN Gap Report CATT discussion Rel-18 NR_XR_enh-Core Proposal 1: Suggest RAN2 to discuss when setting the PDCP SN gap report whether it should clarify the discarded PDCP SDUs only refer to the PDCP SDU(s) which has not been submitted by RLC to lower layers. - LG and Ericsson think that it is already clear in the three conditions for triggering SN gap reporting - Vivo doesn’t think the CR is correct but the issue is correct and the wording in existing spec may need some clarification. Samsung thinks the last change is not correct The CR is postponed

In this contribution, this issue will be further discussed.

Discussion

The discussion is divided into two sections: the first section is aiming to clarify the issue, and the second section tries to find an acceptable solution to solve the issue.

Issue description

In subclause 5.16.1 of TS38.323, regarding to the transmission operation for SN gap report, the following was captured:

### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - the PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - the discarded PDCP SDU(s) have not been submitted by RLC to lower layers. If a PDCP SN gap report is triggered, the transmitting PDCP entity shall: - compile a PDCP SN gap report as indicated below by: - setting the FDC field to the smallest COUNT value among the COUNT values associated with the discarded PDCP SDU(s); - if more than one PDCP SDUs are discarded: - allocating a Discard Bitmap field of length in bits equal to the number of COUNT values from and not including the first discarded PDCP SDU up to and including the last discarded PDCP SDU, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first; - setting in the discard bitmap field as '0' for all PDCP SDUs that have not been discarded; - setting in the discard bitmap field as '1' for all PDCP SDUs that have been discarded; - submit the PDCP SN gap report to lower layers as specified in clause 5.2.1 for Uu interface. NOTE: It is up to UE implementation how to limit the frequency of PDCP SN gap reporting.

Based on the above description, it is clearly that the description in subclause 5.16.1 includes two parts:

- Part 1: The trigger condition for SN gap report (marked with yellow);

- Part 2: How to set the PDCP SN gap report (marked with blue).

For Part 1, when discussing the trigger condition, the SDU should satisfy the following three conditions:

- PDCP SDU(s) are discarded;

  - Discarded may cause by any of the following three cases: 1) due to its own discardTimer expiry; 2) due to the discardTimer  expiry of SDU belonging to the same PDU Set; 3) confirmed by PDCP status report.

- There is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s);

- The discarded PDCP SDU(s) have not been submitted by RLC to lower layers.

For example, in the following figure, PDCP SDU1/2/3 all belongs to PDU Set 1, the PDCP SDU1 has been delivered to MAC by RLC, PDCP SDU2 has been delivered to RLC but has not been delivered to MAC, and PDCP SDU3 is still in PDCP. At T4, the discardTimer of PDCP SDU1 expiries, assuming Pdu-SetDiscard is configured for PDU Set 1, all of PDCP SDU1/2/3 should be discarded at the same time. But considering PDCP SDU1 has been delivered to MAC, it should not trigger SN gap report. If there is other PDCP SDUs which associated COUNT larger than the COUNT of PDCP SDU3, PDCP SDU2 and PDCP SDU3 can trigger SN gap report.

Figure-1 Illustration of the SN gap report trigger

Observation 1: It is possible that more than one PDCP SDUs are discarded simultaneously, but only partial of them can trigger PDCP SN gap report according to the current specification.

In case of more than one PDCP SDUs are discarded simultaneously, but only partial of them trigger PDCP SN gap report, it should further discuss whether the COUNT of the discarded PDCP SDU which with COUNT value smaller than the COUNT value of PDCP SDU which trigger the PDCP SN gap but have been delivered to lower layer by RLC should be included in the PDCP SN gap or not. Take Figure-1 as example, what we need to discuss is that whether the COUNT of PDCP SDU1 should be included in the PDCP SN gap report. There are two options:

- Option 1: The discarded PDCP SDU in the PDCP SN gap report should not include the PDCP SDU(s) which have been delivered to lower layer by RLC.

Take Figure-1 as example, the COUNT of PDCP SDU1 should not be included in the PDCP SN gap report.

- Option 2: The discarded PDCP SDU included in the PDCP SN gap report can include the PDCP SDU(s) which have been delivered to lower layer by RLC.

Take Figure-1 as example, the COUNT of PDCP SDU1 can be included in the PDCP SN gap report.

According to the current description marked with blue (Part 2) of subclause 5.16.1, only “discarded PDCP SDU(s)” are mentioned, it is unclear whether the COUNT of PDCP SDU1 should be included or not.

Proposal 1: RAN2 to confirm which of the following two options is the correct understanding:

- Option 1: The discarded PDCP SDU included in the PDCP SN gap report should not include the PDCP SDU(s) which have been delivered to lower layer by RLC.

- Option 2: The discarded PDCP SDU included in the PDCP SN gap report can include in the PDCP SDU(s) which have been delivered to lower layer by RLC.

Potential solutions

If the answer of Proposal 1 is Option 1(COUNT of PDCP SDU1 does not need to be included in the PDCP SN gap report as in Figure-1), it should further discuss whether any specification effort is needed. There are two alternatives:

- Alternative 1: To clarify that the discarded PDCP SDU(s) does not include the PDCP SDU(s) which has been delivered to lower layer by RLC in the part marked with blue in subclause 5.16.1 of TS38.323.

- Alternative 2: No specification effort is needed, current description is enough.

Proposal 2: If Option 1 is selected in Proposal 1, suggest RAN2 to discuss whether specification modification is needed or not.

If the answer of Proposal 1 is Option 2 (COUNT of PDCP SDU1 needs to be included in the PDCP SN gap report as in Figure-1), no specification effort is needed since “discarded PDCP SDU(s)” can already include the PDCP SDU1 as specified in subcluase 5.3.

Proposal 3: If Option 2 is selected in Proposal 1, no specification effort is needed.

Conclusion

According to the analysis in section 2, it is observed that:

Observation 1: It is possible that more than one PDCP SDUs are discarded simultaneously, but only partial of them can trigger PDCP SN gap report according to the current specification.

Based on the above observation, it is proposed that:

Proposal 1: RAN2 to confirm which of the following two options is the correct understanding:

- Option 1: The discarded PDCP SDU included in the PDCP SN gap report should not include the PDCP SDU(s) which have been delivered to lower layer by RLC.

- Option 2: The discarded PDCP SDU included in the PDCP SN gap report can include in the PDCP SDU(s) which have been delivered to lower layer by RLC.

Proposal 2: If Option 1 is selected in Proposal 1, suggest RAN2 to discuss whether specification modification is needed or not.

Proposal 3: If Option 2 is selected in Proposal 1, no specification effort is needed.

Reference

- TS38.323  NR; Medium Access Control (MAC) protocol specification (Release-18)