3GPP TSG-RAN WG2 Meeting #128                                                        R2-2409861

Orlando, USA, 18th – 22nd Nov 2024

Source:	CATT

Title:	Intra-gNB Service Continuity for Multi-hop U2N Relay

Agenda Item:	8.13.4

Document for:	Discussion and Decision

Introduction

According to the WID of Rel-19 NR sidelink multi-hop relay in [1], the objective on service continuity is as below:

- Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE):

First Priority:

- Intra-gNB multi-hop indirect to direct path switching using existing framework

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

Second Priority in order of importance:

- Intra-gNB direct to multi-hop indirect path switching

- Intra-gNB single-hop indirect to multi-hop indirect path switching

The scenarios C and D are limited to path switching to a target indirect path consisting of the First Relay UE in “direct” RRC Connected mode and all the other Intermediate Relay(s) in “indirect” RRC Connected mode to the same cell.

NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this contribution, the details on intra-gNB service continuity for multi-hop U2N relay will be discussed based on Rel-17/18 procedures.

Discussion

First prority senarios

In this section, firstly, the signaling flows and the details for selected steps of scenario A and scenario B are discussed; and then, the data lossless issue related to both scenario A and B is discussed.

Signaling flow and details of Scenario A

Similar as the signaling flow of Rel-17 U2N Relay I2D path switching, the signaling flow of Scenario A is as below:

Figure-1: Procedure for L2 multi-hop U2N Remote UE intra-gNB switching indirect to direct path

1.	The Uu measurement configuration and measurement report signalling procedures are performed to evaluate both relay link measurement and Uu link measurement.

2.	The gNB decides to switch the L2 U2N Remote UE onto direct Uu path.

3.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The L2 U2N Remote UE stops User Plane and Control Plane transmission via the multi-hop Relay UEs after reception of the RRCReconfiguration message with the path switch configuration.

4.	The L2 U2N Remote UE synchronizes with the gNB and performs Random Access.

5.	The UE (i.e., L2 U2N Remote UE in previous steps) sends the RRCReconfigurationComplete message to the gNB via the direct path, using the configuration provided in the RRCReconfiguration message. From this step, the UE (i.e., L2 U2N Remote UE in previous steps) uses the RRC connection via the direct path to the gNB.

6-8.The gNB sends the RRCReconfiguration message to the First/Intermediate/Last Relay UE to release the Ingress/Egress RLC channel configuration and mapping. Details are FFS and depend on Control Plane discussion.

9.	Either the First Relay UE or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation.

10.	The data path is switched from multi-hop indirect path to direct path between the UE (i.e., previous L2 U2N Remote UE) and the gNB.

NOTE 1:	Step 10 can be executed any time after step 4.

Proposal 1: Suggest RAN2 to adopt Figure-1 as the baseline signaling flow for intra-gNB multi-hop indirect to direct path switching.

Based on the above Figure-1, most of the steps are same as the R17 I2D path switching. Hence, in the following, mainly step 1/6/7/8 will be further discussed.

Step 1

For measurement and reporting configuration, in the above procedure, the remote UE needs to receive the measurement configuration and report the measurement results to the gNB. Generally, for scenario A, the legacy RRC Reconfiguration and Measurement Report signaling procedures (i.e., gNB transmits RRC Reconfiguration with measurement and report configuration to Remote UE and Remote UE sends Measurement Report according to the measurement and report configuration by gNB) can be reused. For example, the legacy RRC Reconfiguration with measurement configuration and measure results including measObjectRelay, sl-MeasResultsCandRelay，sl-MeasResultServingRelay and etc can be considered as baseline. The same principle can also apply to scenario B/C/D, hence, the same issue will not be discussed in Scenario B/C/D again.

Proposal 2: Legacy RRC Reconfiguration with measurement and report configuration for R17/R18 U2N Relay path switching can be used as baseline for R19 multi-hop relay scenarios.

For measurement event, in Rel-17 U2N relay I2D path switching, the related measurement events are X1 and X2. In event X1 and X2, only the channel quality between the remote UE and relay UE is considered, while the other hops, such as the Uu link of relay UE is not considered in R17. For R19 scenario A, compared with legacy procedure, the source channel status may be impacted by extra links, e.g., the link between the first relay UE and the last relay UE. However, similar to the legacy rule, it is unnecessary to consider these extra links when evaluating the measurement event since there is E2E RRC connection between each relay UE and gNB. Hence, it is suggested to reuse the legacy measurement events of R17 I2D for R19 multi-hop indirect to direct path switching. Naturally, the wording of L2 U2N relay UE in event X1/X2 should be replaced by the first Relay UE.

Event X1：Serving L2 U2N Relay UE becomes worse than threshold1 and NR Cell becomes better than threshold2 Event X2：Serving L2 U2N Relay UE becomes worse than threshold

Proposal 3: The measurement event X1/X2 can be reused for intra-gNB multi-hop indirect to direct path switching except that the description on serving L2 U2N Relay UE in event X1/X2 should be revised to include the First Relay UE.

Step 6/7/8

According to the above Figure-1 in step 6/7/8, all the relay UEs in multi-hop link need to release the ingress/egress RLC channel and mapping for remote UE. This is different from the legacy scenario which with only one relay UE.

There are following two alternatives to handle this step:

- Option 1: the network send the RRCReconfiguration message to the Intermediate Relay UE(s) and the First Relay UE to release PC5 relay RLC channel configuration and bearer mapping configuration related to the L2 U2N Remote UE.

- Option 2: the parent relay UE sent the RRCReconfigurationSidelink message to child relay UE to release its PC5 relay RLC channel configuration and bearer mapping configuration related to the L2 U2N Remote UE. For example, the First Relay UE notice its neighbor Intermediate Relay UE to perform the release option.

In order to align the legacy rule and reduce the complexity, it is suggested to apply option1, by which, the network will send the RRCReconfiguration message to each relay UE to release PC5 relay RLC channel configuration and bearer mapping configuration related to the L2 U2N Remote UE, the details could be FFS and wait the progress on CP discussion.

Proposal 4: For Scenario A, the network should send the RRCReconfiguration message to all the relay UEs in the multi-hop link to release the Ingress/Egresss RLC channel configuration and mapping related to the remote UE.

Signaling flow and details of Scenario B

Similar as the signaling flow of Intra-gNB Rel-18 U2N Relay I2I path switching, the signaling flow of Scenario B is as below:

Figure-2: Procedure for L2 multi-hop U2N Remote UE intra-gNB switching from multi-hop indirect to single-hop indirect path via a target L2 U2N Relay UE in RRC_CONNECTED

1.	The L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s) and sidelink measurement between L2 U2N Remote UE and the source first relay UE, after it measures/discovers the candidate L2 U2N Relay UE(s).

2.	The gNB decides to switch the L2 U2N Remote UE to a target L2 U2N Relay UE.

3.  The gNB sends an RRCReconfiguration message to the target L2 U2N Relay UE, which includes at least the L2 U2N Remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

4. The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the target L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic, and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops User Plane and Control Plane transmission over the multi-hop relay path after reception of the RRCReconfiguration message from the gNB.

5.	The L2 U2N Remote UE establishes PC5-RRC connection with target L2 U2N Relay UE.

6.	The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the target L2 U2N Relay UE.

7-9.The gNB sends the RRCReconfiguration message to the First/Intermediate/Last Relay UE to release the Ingress/Egress RLC channel configuration and mapping. Details are FFS and depend on Contol Plane discussion.

10. Either the First Relay UE or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation.

11.	The data path is switched from source multi-hop indirect path to target L2 U2N Relay UE. This step can be any time after step 6.

Proposal 5: Suggest RAN2 to adopt Figure-2 as the baseline signaling flow for intra-gNB multi-hop indirect to single-hop indirect path switching.

Based on the above Figure-2, most of the steps are same as the legacy intra-gNB I2I path switching. Hence, in the following, mainly step 1/7/8/9 will be further discussed.

Step 1

In step 1, the remote UE needs to perform the measurement to the target relay UE. In legacy U2N relay I2I path switching, the related events are event X2/Y2/Z1, only the channel quality between the remote UE and relay UE is considered, while the other hop, such as the link of Uu link of relay UE is not considered in legacy measurement event. For R19 scenario B, compared with legacy mechanism, the source channel quality may be impacted by extra links, e.g. the links between the first relay UE and the last relay UE, however, it does not see the necessity to report more information on the source multi-hop path, since there is E2E RRC connection between each relay UE and gNB. Thus, similarly to the proposal to scenario A, it is suggested to reuse the legacy measurement events for R19 multi-hop indirect to single-hop indirect path switching. Naturally, the description on serving L2 U2N relay UE in event X2/Z1 will be revised to include the First Relay relay UE.

Event X2: Serving L2 U2N Relay UE becomes worse than threshold Event Y2: Candidate L2 U2N Relay UE becomes better than threshold Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2

Proposal 6: The measurement event X2/Y2/Z1 are reused for Intra-gNB multi-hop indirect to single-hop indirect path switch except that the description on serving L2 U2N relay UE in event X2/Z1 should be revised to include the First Relay UE in multi-hop relay scenario.

If any measurement enhancement is needed in scenario B, it should be discussed after we have a common understanding on RRC status of Intermediate UEs and how/whether the source multi-hop links are stable, which may be discussed in CP procedure.

Step 7/8/9

From the view of signaling flow, compared with the legacy procedure for R18 intra-gNB i2i path switch, the scenario B needs to notice the source First/Last/Intermediate Relay UE to release Ingress/Egress RLC channel configuration and mapping related to remote UE, the similar proposal as scenario A is proposed, the details could be FFS and wait the progress on CP discussion.

Proposal 7: For Scenario B, the network should send the RRCReconfiguration message to all the relay UEs in the multi-hop link to release the Ingress/Egresss RLC channel configuration and mapping related to the remote UE.

Data lossless in Scenario A/B

For data lossless, it is necessary to review the history of discussion and legacy procedure for R17 intra-gNB path switch with one U2N serving relay node firstly.

Figure-3 The PDCP SDU transmission before intra-gNB I2D path switch

According to the history discussion, the main issue was the potential data loss from source relay UE to network during intra-gNB I2D path switch. For example, as shown in Figure-3, before path switching, the remote UE may send the SDU 1to Relay UE and receive ACK for SDU1 from Relay UE, then the remote UE will believe that SDU 1 has been transmitted successfully. However, the relay UE may receive NACK for SDU1 from gNB, then the issue is how to keep data lossless transmission during intra-gNB I2D path switching procedure.

For DL, UE will trigger the PDCP status report after performing the PDCP re-establishment or data recovery, then, the network could retransmit the data based on the network implementation. Hence, it was agreed that there is no impact to spec in R17 path switching procedure for DL in RAN2#116e meeting.

For UL, in RAN2#116e/116bis-e meeting, it was agreed that the legacy PDCP re-establishment or data recovery in UL should be performed by the Remote UE during path switch if gNB configures it, and no spec impact for ensuring UL PDCP lossless behaviour in indirect-to-direct path switch (assume it is a corner case or can be addressed by network implementation).

RAN2#116-e agreements: Proposal 24 (modified): The legacy PDCP re-establishment or data recovery in UL should be performed by the Remote UE during path switch if gNB configures it. Proposal 25: No spec impact is required for DL lossless transmission during path switch. RAN2#116bis agreements: No spec impact for ensuring UL PDCP lossless behaviour in indirect-to-direct path switch (assume it is a corner case or can be addressed by network implementation).

Observation 1: For data lossless, there is no spec impact introduced for R17 intra-gNB I2D or D2I path switching in DL/UL.

For R19 Intra-gNB multi-hop indirect to direct path switching, multi-hop source relay nodes are introduced, the issue on data loss caused by the source multi-hop relay to network also exists. For DL, the legacy procedure based on PDCP status report could be reused to keep data lossless transmission. For UL, considering the multi-hop links could be maintained by network, and the source relay node is in “direct”/indirect RRC Connected mode and could have RRC connection with network, so it is also feasible to leave it to network implementation to keep UL data lossless transmission.

Proposal 8: For DL data lossless, the legacy procedure based on PDCP status report could be reused for scenario A/B, there is no spec impact were introduced.

Proposal 9: For UL, the data loss in multi-hop relay node is corner case during path switching, the data lossless could be guaranteed by network implementation, there is no spec impact will be introduced.

Second prority Scenarios

In this section, firstly, the signaling flows and the details for selected steps of scenario C and scenario D are discussed; and then, the data lossless issue related to both scenario C and D is discussed.

Signaling flow and details of Scenario C

Similar as the signaling flow of Rel-17 Intra-gNB U2N Relay D2I path switching, the signaling flow of Scenario C is as below:

Figure-4: Procedure for L2 multi-hop U2N Remote UE intra-gNB switching from direct to multi-hop indirect path via target Relay UEs in RRC_CONNECTED

1.	The L2 U2N Remote UE reports one or multiple candidate L2 Relay UE(s) and Uu measurements, after it measures/discovers the candidate L2 Relay UE(s).

2.	The gNB decides to switch the L2 U2N Remote UE to a target multi-hop relay path.

3-5. The gNB sends the RRCReconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE. Details could be FFS and depend on CP discussion.

6.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the First Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops User Plane and Control Plane transmission over the direct path after reception of the RRCReconfiguration message from the gNB.

7.	The L2 U2N Remote UE establishes PC5-RRC connection with target First Relay UE.

8.	The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the multi-hop relay path.

9.	The data path is switched from direct path to multi-hop indirect path between the L2 U2N Remote UE and the gNB.

NOTE 1:	Before step 4 and step 5, the network will ensure the target Intermediate UE and First Relay UE already establishing the associated PC5 link for RRCreconfiguration message transmission if needed.

Proposal 10: Suggest RAN2 to adopt Figure-4 as the baseline signaling flow for intra-gNB direct to multi-hop indirect path switching.

Based on the above Figure-4, most of the steps are same as the Intra-gNB U2N Relay D2I path switching. Hence, in the following, mainly step 1/3/4/5 will be further discussed.

Step 1

For scenario C, the WID already limit the path switching to a target multi-hop indirect path consisting of the First Relay UE in “direct” RRC Connected mode and all the other Intermediate Relay(s) in “indirect” RRC Connected mode to the same cell, which means that the network needs to ensure the target Last/Intermediate/First Relay UE will be under the control of network, then the RRCReconfiguration message could be transmitted to each relay UE in the target multi-hop path.

Maybe, UE could report more information on the neighbor multi-hop relay path, such as the collected neighbor multi-hop relay path information and whether the relay UE is in RRC connected mode or not to help network to find a suitable target path, furthermore, the final decision on selecting target path is better to be made by gNB other than UE, gNB will determine multi-hop relay path, single-hop relay path or direct path is used based on the information reported from remote UE and relay UE.

Proposal 11: UE could report the collected multi-hop relay information to network to help network select a suitable target path.

Proposal 12: The network will determine and configure the suitable target path (maybe multi-hop relay path) to UE.

In legacy R17 intra-gNB D2I path switching, the measurement event Y1/Y2 could be reused to let network understand the channel status of remote UE and determine whether a path switching is needed. Besides some potential enhancement, such as above proposal, UE reporting the neighbor multi-hop relay information to help network select the suitable path. For R19 intra-gNB direct to multi-hop indirect path switching, the legacy measurement event could be reused except the description on candidate L2 U2N Relay UE will be revised to include the candidate First Relay UE.

Event Y1: PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2) Event Y2: Candidate L2 U2N Relay UE becomes better than threshold

Proposal 13: The UE will perform the legacy measurement event Y1/Y2 except the description on candidate L2 U2N Relay UE in event Y1/Y2 should be revised to include the candidate First Relay UE.

Step 3/4/5

Different from legacy intra-gNB direct to one-hop indirect path, the target path include extra First Relay UE and Intermediate Relay UE(s). Since the First/Intermediate Relay UE already has indirect RRC connection to the network, it is natural for network to send RRC reconfiguration message as in step 4/5 in Figure-4, which include at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE. Then, the First/Intermediate Relay UE could transfer the data from remote UE to Last Relay UE. Details could be FFS and depend on CP discussion.

Proposal 14: For Scenario C, the network will send RRC reconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE.

Signaling flow and details of Scenario D

Similar as the signaling flow of Rel-18 U2N Relay I2I path switching, the signaling flow of Scenario D is as below:

Figure-5: Procedure for L2 multi-hop U2N Remote UE intra-gNB switching from single-hop indirect to multi-hop indirect path via target Relay UEs in RRC_CONNECTED

1.	The L2 U2N Remote UE reports one or multiple candidate L2 Relay UE(s) and Uu measurements, after it measures/discovers the candidate L2 Relay UE(s).

2.	The gNB decides to switch the L2 U2N Remote UE to a target multi-hop relay path.

3-5. The gNB sends the RRCReconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE. Details could be FFS and depend on CP discussion

6.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the First Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end Uu radio bearer(s). The L2 U2N Remote UE stops User Plane and Control Plane transmission over the direct path after reception of the RRCReconfiguration message from the gNB.

7.	The L2 U2N Remote UE establishes PC5-RRC connection with target First Relay UE.

8.	The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the multi-hop Relay path.

9.  The gNB sends the RRCReconfiguration message to the source L2 U2N Relay UE to reconfigure the connection between the source L2 U2N Relay UE and the gNB. The RRCReconfiguration message to the source L2 U2N Relay UE can be sent any time after step 6 based on gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

10.	Either source L2 U2N Relay UE's AS layer or L2 U2N Remote UE's AS layer indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation after step 6 or step7.

11.	The data path is switched from single-hop indirect path to multi-hop indirect path between the L2 U2N Remote UE and the gNB.

NOTE 1:	Before step 4 and step 5, the network will ensure the target Intermediate UE and First Relay UE already establishing the PC5 link if needed.

Proposal 15: Suggest RAN2 to adopt Figure-5 as the baseline signaling flow for intra-gNB direct to multi-hop indirect path switching.

Based on the above figure, most of the steps are same as the R18 Intra-gNB I2I path switching. Hence, in the following, mainly step 1/3/4/5 will be further discussed:

Step 1

For scenario D, similar to the analysis to scenario C, besides the enhanced measurement result reporting on candidate multi-hop relay path information, the legacy measurement event X2/Y2/Z1, as below could be reused. The term of candidate L2 U2N Relay should be revised to include the candidate First Relay UE.

Event X2: Serving L2 U2N Relay UE becomes worse than threshold Event Y2: Candidate L2 U2N Relay UE becomes better than threshold Event Z1: Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2

Proposal 16: The UE will perform the legacy measurement event X2/Y2/Z1 except the description on candidate L2 U2N Relay UE in event Y2/Z1 should be revised to include the candidate First Relay UE.

Step 3/4/5

Different from legacy intra-gNB one-hop indirect to one-hop indirect path, in scenario D, the target path include one extra First Relay UE and Intermediate Relay UE. Since the First/Intermediate Relay UE already has indirect RRC connection to the network, similar to scenario C, it is natural for network to send RRC reconfiguration message in step 3/4/5, which include at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE. Then, the First/Intermediate Relay UE could transfer the data from the remote UE to the Last Relay UE. Details could be FFS and depend on CP discussion.

Proposal 17: For Scenario D, the network will send RRC reconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE.

Data lossless in Scenario C/D

In scenario C, there is no relay node in the source path, so there is no extra data lossless issue during path switching. The data lossless could be guaranteed by legacy procedure.

In scenario D, there is one U2N relay node in the source path, so the data lossless issue caused by this relay has been discussed and guaranteed by the legacy procedure.

Proposal 18: For Scenario C/D, the legacy procedure will be reused to keep DL/UL lossless delivery during the path switch, there is no further impact to specification.

Others

In R17/R18 relay discussion, it didn’t support the CHO and DAPS procedure, the same rule could be applied to R19 multi-hop relay scenarios to reduce the workload in the limited time scheduling.

Proposal 19: CHO-like and DAPS-like path switch procedure is not considered in R19 multi-hop relay scenarios.

In legacy discussion on relay scenarios, RAN2 agreed that S-measure criteria is not used by the Remote UE for direct-indirect path switch at RAN2#115-e meeting, and the S-measure criterion based on SL/SD-RSRP of serving relay during indirect to direct path switching is not introduced at RAN2#116bis-e meeting. Thus, similarly to the analysis in history discussion, we do not see the significant benefits to introduce S-measure criteria for R19 multi-hop relay scenarios, so the S-measure criteria is not used in R19 multi-hop relay scenarios.

Proposal 20: S-measure criterion is not considered in R19 multi-hop relay scenarios.

Conclusion

According to the analysis in section 2, it is proposed:

For scenario A

Proposal 1: Suggest RAN2 to adopt Figure-1 as the baseline signaling flow for intra-gNB multi-hop indirect to direct path switching.

Proposal 2: Legacy RRC Reconfiguration with measurement and report configuration for R17/R18 U2N Relay path switching can be used as baseline for R19 multi-hop relay scenarios.

Proposal 3: The measurement event X1/X2 can be reused for intra-gNB multi-hop indirect to direct path switching except that the description on serving L2 U2N Relay UE in event X1/X2 should be revised to include the First Relay UE.

Proposal 4: For Scenario A, the network should send the RRCReconfiguration message to all the relay UEs in the multi-hop link to release the Ingress/Egresss RLC channel configuration and mapping related to the remote UE.

For scenario B

Proposal 5: Suggest RAN2 to adopt Figure-2 as the baseline signaling flow for intra-gNB multi-hop indirect to single-hop indirect path switching.

Proposal 6: The measurement event X2/Y2/Z1 are reused for Intra-gNB multi-hop indirect to single-hop indirect path switch except that the description on serving L2 U2N relay UE in event X2/Z1 should be revised to include the First Relay UE in multi-hop relay scenario.

Proposal 7: For Scenario B, the network should send the RRCReconfiguration message to all the relay UEs in the multi-hop link to release the Ingress/Egresss RLC channel configuration and mapping related to the remote UE.

Data Lossless for scenario A/B

Observation 1: For data lossless, there is no spec impact introduced for R17 intra-gNB I2D or D2I path switching in DL/UL.

Proposal 8: For DL data lossless, the legacy procedure based on PDCP status report could be reused for scenario A/B, there is no spec impact were introduced.

Proposal 9: For UL, the data loss in multi-hop relay node is corner case during path switching, the data lossless could be guaranteed by network implementation, there is no spec impact will be introduced.

For scenario C

Proposal 10: Suggest RAN2 to adopt Figure-4 as the baseline signaling flow for intra-gNB direct to multi-hop indirect path switching.

Proposal 11: UE could report the collected multi-hop relay information to network to help network select a suitable target path.

Proposal 12: The network will determine and configure the suitable target path (maybe multi-hop relay path) to UE.

Proposal 13: The UE will perform the legacy measurement event Y1/Y2 except the description on candidate L2 U2N Relay UE in event Y1/Y2 should be revised to include the candidate First Relay UE.

Proposal 14: For Scenario C, the network will send RRC reconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE.

For scenario D

Proposal 15: Suggest RAN2 to adopt Figure-5 as the baseline signaling flow for intra-gNB direct to multi-hop indirect path switching.

Proposal 16: The UE will perform the legacy measurement event X2/Y2/Z1 except the description on candidate L2 U2N Relay UE in event Y2/Z1 should be revised to include the candidate First Relay UE.

Proposal 17: For Scenario D, the network will send RRC reconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE.

Data Lossless for scenario C/D

Proposal 18: For Scenario C/D, the legacy procedure will be reused to keep DL/UL lossless delivery during the path switch, there is no further impact to specification.

Others

Proposal 19: CHO-like and DAPS-like path switch procedure is not considered in R19 multi-hop relay scenarios.

Proposal 20: S-measure criterion is not considered in R19 multi-hop relay scenarios.

Reference

- RP-242349 New WID on NR sidelink multi-hop relay