3GPP TSG-RAN WG2 Meeting #128	R2-2409899

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	8.2.5

Source:	Huawei, HiSilicon

Title:	A-IoT Topology 2 specific aspects

Document for: Discussion and Decision

# 1	Introduction

In RAN2 #127 and 127bis meetings, RAN2 discussed the Topology 2 issues and achieved the following agreements.

Agreements for Topology 2 in RAN2 #127 meeting

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

- A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

- RAN2 assumes that intermediate UE authorization is performed by upper layers.  RAN2 impact can be studied after further SA/RAN3 progress.

- RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

- NW is in control of resources to be used by AIoT air interface.

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

Agreements for Topology 2 in RAN2 #127bis meeting

- Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

- The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

- The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

The above agreements already cover all the stage-2 level key RAN2 aspects for Topology 2, and in this contribution, we further discuss the detailed FFS aspects.

# 2	Discussion

## 2.1	Architecture options and E2E protocol stack

SA2 and RAN3 have identified three architecture options, i.e. NAS based solution, PDU/UP based solution and RRC based solution. RAN2 agreed to study the RAN2 impact for the SA2 identified options. And in the previous meeting, the discussion and conclusions intended to cover the three options, in terms of Topology 2 general principle, UE reader authorization, radio resource configuration, RRC states, temporary out-of-connection (i.e. RLF and HO).

Observation 1:	The RAN2 study on Topology 2 has covered the key issues, including Topology 2 general principle, UE reader authorization, radio resource configuration, RRC states, temporary out-of-connection (i.e. RLF and HO), for all the SA2 identified architecture options, i.e. NAS based solution, PDU/UP based solution and RRC based solution.

It has been observed that RAN3 has agreed and captured the general descriptions and call flows of the three options in the TR, including the Uu data forwarding details. Furthermore, the protocol stack of RRC based solution has been captured as well, and RAN3 may do the same for the other two options. In addition, for reader selection in Topology 2, RAN3 also conducted quite a lot of discussion and added an editor note “Editor’s Note 5:	In Topology 2, it is FFS on reader selection, may need coordination between A-IoT RAN node and A-IoT CN.” in the TP. Based on the RAN3 progress, there is no further RAN2 work in study phase for protocol stack, stage-2 call flow, Uu data forwarding and UE reader selection in Topology 2. If there is further RAN2 impact, it can be done in the normative phase.

Observation 2:	The RAN3 study on Topology 2 covers protocol stack, stage-2 call flows, Uu data forwarding and UE reader selection for all the SA2 identified architecture options, and RAN2 has no additional work for those aspects in SI phase.

Then in Oct. meeting, SA2 started conclusion discussion, and achieved interim conclusion on Topology 2 architecture options as below. They will continue the discussion in November meeting which is also the last meeting of SA2 SI phase.

Two options will be specified for Topology 2: - User-plane option as defined in clause 8.1.3.2 - RRC-based option as defined in clause 8.1.3.3 Editor’s note: Whether a NAS-based option is needed in addition is FFS.

From RAN2 perspective, there is no need to down select the SA2 identified options, which should be a RAN plenary level discussion and also requires coordination with SA2.

Proposal 1a:	There is no need for RAN2 to further down select the SA2 identified architecture options of Topology 2 in SI phase, with the understanding that all three options are feasible from RAN2 perspective.

Proposal 1b:	The study on the RAN2 aspects for topology 2 is completed from RAN2 perspective for the SA2 identified architecture options.

## 2.2	Resource allocation and validity

According to the online discussion, the common understanding among companies is that there are several phases during an inventory/command service from AS perspective.

- Preparation phase: before a UE is selected as a UE reader, the UE can be in idle/inactive/connected state.

- UE reader selection phase: once an idle/inactive UE is selected as a UE reader, it should be paged and enter connected state.

- Service initiation phase: the UE receives service request and corresponding dedicated radio resource allocated by gNB when the UE is in connected state.

- Service execution phase: the connected UE performs A-IoT R2D transmission to devices and A-IoT D2R reception from devices, using the dedicated radio resource. And then the UE reports inventory results to CN and receives subsequent R2D command from the CN if any. The further question is whether the UE can be released to idle/inactive state and how to determine if the radio resource is valid or not.

The discussion on resource validity is mainly about when/where the UE can consider the resource is valid.

For the validity area, there is no doubt that the dedicated radio resource is valid in the (serving) cell from which the UE receives the configuration. However, to enable validity across multiple cells, the cross-cell/BS coordination is required. From network perspective, the coordination is complicated, and the efficiency of radio resource usage is reduced since radio resource is reserved/wasted just for a minor case, especially considering that in Rel-19 only macro scenario is supported for Topology 2, which means the coverage of one cell is quite large.

Observation 3:	In order to support the validity across multiple cells, the A-IoT radio resources will be reserved/wasted for a minor and infrequent case, and the cross-cell/BS coordination is complicated.

Proposal 2:	Validity of dedicated A-IoT radio resource across multiple cells is not supported in this release.

For the validity time, the current NR/LTE Uu logic should apply, i.e. that the resource is valid after the resource is configured and becomes invalid if the configuration is released by BS for connected state. This could also be applicable to inactive state, HO and to the duration between RLF detection and successful reestablishment:

- once the UE is released to inactive state, it can still use configured radio resource unless the network explicitly releases the configuration;

- for RLF handling, once the UE detects RLF and initiates the RRC reestablishment in the same serving cell, it can continue using the radio resource until the RRC reestablishment procedure succeeds (after that, network can decide to reconfigure afterwards as agreed in last meeting) or fails (after that, the UE falls back to idle state and releases all the AS configuration/context as legacy);

- for HO, if the target BS includes radio resource configuration in the HO command, the UE can switch to the new radio resource when accessing the target cell.

In this way, the UE behaviour is the same in connected state (including HO and RLF recovery) and inactive state, i.e. the UE is allowed to use the radio resource as long as the resource is configured and not released by BS no matter if the UE is in connected state or inactive state. Compared to the timer-based validity criteria, this configuration-based method has less specification impact, less complexity for both of the network side and the UE side. For instance, from network side, it’s difficult to determine a proper value for an ongoing service. If the BS provides a smaller timer, the UE may need to enter connected state to request more resource, which would interrupt the A-IoT operation, while if the network provides a larger timer, the extra radio resource is wasted and the performance is the same as configuration-based method, which makes the timer not useful at all. This is why a similar timer-based solution has been discussed for NCR and inactive positioning, but not agreed in the end. In this case, we think configuration-based method is sufficient, and timer-based validity criteria is not needed.

Observation 4:	Timer-based validity criteria introduces additional specification impact, with less flexibility than normal RRC reconfiguration based method.

Proposal 3:	In this release, the A-IoT radio resource is valid in the UE reader’s serving cell after the resource is configured by the BS, and becomes invalid if the configuration is released by the BS, which is applicable to:

- Connected state. As to the particular cases:

- RLF: the radio resource is valid during the UE RRC reestablishment procedure in the same serving cell;

- HO: the radio resource configured in HO command is valid when the UE is accessing the target cell.

- Inactive state (if to be supported): the radio resource is valid in the serving cell unless/until it is released by BS explicitly.

For idle state, there seems to be two motivations/target scenarios. One scenario is coverage hole, for example, the UE may experience RLF and cannot find a suitable/acceptable cell to initiate the RRC reestablishment. However, in this scenario, the T311 is going to expire, which is not a temporary out-of-connection condition, and may lead to NAS state change and NAS procedure for registration. Thus, keeping the AS radio resource in such scenario does not bring clear benefit. Another motivation is to save power, which is the same motivation to support inactive state. But supporting inactive state is much easier than idle state, so for the power saving purpose, inactive state should be sufficient. For instance, in current specification, when a UE is released to idle state, the UE shall release all the AS configuration and resources. The BS will also release all the UE context, including the AS configuration as well as the UE specific association between BS and CN. If to support A-IoT radio resource validity for idle mode, the BS needs to maintain the UE context and at least radio resource configuration which is the equivalent workload compared to supporting inactive state. However, when the UE needs to report the inventory results, in order to establish AS RRC connection and CN CP/UP path, the UE still has to perform the whole RRC setup procedure and NAS registration procedure, which brings much more AS and CN signalling overhead than inactive state.

Observation 5:	Compared to inactive state, it’s more difficult to support idle without clear benefit than inactive state.

Another point for radio resource allocation is how to let BS know that radio resources are needed by UE reader. For RRC based solution, since gNB is aware of the service request sent by CN, the gNB can configure the radio resource directly to the UE. On the other hand, for UP/NAS based solution, the service request is considered as UE’s own data/NAS message, which is transparent to the BS, thus further request/indication to the BS is needed. The possible ways could be based on UE request or CN indication, which has been discussed by RAN3 and the following description has been captured in TR by RAN3.

#### 6.5.1.3 AIoT radio resource allocation in case of NAS/UP based solutions In NAS/UP based solutions, AIoT radio resources can be requested in advance to the NAS/UP based communication with the AIoT device or can be requested along with the NAS/UP based communication. There are different ways to trigger AIoT radio resource allocation, e.g. upon CN request or or upon UE request, etc. Some aspects of AIoT radio allocation may be preconfigured by OAM.

The UE request based way has some RAN2 impacts to define new Uu signalling, while CN indication based way may have no RAN2 impact. Since which way to go is also pending to the final decision on the SA2 architecture option, it can be further discussed in normative work.

Proposal 4a:	For UP/NAS based solution, the BS can allocate radio resources to UE readers based on UE request or indication from CN, which is subject to final decision on the architecture option (left to normative work).

Proposal 4b:	For RRC based solution, BS can directly configure the dedicated radio resources based on service request from CN.

## 2.3	UE reader selection

SA2 and RAN3 are discussing UE reader selection, i.e. one/more UEs are selected as reader to receive the service request and perform the A-IoT AS procedure. Based on the reading of the SA2 TR 23.700-013, almost in all the solutions it’s AIoTMF to select the reader based on area/location information. On top of that, RAN3 is discussing whether gNB can further do a filter on the UE readers provided by CN and captured the following agreement in TR.

In Topology 2, the RAN architecture should enable the coordination of the usage of the A-IoT radio resources among readers.

Another issue is that if an idle/inactive UE is selected, network should trigger the UE to move to connected state. But we understand the current paging procedure is sufficient. So from RAN2 perspective, we do not identify any big impact for now, except the UE capability reporting, which can be considered as a business as usual, so there is no need to discuss this in SI.

Observation 6:	If idle/inactive UE is selected by the CN or RAN as the reader, the UE can be paged by the legacy paging procedure (no immediate RAN2 impact foreseen).

Proposal 5:	UE reader selection is up to SA2 and RAN3 discussion, and RAN2 impact can be discussed after further SA2/RAN3 progress, if any.

## 2.4	Others

There might be some other stage-3 issues specific to Topology 2, which can be considered in WI phase.

Observation 7:	Following UE reader operation details and optimization can be considered as stage-3 phase issues (no impact on the feasibility):

- Network may need to indicate the support for UE reader (for access control);

- UE reader may need to indicate its intention to operate as intermediate UE reader to the network, when entering the RRC_CONNECTED state;

- UE reader specific NR Uu capability signalling, if any;

- The assistance information to reader for topology 1 is also applicable to the UE reader for topology 2.

# 3	Conclusion

This contribution makes the following observations and proposals:

Observation 1:	The RAN2 study on Topology 2 has covered the key issues, including Topology 2 general principle, UE reader authorization, radio resource configuration, RRC states, temporary out-of-connection (i.e. RLF and HO), for all the SA2 identified architecture options, i.e. NAS based solution, PDU/UP based solution and RRC based solution.

Observation 2:	The RAN3 study on Topology 2 covers protocol stack, stage-2 call flows, Uu data forwarding and UE reader selection for all the SA2 identified architecture options, and RAN2 has no additional work for those aspects in SI phase.

Observation 3:	In order to support the validity across multiple cells, the A-IoT radio resources will be reserved/wasted for a minor and infrequent case, and the cross-cell/BS coordination is complicated.

Observation 4:	Timer-based validity criteria introduces additional specification impact, with less flexibility than normal RRC reconfiguration based method.

Observation 5:	Compared to inactive state, it’s more difficult to support idle without clear benefit than inactive state.

Observation 6:	If idle/inactive UE is selected by the CN or RAN as the reader, the UE can be paged by the legacy paging procedure (no immediate RAN2 impact foreseen).

Observation 7:	Following UE reader operation details and optimization can be considered as stage-3 phase issues (no impact on the feasibility):

- Network may need to indicate the support for UE reader (for access control);

- UE reader may need to indicate its intention to operate as intermediate UE reader to the network, when entering the RRC_CONNECTED state;

- UE reader specific NR Uu capability signalling, if any;

- The assistance information to reader for topology 1 is also applicable to the UE reader for topology 2.

Architecture options and E2E protocol stack

Proposal 1a:	There is no need for RAN2 to further down select the SA2 identified architecture options of Topology 2 in SI phase, with the understanding that all three options are feasible from RAN2 perspective.

Proposal 1b:	The study on the RAN2 aspects for topology 2 is completed from RAN2 perspective for the SA2 identified architecture options.

Resource allocation and validity

Proposal 2:	Validity of dedicated A-IoT radio resource across multiple cells is not supported in this release.

Proposal 3:	In this release, the A-IoT radio resource is valid in the UE reader’s serving cell after the resource is configured by the BS, and becomes invalid if the configuration is released by the BS, which is applicable to:

- Connected state. As to the particular cases:

- RLF: the radio resource is valid during the UE RRC reestablishment procedure in the same serving cell;

- HO: the radio resource configured in HO command is valid when the UE is accessing the target cell.

- Inactive state (if to be supported): the radio resource is valid in the serving cell unless/until it is released by BS explicitly.

Proposal 4a:	For UP/NAS based solution, the BS can allocate radio resources to UE readers based on UE request or indication from CN, which is subject to final decision on the architecture option (left to normative work).

Proposal 4b:	For RRC based solution, BS can directly configure the dedicated radio resources based on service request from CN.

UE reader selection

Proposal 5:	UE reader selection is up to SA2 and RAN3 discussion, and RAN2 impact can be discussed after further SA2/RAN3 progress, if any.