3GPP TSG-RAN WG2 Meeting #128		     R2-2409908

Orlando, US: 18th November – 22nd November 2024

Source:	Qualcomm Incorporated

Title:	On LCM for UE-sided Models for Beam Management

Agenda Item:	8.1.2.2 LCM for UE-sided model for Beam Management use case

Document for:	Discussion and Decision

# Introduction & Background

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured in the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, RAN #102, a new work item on artificial intelligence/machine learning for NR air interface has been approved in [3].

This paper discusses the “LCM of UE-sided models for beam management use case” considering recent RAN1/RAN2 agreements.

# Discussion on LCM of UE-sided Models for Beam Management

## 2.1 Signaling of Associated ID and RS Configuration

In RAN2#127 [4], RAN2 agreed on the following agreement for determining the applicable functionalities at the UE,

Agreements on procedures - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

In the RAN1#118 meeting [5], RAN1 agreed to support the associated ID for the UE-side model in beam management as captured below

Agreement For UE sided model in beam management, support associated ID

Note that the above RAN1 / RAN2 agreements ensure consistency during model training and inference, i.e. the during the inference AI/ML models are used for the network side additional condition(s) for which they are trained. Therefore, the associated ID and RS configuration should be configured/signaled during both “UE-side data collection” and “AI/ML inference”.

In the RAN2#127bis [6], RAN2 further agree the below

Agreements for BM

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

RAN2 agreed on the above with the understanding that

- It is up to the NG-RAN when it starts providing the training RS configuration and associated ID,

  - NG-RAN may determine when to start providing training RS configuration and associated ID based on UE request (UE directly or UE server)

- NG-RAN determines the training RS configuration and associated ID.

Observation 1: In the RAN2#127bis meeting [6], the RAN2 agreement “Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)” was based on the following understanding

- It is up to the NG-RAN when it starts providing the training RS configuration and associated ID,

  - NG-RAN may determine when to start providing training RS configuration and associated ID based on UE request (UE directly or UE server)

- NG-RAN determines the training RS configuration and associated ID.

Note that for the UE-side data collection, the triggers/conditions for data collection at the UE may depend on available memory, power, computation, and others. Therefore, the UE can be the only entity that can determine triggers/conditions for UE-side data collection. As the UE can be the only entity that can determine the appropriate trigger/condition for data collection, irrespective of the UE-side data collection solution under discussion, the procedure for the UE to request the training RS configuration and associated ID(s) should be supported.

Observation 2: UE is the only entity that can determine if the conditions/triggers for the UE-sided data collection have been met, for example,

- If the UE has sufficient memory to store the measurements based on the training RS configuration and the associated ID provided by the NG-RAN together with any additional UE-specific measurements,

- Whether the UE has sufficient power to perform measurements,

- Whether the UE has sufficient computation for any desired filtering, and

- Others.

Proposal 1: Regardless of the UE-side data collection solutions under discussion, a procedure for UE to request the training RS configuration and associated ID(s) should be supported.

During the RAN2#127bis meeting, some infra-vendors argued that they do not prefer to randomly receive the request for training RS configuration and associated ID(s). To avoid unnecessary requests from the UE, there can be an indication from the NG-RAN on when the UE can request training RS configuration and associated ID(s).

Proposal 2: To avoid unnecessary requests from the UE for training RS configuration and associated ID(s), the NG-RAN can indicate in the SIB that it is ready to transmit training RS and associated ID(s). Upon such an indication from the NG-RAN, the UE interested in the training data collection can request training RS configuration and associated IDs using UAI.

### 2.1.2 Indicating non-applicable CSI-report Configuration (a.k.a. non-applicable functionality)

In the RAN1#118bis meeting [7], RAN1 discussed the following options applicable functionality(ies) and inference configuration signaling

Agreement RAN 1 further study the following options for applicability for inference for UE-side model: Option 1: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed, Option 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters. - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated. - In Step 5, NW configures configuration(s) for CSI report for inference Option 3: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter. - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters. - In Step 5, NW configures configuration(s) for CSI report for inference. Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration.

While it is still FFS (in RAN1) whether UE is provided with CSI-report configuration or inference related parameters, it is clear that CSI-report configuration(s) are configured at the UE for providing inference results.

Observation 3: While it is still FFS (in RAN1) whether UE is provided with CSI-report configuration or inference-related parameters for determining applicability, it is clear that CSI-report configuration(s) are configured at the UE for reporting inference results.

RAN1 further agreed to support multiple CSI reports, e.g., periodic CSI reports, aperiodic CSI reports, and semi-persistent CSI reports.

Agreement For UE-side model, for beam management, for inference report, support periodic CSI report, aperiodic CSI report, and semi-persistent CSI report. Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

Observation 4: in RAN1#118bis meeting [7], RAN1 agreed to support configuration, activation, or trigger of multiple CSI reports for inference for UE-side models.

In RAN2#127bis meeting, RAN2 agreed the below:

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

As RAN1 agreed to support the configuration, activation, or trigger of multiple CSI reports for inference for UE-side models, there may exist a scenario where the configured and applicable CSI report configuration(s) becomes non-applicable. Note that when a configured CSI report configuration becomes non-applicable, it cannot be activated. Therefore, the network should be aware when a configured applicable CSI-report configuration becomes non-applicable.

Proposal 3: When applicable CSI report configuration(s) (both activated or deactivated [or not activated]) becomes non-applicable, the UE should report CSI report configuration ID(s) using UAI. The network shall not activate non-applicable CSI report configuration(s) (deactivated [or not activated]). The network shall release/deactivate non-applicable CSI report configuration(s) (activated).

### 2.1.3 Uniqueness of Associated IDs within A PLMN

In the RAN1#117 meeting [8], RAN1 had the following working assumption

gr

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study).

Observation 5: RAN1 assumed that NW-side additional conditions with the same associated ID are consistent at least within a cell.

While consistency within a cell is useful for UE-side for model development and runtime LCM (e.g., model selection/switching/activation/deactivation), it is not scalable. If the associated ID at different cells represents different network-side additional conditions, then it will be hard for the UE side to generalize the model across multiple cells. This poses challenges not only on the UE-side model development but also on the runtime LCM. Without sufficient generalization, the UE side has to download/get the model frequently causing unnecessary interruptions. Therefore, the uniqueness of associated IDs extended to the PLMN. How the uniqueness of the associated IDs within a PLMN is achieved can be left up to the OAM implementation.

Observation 6: The uniqueness of associated ID within a single cell is not scalable, as it will create challenges for the UE side to generalize models across multiple cells.

Observation 7: The lack of uniqueness of the associated ID poses challenges not only on the UE-side model development but also on the runtime LCM.

Observation 8: Without sufficient generalization, the UE side has to download/get the model frequently causing unnecessary interruptions.

Proposal 4: NW-side additional conditions with the same associated ID are consistent within a PLMN.

Proposal 5: How the uniqueness of the associated IDs within a PLMN is achieved can be left up to the OAM implementation.

# Conclusion

Observation 1: In the RAN2#127bis meeting [6], the RAN2 agreement “Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)” was based on the following understanding

- It is up to the NG-RAN when it starts providing the training RS configuration and associated ID,

  - NG-RAN may determine when to start providing training RS configuration and associated ID based on UE request (UE directly or UE server)

- NG-RAN determines the training RS configuration and associated ID.

Observation 2: UE is the only entity that can determine if the conditions/triggers for the UE-sided data collection have been met, for example,

- If the UE has sufficient memory to store the measurements based on training RS configuration and the associated ID provided by the NG-RAN together with any additional UE-specific measurements,

- Whether the UE has sufficient power to perform measurements,

- Whether the UE has sufficient computation for any desired filtering, and

- Others.

Proposal 1: Regardless of the UE-side data collection solutions under discussion, a procedure for UE to request the training RS configuration and associated ID(s) should be supported.

Proposal 2: To avoid unnecessary requests from the UE for training RS configuration and associated ID(s), the NG-RAN can indicate in the SIB that it is ready to transmit training RS and associated ID(s). Upon such an indication from the NG-RAN, the UE interested in the training data collection can request training RS configuration and associated IDs using UAI.

Observation 3: While it is still FFS (in RAN1) whether UE is provided with CSI-report configuration or inference-related parameters for determining applicability, it is clear that CSI-report configuration(s) are configured at the UE for reporting inference results.

Observation 4: in RAN1#118bis meeting [7], RAN1 agreed to support configuration, activation, or trigger of multiple CSI reports for inference for UE-side models.

Proposal 3: When applicable CSI report configuration(s) (both activated or deactivated [or not activated]) becomes non-applicable, the UE should report CSI report configuration ID(s) using UAI. The network shall not activate non-applicable CSI report configuration(s) (deactivated [or not activated]). The network shall release/deactivate non-applicable CSI report configuration(s) (activated).

Observation 5: RAN1 assumed that NW-side additional conditions with the same associated ID are consistent at least within a cell.

Observation 6: The uniqueness of associated ID within a single cell is not scalable, as it will create challenges for the UE side to generalize models across multiple cells.

Observation 7: The lack of uniqueness of the associated ID poses challenges not only on the UE-side model development but also on the runtime LCM.

Observation 8: Without sufficient generalization, the UE side has to download/get the model frequently causing unnecessary interruptions.

Proposal 4: NW-side additional conditions with the same associated ID are consistent within a PLMN.

Proposal 5: How the uniqueness of the associated IDs within a PLMN is achieved can be left up to the OAM implementation.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, Dec. 2023

- RAN2#127 meeting report

- RAN1#118 meeting report

- RAN2#127bis meeting report

- RAN1#118bis meeting report

- RAN1#117 meeting report