3GPP TSG-RAN2 Meeting #128	R2- 2409911

# Orlando, United states, Nov 18th – 22nd, 2024

Agenda item:		8.7.4.2 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Discussion on DSR enhancement for XR

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 XR[1], it is agreed to specify the DSR enhancements with multiple pairs of remaining time and buffer size for an LCG.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This document continues discussion on further details of DSR enhancement for XR.

2.	Discussion

Configuration of reporting thresholds and triggering threshold

In RAN2#127 meeting [2], it is agreed that multiple reporting threshold(s) can be configured for enhanced DSR, while one triggering threshold is remained as in Rel-18 DSR. Further, in RAN2#127bis meeting [3], it was agreed to further discuss on whether the triggering threshold can be lower than largest reporting threshold of DSR procedure, considering the UE complexity on triggering, calculation, and cancellation of enhanced DSR.

We do not change the definition of delay-critical data Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc.

In order to investigate whether there is any additional complexity due to configuring the triggering threshold lower than maximum reporting threshold, the detailed enhanced DSR procedure of following cases should be reviewed:

- Case 1: DSR triggering threshold is the largest value of the reporting threshold, in order to provide the finer granularity of delay status information when the triggering time point of DSR and the transmission time point of DSR are different.

- Case 2: DSR triggering threshold is the smallest value of the reporting threshold, in order to report the data volume which can possibly trigger the additional DSR.

For enhanced DSR triggering procedure, there is no difference between Case 1 and Case 2, since the DSR is triggered when there is any PDCP SDU whose remaining time until for PDCP discardTimers expiry is lower than the triggering threshold for a LCH, regardless of the reporting threshold(s).

For enhanced DSR cancellation, same principle can be applied as in Rel-18 DSR procedure, i.e., DSR is cancelled when all the PDCP SDU associated with the DSR is discarded or reported. Here, the PDCP SDU associated with DSR is determined based on the triggering threshold of enhanced DSR, so there would be no difference on whether the reporting threshold(s) are larger than the triggering threshold or not.

For data volume calculation of enhanced DSR MAC CE, the same principle can be applied for both cases, i.e., data volume is calculated based on its PDCP discard timer value and reporting threshold(s), regardless of relation with triggering threshold of DSR. For discussion purpose, the PDCP/RLC data whose remaining time till PDCP discardTimer expiry is less than the reporting threshold is defined as ‘delay-reporting data,’

Then, the content of enhanced DSR MAC CE would be determined in PDCP/RLC layer based on delay-reporting data volume associated with reporting thresholds. The only difference for Case 1 and Case 2 for enhanced DSR is as follows:

- For Case 1, all the delay-reporting data in enhanced DSR MAC CE is delay-critical data volume with finer granularity.

- For Case 2, some of delay-reporting data in enhanced DSR MAC CE may be non-delay critical data, and enhanced DSR MAC CE includes further delay information in addition to the delay-critical data information.

In summary, there is no critical difference on triggering, cancellation, and data volume calculation of enhanced DSR, based on whether the triggering threshold is less than the largest value of reporting thresholds. Therefore, it would be better to allow network flexibility to configure delay thresholds considering both cases, given that DSR triggering threshold and DSR reporting thresholds would be separately configured by network.

Proposal 1. No need to specify constraints between DSR triggering threshold and reporting thresholds.

Enhanced DSR MAC CE vs. Rel-18 DSR MAC CE

In RAN2#127bis meeting [3], it was remained as FFS on whether the Rel-18 DSR MAC CE can be transmitted when there is at least one LCG configured with multiple reporting time thresholds, considering that Rel-18 DSR MAC CE has smaller size than the enhanced DSR MAC CE.

FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

Meanwhile, on the details of enhanced DSR MAC CE, it is agreed to include each portion of remaining time only if the amount of buffered data is larger than 0, in order to reduce the overhead of enhanced DSR MAC CE.

- For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion.

In other words, if the amount of buffered data for a portion is zero, corresponding buffer size field and the remaining time field would not be included in the enhanced DSR MAC CE.

During RAN2#127bis meeting, some companies have suggested to transmit Rel-18 DSR MAC CE if the amount of data volume exists in only one portion of the remaining time for a LCG, in order to reduce the overhead of enhanced DSR MAC CE. However, in our understanding, there would be no big size difference between the enhanced DSR MAC CE and Rel-18 DSR MAC CE in this case, since only one pair of buffer size field and the remaining time field is included in enhanced DSR MAC CE. Therefore, the benefits of transmitting Rel-18 DSR MAC CE in this case would be marginal.

On the other hand, if the UE is allowed to transmit the Rel-18 DSR MAC CE, e.g. based on the grant size and/or buffer status, the additional complexity is expected due to mixed procedure of enhanced DSR and Rel-18 DSR. In addition, the additional discussion is needed in order to determine in which case the Rel-18 DSR MAC CE can be transmitted and whether there is any benefit to transmit enhanced DSR MAC CE in each case. For example, even though the amount of data is non-zero for only one portion of remaining time, transmitting enhanced DSR MAC CE could be beneficial, since it may provide detailed delay information with finer granularity.

Therefore, for simplicity, it would be better to not mix the Rel-18 DSR MAC CE and enhanced DSR MAC CE, i.e., always use enhanced DSR MAC CE when the UE is configured with multiple reporting time thresholds for at least one LCG.

Proposal 2. When the UE is configured with multiple reporting thresholds for at least one LCG, the UE always transmits enhanced DSR MAC CE.

Whether to include non-delay critical data ahead of delay-critical data

In RAN2#127 meeting [2], it was discussed on whether the non-delay critical data can be included in enhanced DSR MAC CE, when the non-delay critical is queued ahead of delay critical data.

FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In our understanding, the purpose of including non-critical data ahead of delay-critical data is to ensure the transmission of delay-critical data in some cases, e.g., when the low importance data is in UE buffer in front of delay-critical data when the PSI discard is activated. However, given that the discard timer for low importance data (i.e., discardTimerForLowImportance) is defined to discard faster in congestion scenario, it would be configured with much smaller value than legacy PDCP discard time value. Then, it is less likely that next data is queued in the same buffer, becomes delay-critical data and triggering DSR, the corresponding DSR MAC CE is transmitted, and then the network allocates uplink grant for delay-critical data, while the low importance data ahead of delay-critical data has not been discarded yet. For other cases, given that the PDCP discard timer is started for each PDCP SDU when the PDCP layer receives the PDCP SDU from upper layer, the PDCP SDU with shorter discard timer would be queued first, so it would be corner case that non-critical data is queued ahead of delay-critical data.

On the other hand, if the non-delay critical data is calculated with delay-critical data, the additional complexity is expected, since the data volume calculation of delay-critical data and delay-reporting data should consider not only its running PDCP discard timer value but also UE buffer status in PDCP or RLC layer, e.g., whether the data is queued ahead of other delay-critical data in PDCP/RLC layer. In addition, it also causes additional impacts on the calculation of delay-critical data volume defined in Rel-18 DSR for the case of setting the triggering threshold as the largest value of the reporting thresholds, which changes the definition of delay-critical data.

It should be noted that the same discussion was held in Rel-18 XR, including whether to consider low importance data and whether to consider non-delay critical data ahead of delay-critical data. Based on long discussion, in order to avoid additional complexity on, e.g., triggering/calculation/cancellation procedure of DSR and associated BSR procedure, it was agreed that only delay-critical data is reported in DSR MAC CE, and the delay-critical data is determined only by the remaining time till PDCP discard timer expiry as follows:

3.1 Definitions Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold. (…omitted) 5.15 Data volume calculation for delay status reporting For the purpose of MAC delay status reporting, the transmitting PDCP entity shall consider the following as delay-critical PDCP data volume: - the delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed; - the PDCP Data PDUs that contain the delay-critical PDCP SDUs and have not been submitted to lower layers; - the PDCP Control PDUs; - for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13; - for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5. If a PDCP SDU becomes a delay-critical PDCP SDU, and if the corresponding PDCP Data PDU has already been submitted to lower layers, the delay-critical indication for the PDCP Data PDU is provided to lower layers.

Then, if the enhanced DSR MAC CE includes non-delay critical data ahead of delay-critical data, the buffer size field in enhanced DSR MAC CE would be different from the buffer size field in Rel-18 DSR MAC CE, for the same (reporting) threshold. Given that Rel-19 UEs may transmit Rel-18 DSR MAC CE in some cases, e.g., when only one reporting threshold equal to the triggering threshold is configured for all LCG, it would cause another UE complexity, since the UE need to consider non-delay critical data ahead of delay-critical data in some cases while the UE needs to exclude the non-delay critical data in other cases.

Therefore, for simplicity of enhanced DSR procedure, it is not suggested to consider additional case for calculation of data volume for enhanced DSR MAC CE.

Proposal 3. No further enhancements in the buffer size calculation for DSR to include non-delay critical data ahead of delay critical data

3.	Conclusion

In this contribution, it is discussed for the DSR enhancement for XR. This document includes following proposals.

Proposal 1. No need to specify constraints between DSR triggering threshold and reporting thresholds.

Proposal 2. When the UE is configured with multiple reporting thresholds for at least one LCG, the UE always transmits enhanced DSR MAC CE.

Proposal 3. No further enhancements in the buffer size calculation for DSR to include non-delay critical data ahead of delay critical data

4.	Reference

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Draft Report of 3GPP TSG RAN WG2 meeting #127 Maastricht, Netherlands, 19 - 23 August, 2024

[3] Draft Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China, 14 - 18 October, 2024