3GPP TSG-RAN2 Meeting #128	R2- 2409912

# Orlando, United States, Nov 18th – 22nd, 2024

Agenda item:		8.7.3 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Discussion on MG enhancement for XR

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 XR [1], it is agreed to define an enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements.

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

This document shows initial views on MG enhancement for XR in RAN2 aspects.

2.	Discussion

In RAN1 meetings [2-4], measurement gap skipping operation triggered/enabled by network signalling is discussed. Based on long discussion among multiple alternatives to enable the Tx/Rx procedure during measurement gap/scheduling restrictions for network-initiated solution, it is agreed to define explicit indication by DCI to skip a particular gap/restriction as a working assumption, with further details:

[RAN1#116 agreement] Agreement Consider at least solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. - FFS: Other types of solutions. - Whether or not/how to account for any UE assistance information/indication in addition to other information available at the network [RAN1#118 agreement] Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. [RAN1#118bis agreements] Agreement At least for self-scheduling case, for explicit indication by DCI to skip a particular gap/restriction, one bit indication is included as part of DCI formats 0_1/1_1 and 0_2/1_2. - FFS: Cross carrier scheduling. - FFS: DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. Agreement In case of cross carrier scheduling, for explicit indication by DCI to skip a particular gap/restriction, one bit indication included as a part of DCI formats 0_1/1_1 and 0_2/1_2 corresponds to a scheduled cell.

In order to apply the network-initiated solution with DCI indication to skip the measurement gap in MAC layer, a cross-layer indication is needed to ignore a specific measurement gap/scheduling restriction. For example, if MAC layer receives a notification of skipping a specific measurement gap within the PDCCH indication, the UE may perform UL transmission/DL reception during the indicated measurement gap.

Proposal 1. For network-initiated MG skipping operation, measurement gap can be ignored in MAC layer based on the cross-layer indication from PHY layer.

In RAN2#127bis meeting [5], it was discussed on whether there is any impact on DSR/DRX mechanism due to measurement gap. Based on the discussion, it is assumed that some impacts on DSR/DRX can be avoided by the proper network configuration as a baseline, and further discuss whether any enhancements on DSR/DRX is needed due to overlapping MG or MG skipping indication.

RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear.

For DSR procedure, given that network-initiated solution can be performed only if the network is aware of delay-critical data in UE side, so the transmission of DSR MAC CE may be delayed due to overlapping measurement gaps. However, any changes in current DSR triggering procedure is not desirable, since it would cause the additional UE complexity on current DSR procedure. For example, if the DSR triggering threshold is dynamically changed based on whether there is possibly overlapped MG, the additional complexity is expected on determination of delay-critical data, since UE needs to consider the additional factor, e.g., whether there is any overlapping MG during the certain time of period and the corresponding MG duration. In addition, given that the MG can be skipped based on the network indication, further discussion may be needed whether there is additional impact of skipped MG on DSR triggering procedure. Given that it is agreed to keep the definition of delay-critical data in RAN2#127bis meeting, it is suggested to keep the DSR triggering procedure without any MG-specific enhancement.

For DSR cancellation, no further enhancement is needed. Specifically, if the delay-critical data is discarded during the measurement gap, the corresponding DSR would be cancelled after the measurement gap, so there is no issues on current DSR cancellation mechanism related to overlapping MG.

Therefore, no further enhancement on DSR is needed related to measurement gap skipping.

Proposal 2. No MG-specific enhancements is needed on DSR operation.

During the discussion on RAN2#127bis meeting, it was suggested to consider some enhancements of DRX operation, in order to ensure that the MG skipping procedure based on DCI indication could work properly. For example, if the measurement gap duration starts at the beginning of the On Duration, the UE may not receive the measurement gap skipping indication even though the network decides to skip the measurement gap and transmits the DCI indication, since the UE may not monitor the PDCCH when the UE is not Active time. However, this issue can be handled by the proper network configuration, e.g., if the network aligns the measurement gap and the DRX on duration timer, the network would be able to transmit the DCI indication with measurement gap skipping during the UE’s On Duration. Therefore, the issues on DRX due to MG skipping is not essential and no further enhancements is needed on DRX operation.

Proposal 3. No MG-specific enhancements is needed on DRX operation.

In our understanding, the measurement gap/restriction skipping indication initiated by the network is to enable the data transmission/reception, when the network is already aware of UL/DL traffic information and corresponding delay requirements. Based on the network awareness of UL/DL traffic information, the performance degradation of XR traffic due to measurement gap/scheduling restriction can be avoided, e.g., when the periodicity of XR data is not aligned with the MG periodicity. In other words, when the network is aware of the delay information of XR data for UL/DL traffic, the network may decide to indicate MG skipping using scheduling DCI, in order to ensure the transmission of the UL/DL traffic before exceeding PDB/PSDB requirement of XR traffic.

Meanwhile, when the delay-critical data is arrived in the UE buffer, i.e., when the remaining time until the PDCP discard timer expiry is less than the threshold, DSR is triggered in order to indicate the amount of delay-critical data. However, even though DSR is triggered, the transmission of DSR MAC CE may be delayed due to the measurement gap, causing the discard operation of the corresponding the delay-critical data, due to lack of time of transmitting DSR and scheduling the uplink grant for the correspond delay-critical data.

In this case, MG skipping triggered by the network signalling cannot ensure the transmission of DSR MAC CE, since it is not possible for the network to indicate MG skipping for DSR MAC CE, as the network does not know whether the DSR is triggered or not at UE side. Therefore, UE-initiated MG skipping operation should be defined in order to ensure the transmission of DSR MAC CE, when there is delay-critical data exists in the UE side.

Proposal 4. Define UE-based solution to skip measurement gap/scheduling restriction, in order to ensure the transmission of DSR MAC CE.

3.	Conclusion

In this contribution, it is discussed for the MG enhancement for XR. This document includes following proposals.

Proposal 1. For network-initiated MG skipping operation, measurement gap can be ignored in MAC layer based on the cross-layer indication from PHY layer.

Proposal 2. No MG-specific enhancements is needed on DSR operation.

Proposal 3. No MG-specific enhancements is needed on DRX operation.

Proposal 4. Define UE-based solution to skip measurement gap/scheduling restriction, in order to ensure the transmission of DSR MAC CE.

4.	Reference

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Draft Report of 3GPP TSG RAN WG1 #116

[3] Draft Report of 3GPP TSG RAN WG1 #118

[4] Draft Report of 3GPP TSG RAN WG1 #118bis

[5] Draft Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China, 14 - 18 October, 2024