# 3GPP TSG RAN WG2 Meeting #128      	                           R2-2409942
Orlando, US, 18th – 22th Nov., 2024

Agenda item:	8.5.4

Source:	Apple

Title:	Discussion on common signal transmission adaptation

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

The Rel-19 WID of Network Energy Savings (WID RP-242354) was updated in RAN#105 [1]. The WI objective of adaptation of common signal transmission includes paging adaptation, PRACH adaptation and SSB adaptation, as copied below:

Up to now, only paging adaptation was discussed in RAN2. SSB adaptation and RACH adaptation were not discussed due to time limitation.

In this contribution, we further discuss RAN2 work on adaptation of common signal transmission.

# 2 Discussion

## 2.1 Adaptation of SSB in time domain

In RAN1, the key agreements on SSB adaptation with potential RAN2 impacts are listed below:

Agreement (RAN1#117) For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB Agreement (RAN1#118) For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Conclusion (RAN1#118b) There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

In our understanding, the key points can be summarized as follows:

- Only SSB periodicity adaptation for CONNECTED UEs is supported (i.e., Rel-19 will not support SSB adaptation for IDLE/INACTIVE UEs).

- SCell supports SSB adaptation. FFS whether PCell supports adaptation of non-CD-SSB or SSB off sync raster.

Observation 1: Key RAN1 progress on SSB adaptation with RAN2 impacts can be summarized as below:

- Only SSB periodicity adaptation for CONNECTED UEs is supported (i.e., Rel-19 will not support SSB adaptation for IDLE/INACTIVE UEs).

- SCell supports SSB adaptation. FFS whether PCell supports adaptation of non-CD-SSB or SSB off sync raster.

Although RAN1 discussion on SSB adaptation is not finished, we think potential RAN2 impacts include:

- MAC-CE format design if RAN1 agreed to use MAC-CE to indicate SSB periodicity adaptation.

- L3 measurement due to SSB adaptation, including RRM, RLM and BFD.

On 1), although RAN1 agreed to use MAC-CE to activate / deactivate OD-SSB which is now quite similar to SSB adaptation in Rel-19, RAN2 can wait RAN1 progress.

Proposal 1: On SSB adaptation, RAN2 design MAC-CE format if RAN1 conclude to use MAC-CE for SSB adaptation.

On 2), we first discuss L3 RRM impacts due to SSB periodicity adaptation. Please note that RAN4 has asked RAN2 whether SMTC configuration needs specification changes in their LS (R4-2416911) [2].

However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation.

We think the issue is quite similar to L3 measurement on OD-SSB which was agreed by RAN1 to use MAC-CE to dynamically change OD-SSB periodicity in SCell.

Observation 2: L3 measurement impacts on SSB adaptation is quite similar to L3 measurement on OD-SSB which was agreed by RAN1 to use MAC-CE to dynamically change OD-SSB periodicity in SCell.

As we discussed in our companion contribution on OD-SSB [3], we prefer to keep legacy SMTC based L3 RRM framework. We basically have same justifications to keep SMTC based framework for L3 measurement in SSB adaptation. The specific reasons why we prefer to keep legacy SMTC based L3 framework can be summarized in Observation 3 and Observation 4:

Observation 3: To support UE autonomous SSB measurement outside of SMTC, it will introduce significant RAN2/RAN4/RAN1 specification changes:

  - RAN2: it essentially challenges the per-frequency L3 RRM framework and introduces significant RAN2 specification impacts in Section 5.5 of TS 38.331 and Section 9.2.4 of TS 38.300.

  - RAN4: all L3 measurement requirements need specification changes because all L3 measurement requirements are specified with SMTC configuration rather than SSB configuration in legacy.

  - RAN1: it will result in multiple RAN1 specification changes in Section 5.1 of TS 38.215.

Observation 4: The UE autonomous SSB measurement outside of SMTC is bad for UE power saving and may lead to misalignment on actually measured SSB between UE and NW, especially when SSB is partially overlapped or has small timing gap with SMTC.

Thus, we have similar proposal as OD-SSB:

Proposal 2: On L3 measurement on SSB adaptation, keep SMTC based L3 RRM framework (i.e. not allow UE autonomous SSB measurement outside of SMTC).

As we discussed in our companion contribution on OD-SSB [3], we prefer to introduce dynamic SMTC adaptation in L3 measurement on OD-SSB. We basically have same justifications to introduce dynamic SMTC adaptation for L3 measurement in SSB adaptation. The specific reasons why we think dynamic SMTC adaptation is needed can be summarized in Observation 5 and Observation 6:

Observation 5: As SMTC is configured per frequency rather than per cell, it is difficulty for NW to configure one single SMTC window to cover all SSBs with different periodicities without causing negative UE impacts (e.g. extra power consumption if NW configures a dense SMTC to cover all SSBs).

Observation 6: In legacy, SMTC can only be reconfigured by RRC signaling. However, because SSB adaptation is expected to dynamically change periodicity of SSB via MAC-CE / DCI, one single SMTC can’t quickly align with the dynamic change of SSB periodicity.

Due to the similarity between OD-SSB and SSB adaptation, we prefer to reuse the same dynamic SMTC adaptation solution for SSB adaptation. Thus, we propose:

Proposal 3: On L3 measurement on SSB adaptation, introduce dynamic adaptation between different SMTC configurations. FFS signaling details of how to configure multiple SMTC configurations.

As discussed before, RAN4 has asked RAN2 whether SMTC configuration needs specification changes in their LS (R4-2416911) [2]. RAN2 need to inform RAN4 if Proposal 2 and Proposal 3 are agreed. We provide an example of reply LS in Appendix.

Proposal 4: Send reply LS (as response to RAN4 LS R4-2416911) to inform RAN4 on Proposal 2 and Proposal 3 in Appendix.

Finally, different from OD-SSB which is supported only in SCell, SSB adaptation may be supported in PCell. Thus, we think RAN2 impacts on RLM and BFD need further discussion:

- RLM: In legacy RLM, several SSB(s) can be configured as RLM-RS in RRC. However, the legacy RRC reconfiguration of RadioLinkMonitoringConfig may not work because RRC signaling is slower than DCI / MAC-CE which may lead to failure of UE performing RLM (e.g. one actually transmitted beam configured as RLM-RS is turned off after SSB adaptation).

- BFD: In legacy BFD in SCell, several SSB(s) can be configured as BFD-RS in RRC. However, the legacy RRC reconfiguration of RadioLinkMonitoringConfig may not work because RRC signaling is slower than DCI / MAC-CE which may lead to failure of UE performing BFD (e.g. one actually transmitted beam configured as BFD-RS is turned off after SSB adaptation).

Because RLM and BFD rely on configured SSB periodicity rather than SMTC, we think maybe it is sufficient to allow the UE to dynamically adapt periodicity of RLM-RS and/or BFD-RS upon SSB adaptation.

Observation 7: Because RLM and BFD rely on configured SSB periodicity rather than SMTC, maybe it is sufficient to allow the UE to dynamically adapt periodicity of RLM-RS and/or BFD-RS upon SSB adaptation.

However, because SSB adaptation in PCell is still not finalized in RAN1, we think RAN2 can further discuss their impacts. Thus, we propose:

Proposal 5: RAN2 study the spec impacts on RLM and BFD due to SSB adaptation.

## 2.2 Adaptation of paging occasions

In RAN2#127b, option-b (i.e. extend value range of N and Ns) is agreed as baseline [4]:

Agreements on paging adaptation

- Select option-b as baseline for R19 NES paging enhancement.

- R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Legacy UE is not barred.

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

However, some leftover issues of option-b were discussed but no conclusion can be made. In this section, we discuss the following remaining issues:

- Detailed values of extended N and Ns

- Whether PO offset is sufficient for legacy and R19 UEs co-existence

- How to support PEI when Ns>4

### 2.2.1 Detailed values of extended N and Ns

According to Table. 1, we need to increase N from 16 to 256. Then, if we need to fully compensate the decrease in number of PFs, Ns needs to increase from 4 to 64.

| Value of 1/N | Number of PFs in one paging cycle |  |  |  | Required Ns if fully compensate |
| --- | --- | --- | --- | --- | --- |
| 1/N | rf32 | rf64 | rf128 | rf256 |  |
| 1/16 | 2 | 4 | 8 | 16 | 4 (no need to extend Ns) |
| 1/8 | 4 | 8 | 16 | 32 | 8 |
| 1/4 | 8 | 16 | 32 | 64 | 16 |
| 1/2 | 16 | 32 | 64 | 128 | 32 |
| 1 | 32 | 64 | 128 | 256 | 64 |

Table.1 All possible cases of PF number in one paging cycle

Then, large number of Ns will increase RRC signaling overhead of PMO configuration because its overhead linearly increases with maxPO-perPF (i.e. Ns), as illustrated below:

PMO configuration

PCCH-Config ::=             SEQUENCE {

omit

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT            SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

omit

}

Then, we provide an analysis on required RRC signalling overhead under different Ns values in table.2.

|  | Ns=4 (baseline) | Ns=8 | Ns=16 | Ns=32 | Ns=64 |
| --- | --- | --- | --- | --- | --- |
| Payload of PMO configuration (bit) | 18 | 36 | 72 | 144 | 288 |

Table.2 Required RRC signaling overhead under different Ns

In our understanding, increasing maximum value to 16 is a good compromise:

- Paging latency: In case of 2560ms Paging Cycle, at least 16 PFs can be configured in legacy (with N=1/16). Then, with legacy maximum value of N_s=4, there are totally 64 POs in one Paging Cycle. If we only increase N_s value to 8, it is only 1/8 PO compared with legacy. We think it will increase paging latency which is conflicted with the requirement of WID [1]. Thus, we think increasing Ns to at least 16 is necessary.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

- RRC overhead: According to Table.2, increasing Ns to 16 needs 72bit, which is just 54bit increase compared with legacy. We think it should be acceptable. And at least from RRC overhead point of view, we don’t see need to further optimize RRC signalling structure of firstPDCCH-MonitoringOccasionOfPO.

Observation 8: Increasing maximum value to 16 is a good compromise:

- Paging latency: In case of 2560ms Paging Cycle, if only increase N_s value to 8, there is only 1/8 PO compared with legacy. It will increase paging latency which is conflicted with the requirement of WID.

- RRC overhead: According to Table.2, increasing Ns to 16 needs 72bit, which is just 54bit increase compared with legacy.

Thus, we propose:

Proposal 6: N value is extended with 32, 64, 128, and 256. And Ns value is extended with 8 and 16.

### 2.2.2 Whether PO offset is sufficient for legacy and R19 UE co-existence

During online discussion of RAN2#127b [4], there was concern raised on whether legacy PO offset is sufficient to allow Rel-19 POs not to overlap with legacy POs. However, we are confused about this concern. In our understanding, when Rel-19 PO overlaps with legacy PO, ue-Identity included paging record is used to differentiate Rel-19 UE and legacy UE [5].

Paging ::=                          SEQUENCE {

pagingRecordList                    PagingRecordList                                                        OPTIONAL, -- Need N

lateNonCriticalExtension            OCTET STRING                                                            OPTIONAL,

nonCriticalExtension                Paging-v1700-IEs                                                        OPTIONAL

}

PagingRecordList ::=                SEQUENCE (SIZE(1..maxNrofPageRec)) OF PagingRecord

PagingRecord ::=                    SEQUENCE {

ue-Identity                         PagingUE-Identity,

accessType                          ENUMERATED {non3GPP}    OPTIONAL,   -- Need N

...

}

So, we don’t see any issue on PO offset.

Proposal 7: RAN2 confirm that there is no PO offset issue in option-b because legacy PO and Rel-19 PO can overlap, and ue-Identity included in paging record is used to differentiate Rel-19 UE and legacy UE.

### 2.2.3 How to support PEI when Ns>4

The legacy PEI framework is summarized as follows:

- IE po-NumPerPEI in SIB1 configures how many POs are associated with one PEI.

  - One PEI can be associated to up to 4 POs in one PF and up to 8 POs in two PFs when Ns=4.

- The PEI occasion is determined by a frame-level offset (via IE pei-FrameOffset) and a symbol level offset (via IE firstPDCCH-MonitoringOccasionOfPEI-O-r17) with below details captured in 38.331 [5]:

  - The above highlighted part covers the case that one PEI is assocaited to up to 4 POs in one PF:

    - One example in legacy: assume that po-NumPerPEI = 1 (one PEI is associated with only 1 PO), Ns=4, the UE’s i_s=3 and 15KHz SCS. Then, the UE take the 4th value of sCS15KHZoneT as the symbol offset of the PEI.

  - The above highlighted part covers the case that one PEI is assocaited to up to 8 POs in two PF:

    - One example in legacy: assume that po-NumPerPEI = 8 (one PEI is associated with 8 POs), Ns=4, the UE’s SCS is 15KHz. Then, irrespective of i_s value, the UE take the 1st value of sCS15KHZoneT as the symbol offset of the PEI.

- PEI is transmitted via DCI format 2-7:

  - Length of the PEI field is N * M bits, assuming one PEI is associated with N POs and there are M UE subgroups for each PO.

  - Payload of DCI 2-7 is smaller than 41 bits.

PEI-O configuration

pei-ConfigBWP-r17      SEQUENCE {

pei-SearchSpace-r17                 SearchSpaceId,

firstPDCCH-MonitoringOccasionOfPEI-O-r17  CHOICE {

sCS15KHZoneT                                                     SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                       SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT  SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919),

sCS480KHZoneEighthT                                           SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                                        SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679)

}

maxPEI-perPF-r17                        INTEGER ::= 4       -- Maximum number of PEI occasion per paging frame

firstPDCCH-MonitoringOccasionOfPEI-O Offset, in number of symbols, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O on this BWP, see TS 38.213 [13], clause 10.4A. For the case po-NumPerPEI is smaller than Ns, UE applies the (floor(i_s/po-NumPerPEI)+1)-th value out of (N_s/po-NumPerPEI) configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset. When po-NumPerPEI is one or multiple of Ns, UE applies the first configured value in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset.

Option-b can’t work when Ns is 8 or 16. For example, when po-NumPerPEI = 1 and Ns=8, only the first 4 POs (i.e. i_s =0/1/2/3) can be associated with PEI and the last 4 POs (i.e. i_s =4/5/6/7) can’t be associated with any PEI. The root cause is due to hard coding of ASN.1 with maxPEI-perPF-r17=4.

Observation 9: The legacy PEI framework can’t work when Ns is in increased to 8 or 16. For example, when po-NumPerPEI = 1 and Ns=8, only the first 4 POs (i.e. i_s =0/1/2/3) can be associated with PEI and the last 4 POs (i.e. i_s =4/5/6/7) can’t be associated with any PEI.

One may think that this issue can be resolved by simply increasing maxPEI-perPF-r19 to 8 or 16. However, it will also require to increase the number of associated POs for one PEI and correspondingly will increase length of the PEI field in DCI 2-7 which may violate 41bits payload restriction.

Observation 10: The PEI issue can’t be resolved by simply increasing maxPEI-perPF-r19 to 8 or 16 because it will also increase length of the PEI field in DCI 2-7 which may violate 41bits payload restriction.

We think RAN2 should address this issue. Thus, we propose:

Proposal 8: RAN2 study how to support PEI when Ns value is increased to 8 and 16.

## 2.3 Adaptation of PRACH in time domain

RAN1#116, RAN1#116b and RAN1#117 had discussed RACH adaptation [6][7][8], and a lot of agreements were made. Based on RAN1 agreements, we understand that RAN1 progress can be highlighted as below:

- Introduce adaptation mechanism between legacy PRACH resources and one new additional PRACH resources for NES-capable UE in all RRC states.

  - RAN1 agreed the adaptation signaling can be L1/DCI signaling and RRC/SIB.

- The additional PRACH resources are provided via a prach-ConfigurationIndex and a separate SSB-RO mapping is performed for the additional PRACH resources.

  - It is still under discussion whether to allow any overlapping between legacy PRACH resources and the additional PRACH.

  - prach-ConfigurationIndex for the additional PRACH resources can be same as or different from the one for the legacy resource.

Based on latest RAN1 progress, we share our view on potential RAN2 work in the followed sub-sections.

### 2.3.1 Legacy UE compatibility

First, we discuss the legacy compatibility issue. Because RAN1 agreements have considered legacy UE impacts, our understanding is that RAN2 don’t need to introduce new legacy UE barring mechanism for RACH adaptation. Meanwhile, same as paging adaptation, we don’t think it makes sense to deploy Rel-19 NES cells which only support Rel-19 UEs capable of RACH adaptation because it may lead to coverage holes and goes against the intention of NES.

Proposal 9: RAN2 don’t introduce new legacy UE barring mechanism for RACH adaptation in time domain, i.e., Rel-19 NES cells support both Rel-19 UEs capable of RACH adaptation and legacy UEs.

### 2.3.2 Clarification on RACH framework

We have one general clarification question on the RACH framework to integrate RACH adaptation. In Rel-17, general RACH framework supporting multiple RACH partitions was introduced. As illustrated in Figure.1, a list of up to 255 RACH partitions (i.e. AdditionalRACH-Config) can be configured in SIB1, and the Network can configure different RACH partitions (i.e. AdditionalRACH-Config) associated with different features (e.g., redcap) or feature combination (e.g., SDT+Msg1-repetition).

Figure.1 Illustration of RRC configuration of multiple RACH partitions

Observation 11: In Rel-17, general RACH framework supporting multiple RACH partitions was introduced. A list of up to 255 RACH partitions (i.e. AdditionalRACH-Config) can be configured in SIB1, and the Network can configure different RACH partitions associated with different features (e.g., redcap) or feature combination (e.g., SDT+Msg1-repetition).

Firstly, we discuss whether RACH adaptation is applicable to CBRA or CFRA. Because CFRA may use UE dedicated signaling, the benefit of RACH adaptation seems not quite clear. Thus, we propose to first study CBRA.

Proposal 10: RAN2 first study CBRA in RACH adaptation.

Secondly, even for CBRA, we think it is not clear what is the role of RACH adaptation in the general RACH framework. Specifically, there are two different understandings:

- Understanding 1: RACH adaptation is applied to only Rel-15 RACH resources (i.e. only support adaptation between rach-ConfigCommon and additional PRACH resources).

- Understanding 2: RACH adaptation is applied to all/part of RACH features or feature combination (i.e. support adaptation between rach-ConfigCommon and additional PRACH resources, and adaptation between AdditionalRACH-Config and additional PRACH resources). FFS whether/which RACH features or feature combination are not applicable.

In our understanding, it seems that RAN1 so far only discussed adaptation between rach-ConfigCommon and additional PRACH resources (i.e. Understanding 1). However, we also think it makes sense to support RACH adaptation in at least some RACH features or feature combination (e.g., redcap + additional PRACH resource), but some RACH feature may further discussion (e.g., msg1-repetition + additional PRACH resource). That is why we put an FFS in understanding 2.

We are open to either understanding, but RAN2 should make a conclusion about it from beginning because it will impact the workload.

Proposal 11: RAN2 discuss which is the correct understanding of RACH adaptation in time domain:

- Understanding 1: RACH adaptation is applied to only Rel-15 RACH resources (i.e. only support adaptation between rach-ConfigCommon and additional PRACH resources).

- Understanding 2: RACH adaptation is applied to all/part of RACH features or feature combination (i.e. support adaptation between rach-ConfigCommon and additional PRACH resources, and adaptation between AdditionalRACH-Config and additional PRACH resources). FFS whether/which RACH features or feature combination are not applicable.

### 2.3.3 Potential RAN2 work

Based on latest RAN1 progress, we think that at least the following RRC and MAC impacts are foreseen:

RRC impacts:

- Introduce a separate SSB-RO mapping configuration for additional PRACH resource, e.g. ssb-perRACH-OccasionAndCB-PreamblesPerSSB-NES in RACH-ConfigCommon.

- How to integrate additional PRACH resource set into RACH framework, which couples with which understanding in Proposal 7 is agreed.

MAC impacts:

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in initial Msg1 transmission.

  - For example, Rel-19 UE may always start from additional PRACH resource, considering legacy UE can only use legacy PRACH resource.

  - As another example, if target for RACH latency minimization, Rel-19 UE may prioritize legacy PRACH resource or additional PRACH resource depending on when preamble transmission is triggered (e.g. the UE may prioritize legacy resource when preamble transmission is triggered in T0, and the UE may prioritize additional PRACH resource when preamble transmission is triggered in T1 in Figure.2).

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in Msg1 retransmission.

  - RAN2 need to discuss whether only the same PRACH resource set (i.e. legacy vs additional) as initial transmission can be selected.

- Whether / how to support fallback from one PRACH resource to another PRACH resource.

Figure. 2 Illustration of RO selection in RACH adaptation in time domain

Thus, we propose:

Proposal 12: To support additional PRACH resource adaptation, RRC spec impacts at least include:

- Introduce a separate SSB-RO mapping configuration for additional PRACH resource, e.g. ssb-perRACH-OccasionAndCB-PreamblesPerSSB-NES in RACH-ConfigCommon.

- How to integrate additional PRACH resource set into RACH framework, which couples with which understanding in Proposal 7 is agreed.

Proposal 13: To support additional PRACH resource adaptation, MAC spec impacts at least include:

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in initial Msg1 transmission.

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in Msg1 retransmission.

- Whether / how to support fallback from one PRACH resource to another PRACH resource.

# 3 Conclusion

In this contribution, we further discuss RAN2 work on common signal transmission adaptation. Our observations are:

Observation 1: Key RAN1 progress on SSB adaptation with RAN2 impacts can be summarized as below:

- Only SSB periodicity adaptation for CONNECTED UEs is supported (i.e., Rel-19 will not support SSB adaptation for IDLE/INACTIVE UEs).

- SCell supports SSB adaptation. FFS whether PCell supports adaptation of non-CD-SSB or SSB off sync raster.

Observation 2: L3 measurement impacts on SSB adaptation is quite similar to L3 measurement on OD-SSB which was agreed by RAN1 to use MAC-CE to dynamically change OD-SSB periodicity in SCell.

Observation 3: To support UE autonomous SSB measurement outside of SMTC, it will introduce significant RAN2/RAN4/RAN1 specification changes:

  - RAN2: it essentially challenges the per-frequency L3 RRM framework and introduces significant RAN2 specification impacts in Section 5.5 of TS 38.331 and Section 9.2.4 of TS 38.300.

  - RAN4: all L3 measurement requirements need specification changes because all L3 measurement requirements are specified with SMTC configuration rather than SSB configuration in legacy.

  - RAN1: it will result in multiple RAN1 specification changes in Section 5.1 of TS 38.215.

Observation 4: The UE autonomous SSB measurement outside of SMTC is bad for UE power saving and may lead to misalignment on actually measured SSB between UE and NW, especially when SSB is partially overlapped or has small timing gap with SMTC.

Observation 5: As SMTC is configured per frequency rather than per cell, it is difficulty for NW to configure one single SMTC window to cover all SSBs with different periodicities without causing negative UE impacts (e.g. extra power consumption if NW configures a dense SMTC to cover all SSBs).

Observation 6: In legacy, SMTC can only be reconfigured by RRC signaling. However, because SSB adaptation is expected to dynamically change periodicity of SSB via MAC-CE / DCI, one single SMTC can’t quickly align with the dynamic change of SSB periodicity.

Observation 7: Because RLM and BFD rely on configured SSB periodicity rather than SMTC, maybe it is sufficient to allow the UE to dynamically adapt periodicity of RLM-RS and/or BFD-RS upon SSB adaptation.

Observation 8: Increasing maximum value to 16 is a good compromise:

- Paging latency: In case of 2560ms Paging Cycle, if only increase N_s value to 8, there is only 1/8 PO compared with legacy. It will increase paging latency which is conflicted with the requirement of WID.

- RRC overhead: According to Table.2, increasing Ns to 16 needs 72bit, which is just 54bit increase compared with legacy.

Observation 9: The legacy PEI framework can’t work when Ns is in increased to 8 or 16. For example, when po-NumPerPEI = 1 and Ns=8, only the first 4 POs (i.e. i_s =0/1/2/3) can be associated with PEI and the last 4 POs (i.e. i_s =4/5/6/7) can’t be associated with any PEI.

Observation 10: The PEI issue can’t be resolved by simply increasing maxPEI-perPF-r19 to 8 or 16 because it will also increase length of the PEI field in DCI 2-7 which may violate 41bits payload restriction.

Observation 11: In Rel-17, general RACH framework supporting multiple RACH partitions was introduced. A list of up to 255 RACH partitions (i.e. AdditionalRACH-Config) can be configured in SIB1, and the Network can configure different RACH partitions associated with different features (e.g., redcap) or feature combination (e.g., SDT+Msg1-repetition).

Based on observations, our proposals are:

Adaptation of SSB in time domain

Proposal 1: On SSB adaptation, RAN2 design MAC-CE format if RAN1 conclude to use MAC-CE for SSB adaptation.

Proposal 2: On L3 measurement on SSB adaptation, keep SMTC based L3 RRM framework (i.e. not allow UE autonomous SSB measurement outside of SMTC).

Proposal 3: On L3 measurement on SSB adaptation, introduce dynamic adaptation between different SMTC configurations. FFS signaling details of how to configure multiple SMTC configurations.

Proposal 4: Send reply LS (as response to RAN4 LS R4-2416911) to inform RAN4 on Proposal 2 and Proposal 3 in Appendix.

Proposal 5: RAN2 further study the spec impacts on RLM and BFD due on SSB adaptation.

Adaptation of paging occasions

Proposal 6: N value is extended with 32, 64, 128, and 256. And Ns value is extended with 8 and 16.

Proposal 7: RAN2 confirm that there is no PO offset issue in option-b because legacy PO and Rel-19 PO can overlap, and ue-Identity included in paging record is used to differentiate Rel-19 UE and legacy UE.

Proposal 8: RAN2 study how to support PEI when Ns value is increased to 8 and 16.

Adaptation of PRACH in time domain

Proposal 9: RAN2 don’t introduce new legacy UE barring mechanism for RACH adaptation in time domain, i.e., Rel-19 NES cells support both Rel-19 UEs capable of RACH adaptation and legacy UEs.

Proposal 10: RAN2 first study CBRA in RACH adaptation.

Proposal 11: RAN2 discuss which is the correct understanding of RACH adaptation in time domain:

- Understanding 1: RACH adaptation is applied to only Rel-15 RACH resources (i.e. only support adaptation between rach-ConfigCommon and additional PRACH resources).

- Understanding 2: RACH adaptation is applied to all/part of RACH features or feature combination (i.e. support adaptation between rach-ConfigCommon and additional PRACH resources, and adaptation between AdditionalRACH-Config and additional PRACH resources). FFS whether/which RACH features or feature combination are not applicable.

Proposal 12: To support additional PRACH resource adaptation, RRC spec impacts at least include:

- Introduce a separate SSB-RO mapping configuration for additional PRACH resource, e.g. ssb-perRACH-OccasionAndCB-PreamblesPerSSB-NES in RACH-ConfigCommon.

- How to integrate additional PRACH resource set into RACH framework, which couples with which understanding in Proposal 7 is agreed.

Proposal 13: To support additional PRACH resource adaptation, MAC spec impacts at least include:

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in initial Msg1 transmission.

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in Msg1 retransmission.

- Whether / how to support fallback from one PRACH resource to another PRACH resource.

# 4 References

[1] RP-242354, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] R4-2416911, LS on SSB adaptation.

[3] R2-2409940, Discussion on on-demand SSB for SCell, Apple

[4] RAN2#127b, Chair Notes.

[5] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[6] RAN1#116, Chair Notes.

[7] RAN1#116b, Chair Notes.

[8] RAN1#117, Chair Notes.

# Appendix

# 3GPP TSG RAN WG2 Meeting #128      	                           R2-240xxxx
Orlando, US, 18th – 22th Nov., 2024

Title:	Reply LS on SSB adaptation

Response to:	R4-2416911

Release:	Release - 19

Work Item:	Netw_Energy_NR_enh-Core

Source:	RAN2

To:	RAN4

Cc:	RAN1

Contact Person:

#### Name:	Peng Cheng

E-mail Address:	< pcheng24@apple.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN2 thanks RAN4 for the LS on SSB adaptation. Regarding the following question from RAN4:

“However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation.”

RAN2 conclude to keep SMTC based L3 RRM framework (i.e. not allow UE autonomous SSB measurement outside of SMTC). And dynamic adaptation between different SMTC configurations can be introduced to adapt to dynamic change of SSB periodicity.

2. Actions:

To RAN2.

ACTION: 	RAN2 respectfully asks RAN4 to take the above information into account.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP TSG RAN WG2#128	     18 November - 22 November 2024	             Orlando, US

3GPP TSG RAN WG2#129	     17 February - 21 February 2025		Athens, GR