3GPP TSG-RAN WG2 #128                                             	    R2-2409988

Orlando, USA, November 18th –22nd, 2024

Agenda item:	   8.6.4

Source: 		Qualcomm Incorporated

Title: 	Conditional intra-CU LTM

Document for:	Discussion

# 1	Introduction

The WID for mobility enhancements includes the following objective [1]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

RAN2#127b agreed the following [2]:

Agreements on C-LTM - Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. - Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. - Source cell and each candidate cell provides its own execution condition for conditional LTM. - It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. - RACH-less Conditional intra-CU LTM is supported. - RACH based conditional intra-CU LTM is supported. - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM. - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. - For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. - The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

This paper discusses further aspects related to conditional intra-CU LTM.

# 2 	C-LTM preparation

## 2.1	C-LTM candidate cells as a subset of LTM candidate cells

In L3 mobility, a UE may receive an L3 HO command towards a candidate cell for which the UE has previously received a stored CHO configuration. Similarly, the network should be allowed to trigger LTM towards a target cell that is a C-LTM candidate cell. This implies that C-LTM candidate cells are a subset of LTM candidate cells.

Observation 1: In L3 mobility, the network may trigger L3 HO towards a CHO candidate cell.

Proposal 1a: C-LTM candidate cells are a subset of LTM candidate cells. Network-triggered LTM towards a C-LTM candidate cell is always supported.

The UE determines which subset of the LTM candidate cells are also C-LTM candidate cells based on configuration. Specifically, if the LTM candidate cell configuration also includes configuration for C-LTM condition evaluation, then the LTM candidate cell is also a C-LTM candidate cell.

Proposal 1b: A UE identifies an LTM candidate cell as a C-LTM candidate cell if the LTM candidate cell is included in a configuration for C-LTM condition evaluation.

## 2.2	Subsequent C-LTM as a baseline

The WID states that RAN2 should aim to support subsequent C-LTM. Since avoiding RRC configuration between cell switches is a key advantage of LTM, subsequent C-LTM should be the baseline.

Proposal 2a: Subsequent C-LTM is the baseline: UE does not release LTM/C-LTM configuration following a C-LTM cell switch.

Once a cell switch is triggered, it makes no difference whether that triggering is a network trigger or based on UE evaluation of conditions. For instance:

- In both cases, the UE applies the stored RRC configuration and sends the RRC complete message.

- In both cases, the target cell notifies the gNB-CU of the UE’s successful access

- In both cases, the gNB-CU switches the UP datapath towards the target cell.

This implies the UE should be able to perform any sequence of network-triggered LTM/C-LTM cell switch executions in any order.

Proposal 2b: RAN2 to support a unified framework for LTM and C-LTM, where the UE may perform any sequence of LTM/C-LTM cell switch executions among the candidate cells.

## 2.3	Multiple event conditions for C-LTM

Similar to CHO, having multiple event conditions for C-LTM can be optionally supported in which case all event conditions shall be met to trigger C-LTM.

Proposal 3a: Multiple event conditions for C-LTM evaluation can be optionally for C-LTM similar to CHO. Support of more than 2 event conditions is FFS.

Proposal 3b: When multiple event conditions are configured, then all conditions shall be met to trigger C-LTM.

# 3 	C-LTM early synchronization

## 3.1	Early TA acquisition

In CHO, after UE evaluates the event condition(s), it is left up to UE to decide the best SSB index to be used for DL reception and UL transmission after the cell switch. Such UE autonomous approach would allow UE to select the best SSB index (or beam direction) at the cell switch moment.

In C-LTM, the circumstance remains same, i.e., UE would know the most knowledge about the best target candidate cell and its candidate beam when C-LTM decision is made. As agreed in RAN2#127b, PDCCH ordered early TA acquisition is supported for C-LTM, but there are still potential issues with reusing Rel-18 LTM PDCCH order mechanism (i.e., serving cell decides PDCCH ordered RACH based on measurement reports), due to the lack of C-LTM timing information at serving cell. Firstly, the selected candidate cell and candidate beam for PDCCH ordered RACH may not be the optimal decision when C-LTM is triggered. Secondly, even though the candidate cell and candidate beam remain the best, the acquired TA value may become outdated when C-LTM is triggered.

In order to improve the situation, the UE autonomous decision of candidate cell and its beam for PDCCH ordered early TA acquisition can be considered. Since UE has been already monitoring the event condition for C-LTM, UE could make the better decision on both target candidate cell & its beam and the timing of RACH transmission.

Observation 2a: Due to the unknown timing of C-LTM trigger at serving cell, Rel-18 LTM PDCCH ordered RACH may not perform well, e.g., non-optimal candidate cell and/or candidate beam selection, or TA value aging.

Observation 2b: Since UE keeps monitoring the event conditions for C-LTM, UE could make the better decision on RACH transmission.

Proposal 4: RAN2 to support a PDCCH-order request signaling (from UE to serving cell) to trigger PDCCH ordered RACH.

When PDCCH-ordered early TA acquisition is supported for C-LTM, the signaling method to convey the TA value to UE needs to be discussed. Unlike Rel-18 LTM where LTM CSC MAC-CE contains the TA value obtained by PDCCH-ordered RACH, currently there’s no such signaling defined for C-LTM. Therefore, RAN2 needs to discuss the new signaling method(s) to convey the TA value to UE for C-LTM, which can be either enhancing the existing LTM CSC MAC-CE and/or defining the new MAC-CE.

Observation 3: In Rel-18 LTM procedure, there’s no way from serving cell to send the TA value to UE.

Proposal 5: RAN2 to support a new MAC-CE based TA signaling method for C-LTM UE.

- Option 1: Rel-18 LTM CSC is enhanced to signal TA value without triggering LTM cell switch

- Option 2: New MAC-CE is defined for TA value signaling.

## 3.2	Early TCI state activation

RAN2#127b agreed to support Rel-18 early candidate TCI State activation/deactivation for C-LTM. Even though Rel-18 approach (i.e., gNB makes a decision on TCI state to be activated/deactivated) is still valid for C-LTM, the potential issues discussed in Section 3.1 (early TA acquisition) remain same, i.e., due to the lack of information on C-LTM timing at serving cell, early activated TCI state may not be optimal when C-LTM is triggered. Therefore, it would be beneficial to introduce UE assisted TCI state activation/deactivation mechanism for C-LTM. UE assisted TCI state activation/deactivation could be implemented such that UE provides a list of candidate cells and their beams to be activated/deactivated. In response to the request message, serving cell can send Rel-18 candidate TCI state activation/deactivation MAC-CE message. UE autonomous TCI state activation/deactivation could be implemented such that UE activates/deactivates TCI states autonomously based on the measurement data, and then may notify the serving cell of the activated/deactivated TCI state information.

Observation 4: Due to the unknown timing of C-LTM trigger at serving cell, Rel-18 LTM early candidate TCI state activation/deactivation may not perform well, e.g., non-optimal candidate cell and/or candidate beam selection.

Proposal 7: RAN2 to discuss UE autonomous and/or assisted TCI state activation mechanisms.

- Option 1: UE requests to activate TCI state(s) of candidate cell(s), and serving cell sends Rel-18 candidate cell TCI state activation/deactivation MAC-CE.

- Option 2: UE autonomously activates TCI state(s) of candidate cell(s), and notifies serving cell of activation information.

# 4	C-LTM evaluation and execution

RAN2#127 agreed that MAC layer handles the event evaluation and measurement report triggering for L1 event-triggered measurement reporting for LTM. Therefore, we can follow the same principle for C-LTM.

Proposal 8: MAC layer handles the event evaluation and determines the C-LTM trigger.

When multiple candidate cells fulfill the C-LTM triggering condition, then the target cell selection can be left to UE implementation, similar to CHO.

Proposal 9: When more than one candidate cells fulfill the C-LTM triggering condition, then it is up to UE to decide which candidate cell UE would perform C-LTM to.

RAN2#127b agreed to support CG-based first UL transmission on target cell for RACH-less C-LTM. Due to the unknown C-LTM cell switch timing to both serving cell and multiple candidate cells, CG time/freq resources would have to be reserved and most of them would be left unused, i.e., those CG resources cannot be used for other DG allocations due to the potential collision with C-LTM UE’s first UL transmission. As more candidate cells are configured with such CG resources, the resource inefficiency could become more severe. Therefore, it would be beneficial to introduce a new signaling procedure to notify early the potential target cell for C-LTM from UE to the serving cell. This signaling would allow only the target candidate cell to monitor CG resources for a certain time period so that other candidate cells (not selected) can reuse CG resources to DG allocations without any collision issue.

Observation 5: CG resource reservations from multiple target cells may lead to a significant inefficiency in time/freq resources due to a large number of resource reservations.

Proposal 11a: For RACH-less C-LTM, RAN2 to support an early target cell notification signaling procedure to optimize the CG resource reservation at target cell(s). Details signaling/condition are FFS.

Regarding DG-based first UL transmission, the biggest problem is the DG timing uncertainty for which the target cell doesn’t know when DG allocation has to be sent towards C-LTM UE. With the early notification from Proposal 11a, the DG timing uncertainty could be also significantly alleviated. Once the target candidate cell receives the notification from the serving cell, then it can send DG allocations towards the C-LTM UE for a certain time period.

Observation 6: An early target cell notification signaling from Proposal 11a can also help reduce the DG timing uncertainty at candidate cells.

Proposal 11b: Along with the early target cell notification signaling, RAN2 to support DG-based first UL transmission for RACH-less C-LTM.

# Conclusion

This contribution discussed aspects related to conditional intra-gNB LTM. The following observations and proposals have been made:

C-LTM candidate cells as a subset of LTM candidate cells

Observation 1: In L3 mobility, the network may trigger L3 HO towards a CHO candidate cell.

Proposal 1a: C-LTM candidate cells are a subset of LTM candidate cells. Network-triggered LTM towards a C-LTM candidate cell is always supported.

Proposal 1b: A UE identifies an LTM candidate cell as a C-LTM candidate cell if the LTM candidate configuration cell is included in includes a configuration for C-LTM condition evaluation.

Subsequent C-LTM is baseline

Proposal 2a: Subsequent C-LTM is the baseline: UE does not release LTM/C-LTM configuration following a C-LTM cell switch.

Proposal 2b: RAN2 to support a unified framework for LTM and C-LTM, where the UE may perform any sequence of LTM/C-LTM cell switch executions among the candidate cells.

Multiple event conditions for C-LTM

Proposal 3a: Multiple event conditions for C-LTM evaluation can be optionally for C-LTM similar to CHO. Support of more than 2 event conditions is FFS.

Proposal 3b: When multiple event conditions are configured, then all conditions shall be met to trigger C-LTM.

Early TA acquisition

Observation 2a: Due to the unknown timing of C-LTM trigger at serving cell, Rel-18 LTM PDCCH ordered RACH may not perform well, e.g., non-optimal candidate cell and/or candidate beam selection, or TA value aging.

Observation 2b: Since UE keeps monitoring the event conditions for C-LTM, UE could make the better decision on RACH transmission.

Proposal 4: RAN2 to support a PDCCH-order request signaling (from UE to serving cell) to trigger PDCCH ordered RACH.

Observation 3: In Rel-18 LTM procedure, there’s no way from serving cell to send the TA value to UE.

Proposal 5: RAN2 to support a new MAC-CE based TA signaling method for C-LTM UE.

- Option 1: Rel-18 LTM CSC is enhanced to signal TA value without triggering LTM cell switch

- Option 2: New MAC-CE is defined for TA value signaling.

Early TCI state activation

Observation 4: Due to the unknown timing of C-LTM trigger at serving cell, Rel-18 LTM early candidate TCI state activation/deactivation may not perform well, e.g., non-optimal candidate cell and/or candidate beam selection.

Proposal 7: RAN2 to discuss UE autonomous and/or assisted TCI state activation mechanisms.

- Option 1: UE requests to activate TCI state(s) of candidate cell(s), and serving cell sends Rel-18 candidate cell TCI state activation/deactivation MAC-CE.

- Option 2: UE autonomously activates TCI state(s) of candidate cell(s), and notifies serving cell of activation information.

C-LTM evaluation and execution

Proposal 8: MAC layer handles the event evaluation and determines the C-LTM trigger.

Proposal 9: When more than one candidate cells fulfill the C-LTM triggering condition, then it is up to UE to decide which candidate cell UE would perform C-LTM to.

Observation 5: CG resource reservations from multiple target cells may lead to a significant inefficiency in time/freq resources due to a large number of resource reservations.

Proposal 10a: For RACH-less C-LTM, RAN2 to support an early target cell notification signaling procedure to optimize the CG resource reservation at target cell(s). Details signaling/condition are FFS.

Observation 6: An early target cell notification signaling from Proposal 11a can also help reduce the DG timing uncertainty at candidate cells.

Proposal 10b: Along with the early target cell notification signaling, RAN2 to support DG-based first UL transmission for RACH-less C-LTM.

# References

- RP-242356; Revised Work Item: NR mobility enhancements Phase 4; 3GPP TSG RAN meeting #105; Melbourne, Australia, Sep 9-12, 2024

- Chairman Notes; TSG RAN WG2 Meeting #127bis