3GPP TSG-RAN WG2 Meeting #128	R2-2410020

Orlando, USA, Nov.  18th – 22nd , 2024

Source:	NTT DOCOMO, INC.

Title:	Simulation results on the RRM measurement prediction and discussions

Agenda Item:	8.3.2.1

Document for: 	Discussion and Decision

- Introduction

At RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements, as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

At RAN2 #125bis meetings, it was agreed to start the simulations for the following items regarding the RRM measurement prediction on both FR1 and FR2 and three cases will be considered for the cell-level measurement predictions [2].

Agreements to start evaluations

- FR1-to-FR1

  - Focus on intra-frequency in time domain prediction for the purpose of measurement reduction

  - Study inter-frequency scenario in terms of which scenarios can be studied without requiring new channel model and also resolving any simulation assumptions (if possible).

- FR2-to-FR2

  - Focus on intra-frequency

  - Perform evaluation both in time and spatial domain

Agreements

- For cell level measurement prediction model, at least consider the following cases:

  - Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

  - Case 2: To directly predict cell level results based on cell level results.

  - Case 3: To directly predict cell level results based on beam level results.

At RAN2 #126 meetings, six cases and their priorities were agreed, and the detailed simulation assumptions were listed [4].

The following is agreed

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this paper, we submit our evaluation results about the 3 high-priority cases and discuss some remaining issues on the simulations.

- Performance trends on 3 sub-use cases

At the RAN2 #127bis meeting, the majority view about 3 sub-use cases is the following [6],

Majority of companies observe that among sub cases 1, 2, and 3, at least with shorter prediction window sub case 2 demonstrates the highest prediction accuracy.

To analyze the conditions of this observation further, we take Case 4 as an example case and check the performance of 3 sub-cases under different conditions. After analyzing the results submitted to the RAN2 #127bis meeting, we observed that different AI/ML backbones are used among companies, such as CNN, LSTM, and Transformer (TF). Among them, many companies considered an LSTM-based model backbone for the prediction, and we use the TF. Therefore, we also evaluate the performance with an LSTM-based model besides the TF we used for all cases.

Figure 1 demonstrates the performance of TF and LSTM for 3 sub-cases, where the RSRP difference is the averaged value over a 2s prediction window. When using LSTM, Sub-Case 2 is superior to the others, consistent with the majority view. However, if the TF is used, it performs best with Sub-Case 3, and the performance is also the best among all results.

Figure 1 Average RSRP difference between LSTM and TF for an example prediction.

The different characteristics between the LSTM and TF lead to this performance trend. The LSTM has a memory unit that explores and records the correlations in a series. The LSTM has memory and gates to explore the correlations in the input series. However, when the input dimension or length is high, the gradient-vanishing issue may occur and then cause performance degradation. Therefore, LSTM is more potent for Sub-Case 2, where the input dimension is more minor. The TF has different characteristics in that it explores the global correlations in the input series regardless of length. It can abstract more hidden information and features when the L1-RSRPs are input to the TF, which can be used to produce better predictions.

In summary, we have the following observations based on our analysis and evaluations,

Observation 1

- The performance of 3 sub-cases depends on the characteristics of the AI/ML model backbones used for the evaluations.

  - The TF-based model can extract more information in the raw observations (L1-RSRP), resulting in better prediction accuracy for Sub-Case 3.

  - The LSTM-based one performs better for Sub-Case 2, where the input dimension is more minor.

  - Overall, the TF-based model performs best (w/ Sub-Case 3).

The performance trends of 3 sub-cases depend on implementation issues, where the model backbone may only be one of them. Therefore, we have no substantial evidence to support the down-selection among sub-cases, and we have the following proposal.

Proposal 1

- RAN2 continues the study with sub-cases up to the companies’ preferences.

- Remaining issues on the evaluation results collection

During the Post-RAN2 #127bis email discussions on the results collection, the issues about how to report the results across cells are raised. For this issue, we have the following opinions.

At least for the RRM measurement prediction and measurement event (Event A3) prediction, the measurement predictions inevitably involve multiple cells. The predictions for serving and several strong neighboring cells are necessary for the target cell selection, measurement event prediction, etc.

The AI/ML model may perform differently for predictions in serving or neighboring cells because of the different RSRP levels. The prediction error in different cells also impacts differently on the HO performance. For example, the prediction error in serving and some stronger neighboring cells significantly impact the triggering of the measurement report or cell selection. However, the prediction error in weak neighboring cells impacts nothing, even when significant.

Currently,  the cell type information is not reported along with the simulation results. Different companies may have different assumptions on which cells are predicted. We suggest aligning the assumptions and methods to calculate the RSRP differences to compare the collected results.

Proposal 2

- Evaluate the RRM measurement prediction in serving and Top-K neighboring cells, where K=[1,2,… ].

  - Note: It is up to companies to choose the cell-common, cell-specific, or cluster-based approach.

- RAN2 selects one of the following options for the RSRP difference report.

  - Opt.1: Separately report for serving and neighboring. The RSRP is averaged across neighboring when K > 1.

  - Opt.2: Averaged across all K + 1 cells.

  - Opt.3: Report the RSRP differences averaged across the Top-M (M=1,2,…,K) cells at the prediction time instance.

  - Other approaches.

- Evaluation results

We uploaded the detailed simulation assumptions and results in the following files,

- Scenario 2-128_DCM_2410020_V1.xlsx for Case 2 (FR1-to-FR1 intra-frequency temporal domain Case B).

- Scenario 3-128_DCM_2410020_V1.xlsx for Case 2 (FR1-to-FR1 inter-frequency).

- Scenario 4-128_DCM_2410020_V1.xlsx for Case 2 (FR2-to-FR2 intra-frequency temporal domain Case A).

Some observations from the simulation results are shown in the following sections.

  - Case 2, FR1-to-FR1 intra-frequency temporal domain case B

We evaluated this case with MRRT = 1/2 and 3/4, respectively. The sample-and-hold (SaH) approach, which uses the latest measured results as predictions, is also evaluated as the comparison baseline. The results are also plotted in Figure 2 to show the increasing tendency of the RSRP gap with the prediction distance.

Figure 2 Evaluation results for Case 2.

For Case 2, we have the following observations,

Observation 2

- For Case 2 (FR1-to-FR1 intra-frequency temporal domain case B)

  - Compared the results with MRRT = 0.5 and MRRT = 0.75, when more temporal domain measurements are reduced, the prediction performance is degraded even with the same prediction distance (e.g., 200ms).

  - With the TF-based predictor, the performance of Sub-Case 3 is better.

  - The AI/ML predictor outperforms the SaH for all sub-cases.

  - Case 3, FR1-to-FR1 inter-frequency (frequency domain)

As discussed during RAN2 #127 meeting, we evaluate the performance of Case 3 considering both the prediction for 4 GHz based on the measurements on 2 GHz band and the prediction for 2 GHz based on the measurements on 4 GHz band.

For comparison, the performance of the RSRP offset scheme is also evaluated, where a 6 dB offset is compensated to the RSRP according to the frequency differences between the two bands, and the compensated RSRP is used as the prediction of another band.

Figure 3 Average RSRP difference of Case 3

For Case 3, we have the following observations,

Observation 3

- For Case 3 (FR1-to-FR1 inter-frequency)

  - The AI/ML-based predictor outperforms the RSRP offset scheme.

  - With the TF-based predictor, the performance of Sub-Case 3 is better.

  - Case 4, FR2-to-FR2 intra-frequency temporal domain case A

Figure 4 Evaluation results for Case 4.

For Case 4, 5 measurement samples with 400 ms intervals are predicted. We report the average L3 cell-level RSRP for these 5 predictions in Table 7 and plot them in Figure 8. From the results, we have the following observations,

Observation 4

- For Case 4 (FR2-to-FR2 intra-frequency temporal domain case A)

  - The prediction performance is degraded with the UE velocity.

  - The AI/ML-based predictor outperforms the SaH.

  - With the TF-based predictor, the performance of Sub-Case 3 is better.

For Case 4, the performance of the LSTM-based predictor is also evaluated and reported. The results have been discussed in Section 2.

- Conclusion

In this contribution, we have the following observations and proposals,

Observation 1

- The performance of 3 sub-cases depends on the characteristics of the AI/ML model backbones used for the evaluations.

  - The TF-based model can extract more information in the raw observations (L1-RSRP), resulting in better prediction accuracy for Sub-Case 3.

  - The LSTM-based one performs better for Sub-Case 2, where the input dimension is more minor.

  - Overall, the TF-based model performs best (w/ Sub-Case 3).

Proposal 1

- RAN2 continues the study with sub-cases up to the companies’ preferences.

Proposal 2

- Evaluate the RRM measurement prediction in serving and Top-K neighboring cells, where K=[1,2,… ].

  - Note: It is up to companies to choose the cell-common, cell-specific, or cluster-based approach.

- RAN2 selects one of the following options for the RSRP difference report.

  - Opt.1: Separately report for serving and neighboring. The RSRP is averaged across neighboring when K > 1.

  - Opt.2: Averaged across all K + 1 cells.

  - Opt.3: Report the RSRP differences averaged across the Top-M (M=1,2,…,K) cells at the prediction time instance.

  - Other approaches.

Observation 2

- For Case 2 (FR1-to-FR1 intra-frequency temporal domain case B)

  - Compared the results with MRRT = 0.5 and MRRT = 0.75, when more temporal domain measurements are reduced, the prediction performance is degraded even with the same prediction distance (e.g., 200ms).

  - With the TF-based predictor, the performance of Sub-Case 3 is better.

  - The AI/ML predictor outperforms the SaH for all sub-cases.

Observation 3

- For Case 3 (FR1-to-FR1 inter-frequency)

  - The AI/ML-based predictor outperforms the RSRP offset scheme.

  - With the TF-based predictor, the performance of Sub-Case 3 is better.

Observation 4

- For Case 4 (FR2-to-FR2 intra-frequency temporal domain case A)

  - The prediction performance is degraded with the UE velocity.

  - The AI/ML-based predictor outperforms the SaH.

  - With the TF-based predictor, the performance of Sub-Case 3 is better.

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR”, Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #125bis Chair Notes”, Apr., 2024.

- 3GPP TR 38.901 v17.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”, Mar., 2022.

- RAN2 Chair (InterDigital), “RAN2 #126 Chair Notes”, May, 2024.

- RAN2 Chair (InterDigital), “RAN2 #127 Chair Notes”, Aug., 2024.

- RAN2 Chair (InterDigital), “RAN2 #127bis Chair Notes”, Oct., 2024.