3GPP TSG-RAN WG2 Meeting #128	R2-2410021

Orlando, USA, Nov. 18th – 22nd , 2024

Source: 	Xiaomi

Title:  	Discussion on inter-CU LTM

Agenda Item:	8.6.2

Document for:	Discussion and Decision

Introduction

A Revised WID on NR mobility enhancements Phase 4 [1] has been approved. In the WID, one of the objectives is to specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM), as shown below.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In this contribution, we provide our understandings on inter-CU LTM.

Discussion

  - Inter-CU LTM when DC is not configured

Early sync

RAN2 has discussed the early sync phase in previous meetings, and early TA acquisition has been supported for inter-CU LTM but whether to support UE based TA measurement is FFS. From RAN2 perspective, supporting UE based TA measurement is beneficial for inter-CU LTM, which can reduce the cell switch interruption when the UE doesn’t perform early TA acquisition. To support UE based TA measurement may need to meet some synchronization requirement. Although RAN4 already defined UE requirement for UE based TA measurement for intra-CU LTM, whether the same requirements are applied for Rel-19 inter-CU LTM needs to be confirmed by RAN4. Hence, from RAN2 perspective, UE-based TA measurement for inter-CU LTM could be considered, but it should be finally decided by RAN4.

Proposal 1: From RAN2 perspective, supporting UE-based TA measurement is beneficial for inter-CU LTM, but it should be finally decided by RAN4.

Discussion on SA3’s LS for the security issues

To support subsequent LTM for the inter-CU LTM, how to handle the security key upon cell switch needs to be discussed, as captured in the WID [1]. In the previous meeting, RAN2 has sent an LS [2] to SA3, including many potential solutions as follows: Option 1 to Option 4, and asked SA3 the feasibility of these potential solutions for the security key handling of inter-CU LTM.

Option 1: Use a new information in MAC CE to deliver the security information. Whether the UE uses horizontal or vertical key derivation is derived from this new information in MAC CE (which is currently, neither integrity protected nor ciphered). Option 1A: the NCC value to be used at inter-CU LTM execution is included in the LTM cell switch command MAC CE. Option 1B: the UE is preconfigured with a list of NCC values in a ciphered and integrity protected RRC message and the index of an NCC value in the list is included in the LTM cell switch command MAC CE. Option 2: Similar to Rel-18 S-CPAC key update mechanism, the UE is preconfigured from the source gNB with a list of NCC values per CU using RRC signalling (that is both integrity protected and ciphered). It is expected that the participating gNBs (CUs) would need to be aware of the list and how the UE applies the list during LTM cell switches: Option 2A: UE chooses the first unused NCC for the target CU upon inter-CU LTM execution. Option 2B: As an alternative to choosing the next unused NCC (as in option 2A), horizontal key derivation is used in this option if the LTM cell switch is between the same two CUs. Option 3: After the execution of inter-CU LTM cell switch, the participating gNBs are expected to be updated with new K-gNB* to be used for the next inter-CU LTM cell switch. The UE and CN are aware of how the UE would use the next NCC value. Option 3A: The UE determines the following NCC value to use by itself (e.g., increase by 1) after subsequent inter-CU LTM execution. Option 3B: UE is pre-configured by the CN (via source gNB RRC signalling) with a list of NCC values and the UE chooses the first unused NCC value as the next NCC value. Option 4: After every inter-CU LTM cell switch execution, the UE is provided via RRC signalling with the NCC value to be used by the UE for key derivation at the next inter-CU LTM cell switch.

The Reply LS from SA3 [3] indicates the rekeying method preferred by most companies in SA WG3, as follows:

SA WG3 thanks RAN WG2 for the LS on the security handling for inter-CU LTM in non-DC cases. SA WG3 has been considering the security mechanisms proposed for the inter-CU LTM feature. For this feature, SA WG3 want to preserve the principle that when there is a PDCP anchor change, then it is necessary to perform a key change (i.e. for solutions or handovers that do not require a PDCP anchor change, there is no need to provide rekeying). The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure. SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like, - Security threat of the unprotected MAC CE to deliver the NCC value. - Sending NCC value per candidate cell via RRC message during LTM preparation phase. SA WG3 will keep RAN WG2 and RAN WG3 informed as we make further progress on this topic.

According to the LS from SA3, the preferred method is Option 4, which requires RRC configuration/signalling between cell switches for subsequent LTM procedures. Hence, SA3 want to know whether it is necessary for the RAN WGs to avoid RRC signalling between cell switches. In the WID [1], one goal is to specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches. The motivation of “subsequent LTM aiming to avoid RRC configuration” is to avoid heavy network signalling, such as re-configuring LTM candidate cell configuration, L1 measurement configuration and so on. The RRC configuration/signalling for NCC update is minimal. Moreover, the inter-CU LTM is controlled by the network and the network can guarantee the new NCC is available in UE for the subsequent inter-CU LTM. Option 4 has been provided in RAN2 LS [2], which means RAN2 has reached a consensus that Option 4 is available for the security issue of inter-CU LTM. Hence, RAN2 confirms RRC configuration/signalling between cell switches for key update can be considered.

Proposal 2: For inter-CU LTM, RAN2 confirms RRC configuration/signalling between cell switches for key update can be considered.

Rel-19 LTM based failure recovery
In RAN2#127bis meeting, Rel-19 LTM based fast failure recovery has been discussed and reached the following agreement:

1. The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

RAN2 has agreed to reuse the Rel-18 handling for intra-CU LTM failure. The Rel-18 LTM based failure recovery can also be used for radio link failure, as specified in TS 38.331：

3GPP TS 38.331 V18.3.0 1> if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and 1> if attemptLTM-Switch is configured; and 1> if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG: 2> perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

In Rel-19 LTM, RAN2 supports the configuration with a mixture of intra-CU and inter-CU candidate LTM cells. For radio link failure of the MCG, the Rel-18 handling on failure can also be reused in Rel-19, i.e., in Rel-19, if the UE selects an intra-CU LTM candidate cell after radio link failure of the MCG, the Rel-18 handling on failure can also be reused.

Proposal 3: The Rel-18 handling on failure is reused in Rel-19, if the UE selects an intra-CU LTM candidate cell after radio link failure of the MCG.

Another issue to reuse Rel-18 handling on failure in Rel-19 is how to determine whether the UE selects an intra-CU LTM candidate cell or not after failure. For the issue, reusing Rel-19 set ID is the straightforward method. Specifically, after detecting RLF of the MCG, if the Rel-19 set ID of the selected cell is equal to the Rel-19 set ID of the serving cell, UE can perform cell switch for the selected LTM cell for fast failure recovery. For LTM failure, only intra-CU LTM failure can be recovered through intra-CU LTM, according to the current agreement. Hence, after detecting LTM failure, if the Rel-19 set ID of the selected LTM cell is equal to the Rel-19 set ID of the previous serving cell and the failed LTM cell, UE can perform cell switch for the selected cell for fast failure recovery.

Proposal 4: After detecting RLF of the MCG, if the Rel-19 set ID of the selected LTM cell is equal to the Rel-19 set ID of the serving cell, UE can perform cell switch for the selected cell for fast failure recovery.

Proposal 5: After detecting LTM failure, if the Rel-19 set ID of the selected LTM cell is equal to the Rel-19 set ID of the previous serving cell and the failed LTM cell, UE can perform cell switch for the selected cell for fast failure recovery.

  - Inter-CU LTM when DC is configured

In the WID [1], two secondary priority cases need to be discussed for the inter-CU LTM when DC is configured, as shown below:

- When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

Inter-CU SCG LTM

For inter-CU MCG LTM, RAN2 has agreed “Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch and UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells”. For inter-CU SCG LTM, the mixture of intra-CU and inter-CU SCG LTM can also be supported.

Proposal 6: The UE can be configured with a mixture of intra-CU and inter-CU SCG LTM.

How to configure the UE with a mixture of intra-CU and inter-CU SCG LTM shall be discussed. According to the current agreements and specifications, the Rel-18 intra-CU SCG LTM is configured to UE in SN format (via SN RRCReconfiguration), but the inter-CU SCG LTM configuration is included in the MN RRC configuration message, i.e., provided in MN format. Since the subsequent SCG LTM is supported, if UE is configured with a mixture of intra-CU and inter-CU SCG LTM, when the inter-CU SCG LTM cell switch is executed firstly, the intra-CU SCG LTM needs to be taken as an inter-CU SCG LTM for the subsequent SCG LTM.

For the above description, we provide the Figure 1 as an illustration:

At T1, UE has been configured with a mixture of intra-CU and inter-CU SCG LTM, i.e., the intra-CU SCG LTM for Candidate PSCell 1 and inter-CU SCG LTM for Candidate PSCell 2. At T2, UE has performed the inter-CU SCG LTM cell switch and the current serving PSCell is PSCell 2, and, the LTM configuration for Candidate PSCell 1 has transitioned into an inter-CU SCG LTM. Then, at T3, UE performs the subsequent SCG LTM for candidate PSCell 1, which is the inter-CU SCG LTM.

Figure 1

Hence, a simple way to allow the change from intra-CU to inter-CU is to use a unified format to configure both intra-CU and inter-CU SCG LTM configuration for the mixture case. Rel-18 SCPAC faces a similar issue, and MN format is supported for intra-SN case (in addition to SN format). It seems feasible to use the same method for Rel-19 SCG LTM.

In addition, when the inter-CU SCG LTM in MN format and the intra-CU SCG LTM in SN format are configured to UE at a same time, there are two different LTM candidate ID pool. Because both inter-CU SCG LTM and intra-CU SCG LTM are triggered by the Cell Switch Command MAC CE from the serving SN, the UE cannot determine whether the LTM configuration ID indicated in the SN Cell Switch Command MAC CE is from the MN format ID pool or the SN format ID pool. Therefore, the common LTM candidate ID pool is needed for the mixture of intra-CU and inter-CU SCG LTM. In Rel-18 SCPAC, for one UE, only either MN format or SN format (only intra-SN case is possible) is used. The similar restriction, that the mixture of intra-CU and inter-CU SCG LTM is configured to the UE only in MN format (i.e., MN RRCReconfiguration message), can address the common LTM candidate ID pool issue for both intra-CU and inter-CU SCG LTM. Based on the above analysis, our recommendations are provided in P7 and P8.

Proposal 7: Intra-CU SCG LTM in MN format (i.e., MN RRCReconfiguration message) is supported, in addition to SN format.

Proposal 8: The mixture of intra-CU and inter-CU SCG LTM is configured to the UE only in MN format (i.e., MN RRCReconfiguration message).

Inter-CU MCG LTM when DC is configured

For the inter-CU MCG LTM in DC case, there are two sub-cases:

- Sub-Case 1: Inter-CU LTM where CU is acting as MN and SN is unchanged

- Sub-Case 2: Inter-CU LTM where CU is acting as MN and SN is released

And RAN2 has agreed that SCG configuration can be changed in inter-CU MCG LTM and leave how to handle SCG part up to NW implementation (e.g., release or reconfiguration). As shown in the Figure 2, if the UE is simultaneously configured with MCG LTM candidate configurations that include an SCG configuration and others that involve SN release, when the UE performs an MCG LTM cell switch with SCG release (as shown in step 1 of Figure 2), it raises the question of whether to support the subsequent LTM for the candidate configuration with an SCG configuration (as shown in step 2 of Figure 2), i.e., when an inter-CU MCG LTM cell switch is executed, whether to support the SCG addition.

Figure 2

For inter-CU MCG LTM with SCG addition, there seems no extra work in RAN2. This is because inter-CU MCG LTM with SCG change has already been supported. If inter-CU MCG LTM with SCG addition is supported, some work in RAN3 may be necessary to specify the inter-node interaction for this scenario. Therefore, the decision to support the case of inter-CU MCG LTM with SCG addition could be left to RAN3.

Proposal 9: Whether to support SCG addition when an inter-CU MCG LTM cell switch is executed is up to RAN3.

The coexistence of intra-MN/inter-MN MCG LTM and inter-SN/intra-SN SCG LTM

In RAN2#127bis meeting, RAN2 has confirmed to support the coexistence of inter-MN LTM and intra-SN LTM and the coexistence of inter-SN LTM and intra-MN LTM, and it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen.

Generally, the MN RRCReconfiguration is used to model the candidate configuration and reference configuration of intra-MN and inter-MN MCG LTM. And the SN RRCReconfiguration is used to model the configuration of Rel-18 intra-SN SCG LTM. For Rel-19 inter-SN LTM, RAN2 has agreed the candidate configuration and reference configuration are modelled as an MN RRCReconfiguration message.

As described above, we naturally provide the following observations:

Observation 1: For the coexistence of inter-MN MCG LTM and intra-SN SCG LTM, the MN RRCReconfiguration is used to configure the inter-MN MCG LTM and the SN RRCReconfiguration is used to configure intra-SN SCG LTM.

Observation 2: For the coexistence of intra-MN LTM and inter-SN LTM, the MN RRCReconfiguration is used to configure both intra-MN MCG LTM and inter-SN SCG LTM.

As in Rel-18 LTM, the inter-MN MCG LTM and intra-SN SCG LTM are also configured by and stored in two independent ltm-Config respectively, one is associated with the MCG and the other is associated with the SCG. But both intra-MN MCG LTM and inter-SN SCG LTM are configured by and stored in the ltm-Config associated with the MCG. Thus, it shall to be discussed how to configure intra-MN MCG LTM and inter-SN SCG LTM by the ltm-Config associated with the MCG at the same time. In the L3 conditional reconfiguration, both CHO and inter-SN CPC are configured by one conditionalReconfiguration associated with MCG. Similar to the coexistence of CHO and inter-SN CPC, a simple method is to use one ltm-Config associated with MCG to configure both intra-MN MCG LTM and inter-SN SCG LTM, and the unified LTM candidate ID pool is used for both.

Proposal 10: A single ltm-Config is used to configure both intra-MN MCG LTM and inter-SN SCG LTM, and the unified LTM candidate ID pool is used for both.

The Rel-18 ltm-Config associated with MCG only includes one reference configuration. We need to analyse whether a single reference configuration is suitable for both intra-MN MCG LTM and inter-SN SCG LTM. For the reference configuration of inter-SN SCG LTM, RAN2 has reached the following agreements:

4. Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline: - The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs.

The reference configuration of inter-SN SCG LTM is modelled as an MN RRCReconfiguration message and can include the MCG part based on the NW implementation. In Rel-19, SCG configuration can be changed in MCG LTM. So, it is sufficient to have one reference configuration to support the coexistence of the intra-MN MCG LTM and inter-SN SCG LTM.

Proposal 11: Only one reference configuration is configured for the simultaneous configuration of intra-MN MCG LTM and inter-SN SCG LTM.

Conclusions

According to the analysis given above, we have the following observations and proposals:

2.1	Inter-CU LTM when DC is not configured

Early sync

Proposal 1: From RAN2 perspective, supporting UE-based TA measurement is beneficial for inter-CU LTM, but it should be finally decided by RAN4.

Discussion on SA3’s LS for the security issues

Proposal 2: For inter-CU LTM, RAN2 confirms RRC configuration/signalling between cell switches for key update can be considered.

Rel-19 LTM based failure recovery

Proposal 3: The Rel-18 handling on failure is reused in Rel-19, if the UE selects an intra-CU LTM candidate cell after radio link failure of the MCG.

Proposal 4: After detecting RLF of the MCG, if the Rel-19 set ID of the selected LTM cell is equal to the Rel-19 set ID of the serving cell, UE can perform cell switch for the selected cell for fast failure recovery.

Proposal 5: After detecting LTM failure, if the Rel-19 set ID of the selected LTM cell is equal to the Rel-19 set ID of the previous serving cell and the failed LTM cell, UE can perform cell switch for the selected cell for fast failure recovery.

2.2	Inter-CU LTM when DC is configured

Inter-CU SCG LTM

Proposal 6: The UE can be configured with a mixture of intra-CU and inter-CU SCG LTM.

Proposal 7: Intra-CU SCG LTM in MN format (i.e., MN RRCReconfiguration message) is supported, in addition to SN format.

Proposal 8: The mixture of intra-CU and inter-CU SCG LTM is configured to the UE only in MN format (i.e., MN RRCReconfiguration message).

Inter-MN MCG LTM when DC is configured

Proposal 9: Whether to support SCG addition when an inter-CU MCG LTM cell switch is executed is up to RAN3.

The coexistence of intra-MN/inter-MN MCG LTM and inter-SN/intra-SN SCG LTM

Observation 1: For the coexistence of inter-MN MCG LTM and intra-SN SCG LTM, the MN RRCReconfiguration is used to configure the inter-MN MCG LTM and the SN RRCReconfiguration is used to configure intra-SN SCG LTM.

Observation 2: For the coexistence of intra-MN LTM and inter-SN LTM, the MN RRCReconfiguration is used to configure both intra-MN MCG LTM and inter-SN SCG LTM.

Proposal 10: A single ltm-Config is used to configure both intra-MN MCG LTM and inter-SN SCG LTM, and the unified LTM candidate ID pool is used for both.

Proposal 11: Only one reference configuration is configured for the simultaneous configuration of intra-MN MCG LTM and inter-SN SCG LTM.

Reference

[1] RP-242356, “Revised WID: NR mobility enhancements Phase 4”, Apple Inc, China Telecom

[2] R2-2404037, “LS on security handling for inter-CU LTM in non-DC cases”

[3] S3-244316, “Reply LS on security handling for inter-CU LTM in non-DC cases”