3GPP TSG-RAN WG2 Meeting #128	R2-2410023

Orlando, USA, Nov.  18th – 22nd , 2024

Source:	NTT DOCOMO, INC.

Title:	Discussions on evaluation methodology of AI/ML for mobility

Agenda Item:	8.3.5

Document for: 	Discussion and Decisions

- Introduction

During the RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements, as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

This paper will discuss the detailed evaluation methodology issues for the measurement event predictions.

- Evaluation methodology on the generalization performance

Based on the agreements from the last meeting [2], we discuss the details of the generalization evaluation in this paper. During the Rel. 18 studies about AI/ML for PHY, the generalization evaluation performance of AI/ML models is verified by testing its performance with different training and test datasets [3]. Configuration #A and Configuration #B are considered for the distribution variations concerning one or multiple generalization aspects. For example, Configuration #A is 30 kmph UE velocity, while Configuration #B is 60 kmph. Then the generalization performance is evaluated with the following three generalization cases, which are

- Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B.

  - Note: GC#1 is the baseline for the generalization evaluations.

- Generalization Case #2 (GC#2): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B.

- Generalization Case #3 (GC#3): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

The generalization performance of AI/ML can be verified by comparing the performance of GC#2 and GC#3 with GC#1. If a minor performance loss is observed, the generalization can be achieved. The same methodology can be used for the AI/ML Mobility studies. Therefore, we propose,

Proposal 1

- Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following 3 cases,

  - Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B.

    - Note: GC#1 is the baseline for the generalization evaluations.

  - Generalization Case #2 (GC#2): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B.

  - Generalization Case #3 (GC#3): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

The generalization study aims to verify if the generalization aspects introduce significant performance degradation. Therefore, the relative performance regarding intermediate KPIs can be used, and it is unnecessary to consider the final handover KPIs.

Proposal 2

- Use the intermediate KPIs for the generalization study. The relative performances of intermediate KPIs between GC#2 / GC#3 and GC#1 are used.

Regarding the detailed configurations for Configuration #A and #B, the cases under study involve temporal, frequency, and spatial domain predictions. The different aspects of the generalization should be studied for the predictions on different domains.

For the temporal domain prediction, the most critical aspect of mobility is the UE speed. In practical scenarios, the moving speed of the users is distributed in an extensive range and variant from time to time. The AI/ML models should be robust against the UE speed. Other aspects of the temporal domain include the different measurement intervals and OW/PW configurations.

For the frequency domain prediction, the frequency gap between two bands is an issue that should be analyzed due to the complex practical spectrum allocations and deployments.

For the spatial domain prediction, the different TXRU, tilt angle, and SSB/CSI-RS beam patterns significantly impact the consistency of the spatial domain distribution, which can be studied.

Proposal 3

- Study the generalization aspects according to the prediction domains of the use cases.

  - For Case 2 and 4 (temporal domain), the aspects are

    - UE speed,

    - Temporal domain configurations (measurement period, observation window length, prediction window length).

  - For Case 3 (frequency domain), the aspects are

    - The carrier frequencies and frequency gap between the two bands.

  - For Case 6 (spatial domain), the aspects are

    - TXRU mapping, tilt angle, and SSB/CSI-RS beam number and pattern.

- Remaining issues from Post-RAN2 #127bis email discussions

In the Post-RAN2 #127bis [022][AI mobility] email discussions, the HO models for the system-level simulations were discussed based on the following proposals.

The HO model for FR2 temporal domain case A is defined as following:

- If a predicted A3 event at future t1 is reported at t0 and one potential real A3 event is triggered at t2, then HO command is transmitted at t3, where t3=t0+ max(HO prep time, min(t1-t0,t2-t0)). After that one fixed execution time is assumed. For direct prediction, t1 is the middle of the event occurrence window.

The HO model for FR1 temporal domain case B is defined as following:

- If a predicted A3 event at t1 is reported at t0 (t0<=t1) then HO command is transmitted at t3, where t3=t0+max(HO prep time, t1-t0). After that one fixed execution time is assumed.

We have the following comments and proposals on the HO models. For Case 4 (FR2 temporal domain Case A), when the predicted Event A3 is triggered earlier than the legacy Event A3, it gives the NW more margin to choose a suitable instance to send the HO command. The measured and predicted measurement events coexist, and the NW may receive two measurement reports corresponding to the prediction and actual measurements. The contents reported with these events have not yet been discussed in RAN2. We have no clear solution for deciding which event should be used and what should be reported to the NW for the HO decisions. Without the simulations, it is hard to judge if the current proposal is optimal. Therefore, we prefer leaving some optimization spaces at the current stage. The proposed revision is,

Proposal 4

- The HO model for FR2 temporal domain case A (Case 4) is defined as the following:

  - If a predicted A3 event at future t1 is reported at t0 and one potential real A3 event is triggered at t2, then HO command is transmitted at t3, where (t0 + HO prep time) <= t3 <= (t0+ max(HO prep time, max(t1-t0, t2-t0))). After that, one fixed execution time is assumed. For direct prediction, t1 is the middle of the event occurrence window.

    - Further study with evaluations to narrow down the value of t3.

Our study goal for Case 2 (FR1 temporal domain Case B) is the measurement overhead reduction. Therefore, we do not expect the HO performance gain for this case. The objective of the system-level simulations is to verify if the predictions are good enough to replace the actual measurements. Therefore, we can trigger the measurement report and send the HO command as usual, and see if the predictions significantly impact HO performance.

The same methodology can be applied to other cases with the same study goal (overhead reduction). Therefore, we have the following proposal,

Proposal 5

The HO model for FR1 temporal domain case B and inter-frequency prediction is defined as the following:

- The predicted A3 event at t1 is reported at t1, then HO command is transmitted at t3, where t3=t1+HO prep time. After that, one fixed execution time is assumed.

- Conclusion

In this contribution, we have the following observations and proposals,

Proposal 1

- Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following 3 cases,

  - Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B.

    - Note: GC#1 is the baseline for the generalization evaluations.

  - Generalization Case #2 (GC#2): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B.

  - Generalization Case #3 (GC#3): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

Proposal 2

- Use the intermediate KPIs for the generalization study. The relative performance of intermediate KPIs between GC#2/#3 and GC#1 is used.

Proposal 3

- Study the generalization aspects according to the prediction domain of the use cases.

  - For Case 2 and 4 (temporal domain), the aspects are

    - UE speed,

    - Temporal domain configurations (measurement period, observation window length, prediction window length).

  - For Case 3 (frequency domain), the aspect is

    - The carrier frequencies and frequency gap between the two bands.

  - For Case 6 (spatial domain), the aspects are

    - TXRU mapping, tilt angle, and SSB/CSI-RS beam number and pattern.

Proposal 4

- The HO model for FR2 temporal domain case A (Case 4) is defined as the following:

  - If a predicted A3 event at future t1 is reported at t0 and one potential real A3 event is triggered at t2, then HO command is transmitted at t3, where (t0 + HO prep time) <= t3 <= (t0+ max(HO prep time, max(t1-t0, t2-t0))). After that, one fixed execution time is assumed. For direct prediction, t1 is the middle of the event occurrence window.

    - Further study with evaluations to narrow down the value of t3.

Proposal 5

The HO model for FR1 temporal domain case B and inter-frequency prediction is defined as the following:

- The predicted A3 event at t1 is reported at t1, then HO command is transmitted at t3, where t3=t1+HO prep time. After that, one fixed execution time is assumed.

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR,” Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #127bis Chair Notes”, Oct., 2024.

- 3GPP TR 38.843 v2.0.0, Study on Artificial Intelligence (AI)/Machine Learning(ML) for NR air interface (Release 18), Dec., 2023.