3GPP TSG-RAN WG2 #128	R2-2410064

Orlando, USA, Nov 18th – 22nd, 2024

Source:		NEC

Agenda item:		8.6.4	Conditional intra-CU LTM

Title:	Discussion on conditional intra-CU LTM

Document for:		Discussion and decision

- Introduction

During last RAN2 meeting, there was an initial discussion on intra-CU LTM and the following agreements were made [1]:

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In this paper, we provide our further consideration on conditional intra-CU LTM.

- Discussion

2.1 Early synchronization

After configuring conditional LTM for the UE, the network may trigger early UL/DL synchronization for the UE. In Rel-18, the network determines to trigger early DL/UL synchronization based on L1/L3 measurement report from the UE. During last meeting, there was a short discussion on some further enhancements e.g., whether to support “conditional early UL/DL early synchronization”, i.e., the UE autonomously performs early DL/UL synchronization when an L1/L3 measurement event is fulfilled, or the UE transmits an indication to the network to request for early UL/DL early synchronization. In our understanding, the benefit of those enhancements is only for signalling overhead reduction for the early sync phase, which is not the target of conditional LTM. And since we will support event triggered L1 measurement report, the signalling overhead is already reduced compared with Rel-18, so there is no need for such enhancements and we can just rely on L1/L3 measurement report from the UE.

Proposal 1. Early UL/DL synchronization is triggered by network based on L1/L3 measurement report from the UE, i.e., no further enhancement is needed for conditional LTM.

During last meeting, it was agreed that PDCCH ordered early TA acquisition is supported for conditional LTM. In Rel-18, the timing advance of the target candidate cell is sent to the UE by LTM cell switch command. For conditional LTM, since there is no LTM cell switch command transmitted to the UE, the source cell can transmit the timing advance command to the UE by using MAC CE prior to the execution of LTM cell switch execution.

Proposal 2: Source cell can send to the UE a MAC CE comprising timing advance command for the LTM candidate cells.

The UE, after reception of the MAC CE comprising the timing advance command for the LTM candidate cells, should perform TA maintenance for the LTM candidate cells, by starting or restarting time alignment timer associated with the LTM candidate cells.

Currently, for TA maintenance of serving cells, the concept of timing advance group (TAG) is supported. The serving cells associated with the same TAG use the same timing reference cell and the same Timing Advance value. Each TAG is associated with one time alignment timer. And at most four TAGs can be configured for one cell group. For TA maintenance of candidate cells, if the UE maintains TA and time alignment timer for each of LTM candidate cells, it means that the UE needs to at most maintain 8 additional TA and running 8 additional time alignment timers for one cell group, even if some of the LTM candidate cells are located in the same DU or TRP. Therefore, we think it is beneficial to apply the concept of TA group to the TA maintenance of LTM candidate cells.

Proposal 3: After reception of timing advance command for the LTM candidate cell, the UE starts or restarts time alignment timer associated with the LTM candidate cell.

Proposal 3a: RAN2 to support TA group for TA maintenance of the LTM candidate cells.

2.2 LTM cell switch execution

Upon initiation of LTM cell switch execution, the MAC layer shall determine whether RACH-less LTM can be performed. And one factor for initiating RACH-less LTM is that the UE has a valid TA for the target LTM candidate cell. To determine whether the TA for the target candidate cell is valid, at least the running of time alignment timer associated with the LTM candidate cell should be taken into account, i.e., if the time alignment timer expires, the UE shall consider that there is no valid TA for the LTM candidate cell.

For Rel-19 LTM, to avoid large signalling overhead, the network may not have measurement report from the UE to tigger anther early UL sync for the LTM candidate cell in time. For example, the UE is only configured with event based L1 measurement report for one LTM candidate cell, and if the L1 report indicates that the candidate cell’s radio condition is good enough, the source cell triggers PDCCP order for early TA acquisitor for the UE. However, when the channel condition between the UE and candidate cell changes, the source cell may be not able to trigger another PDCCH ordered early UL sync, because the event triggered the L1 measurement report is not fulfilled. Therefore, it is beneficial for the UE to also take the change of measurement result for the candidate cell into account when determining the TA validity, i.e., if the measurement result (e.g., RSRP) of the LTM candidate cell changes above a threshold compared to the measurement result when the TA is received, the TA is considered as invalid, similar as it is supported in SDT.

Proposal 4: UE determines whether there is valid TA for RACH-less conditional LTM by taking both the running of time alignment timer and the change of measurement result for the candidate cell into account.

At last meeting, RAN agreed to support CG-based RACH-less LTM. And for CG-based RACH-less LTM, the UE needs to perform a beam selection to further determine valid CG resource for first PUSCH transmission. And how to select the beam for CG-based RACH-less is one remaining issue. Since the L1 event for execution condition is one or more beams associated with the target candidate cell fulfils a condition, which means that the beam fulfilling the condition are stable good enough, therefore the selected beam should be at least one beam fulfiling the conditions of L1 event.

Proposal 5: For RACH-less conditional LTM, UE selects a beam fulfilling the L1 event of the execution condition.

To faster the delay caused by the DL synchronization, it is also beneficial that if the selected beam is associated with the activated TCI state. In case that there are multiple beams fulfilling the L1 event, and part of the multiple beams is associated with activated TCI state, the UE should select beam which is also associated with activated state.  An alternative way is that the UE only evaluate beams associated with activated TCI state for L1 event fulfilment, however this requires that early DL synchronization is always performed for CLTM:

Proposal 5a: RAN2 to further down-selects between the following options:

- Option 1. If there are multiple beams fulfils the L1 event condition, UE selects from the beams which are also associated with activated TCI state

- Option 2. UE only evaluates the beams which are associated with activated TCI states for L1 event condition fulfilment

At last meeting, RAN agreed to support CG-based RACH-less LTM, and FFS on whether to support DG based LTM. For CLTM, since the network does not have information of when the LTM cell switch is triggered, and which beam or TCI state will be selected or used by the UE, it cannot provide UL grant for dynamic scheduling of the UE properly. And a “bye message” from the UE to the network would introduce additional latency and complexity, therefore we think there is no need to support DG-based RACH-less conditional LTM.

Proposal 6: Do not support DG-based first UL transmission for RACH-less conditional LTM.

2.3 Support of L3 event based conditional LTM

At previous meeting, it was agreed that LTM3-like and LTM5-like are used as the conditional LTM execution condition, and it is gNB-DU to generate the L1 event condition. And whether to support CHO conditions for LTM is FFS. In Rel-18 LTM discussion, it was agreed that the UE capability for LTM does not need to be coupled with L1 measurement capability, and therefore LTM can be triggered by L3 measurement. Therefore, similarly, due to the decuple of LTM capability and L1 measurement capability, we should support L3 event as execution condition for conditional LTM. And for this case, it should be gNB-CU to generate the L3 event based execution conditions.

Proposal 7: Support L3 event (CondEventA3 and CondEventA5) as execution conditions for conditional LTM.

Proposal 7a: It is CU to generate the L3 event based execution conditions for conditional LTM.

In case of L1 event triggered conditional LTM, same as Rel-18 LTM, it is MAC layer to determine the triggering of LTM and indicate to the RRC layer. For the case of L3 event triggered conditional LTM, it should be the RRC layer to determine the triggering of LTM, and indicates to the MAC layer, and then the MAC layer determines whether to performs RACH-less LTM.

Proposal 7b: For L3 event based conditional LTM, RRC layer indicates the initiation of LTM cell switch to MAC layer, and then the MAC layer determines whether to perform RACH-less LTM after reception of the indication.

2.4 LTM recovery

In Rel-18, LTM recovery is only RACH-based, even if the UE has measured TA for the target cell of the LTM recovery. For conditional LTM, the UE may have TA value of the candidate cell provided by the network, or measured by itself, and RACH-less conditional LTM can be performed. For LTM recovery, one question is whether the LTM recovery to a candidate cell of conditional LTM can be also RACH-less or RACH-based only. In our understanding, except the triggering conditions and beam selection, there is no other difference between the initial LTM cell switch and the LTM recovery for RACH-less based conditional LTM. And we think RACH-less based LTM recovery can be easily supported in Rel-19 to further reduce the latency of LTM recovery procedure.

Proposal 8: Support RACH-less based LTM recovery to a candidate cell of conditional LTM.

# 3. Conclusion

In this paper, we provide our initial consideration for the support of CLTM, and have the following proposals:

Early synchronization

Proposal 1. Early UL/DL synchronization is triggered by network based on L1/L3 measurement report from the UE, i.e., no further enhancement is needed for conditional LTM.

Proposal 2: Source cell can send to the UE a MAC CE comprising timing advance command for the LTM candidate cells.

Proposal 3: After reception of timing advance command for the LTM candidate cell, the UE starts or restarts time alignment timer associated with the LTM candidate cell.

Proposal 3a: RAN2 to support TA group for TA maintenance of the LTM candidate cells.

LTM cell switch execution

Proposal 4: UE determines whether there is valid TA for RACH-less conditional LTM by taking both the running of time alignment timer and the change of measurement result for the candidate cell into account.

Proposal 5: For RACH-less conditional LTM, UE selects a beam fulfilling the L1 event of the execution condition.

Proposal 5a: RAN2 to further down-selects between the following options:

- Option 1. If there are multiple beams fulfils the L1 event condition, UE selects from the beams which are also associated with activated TCI state

- Option 2. UE only evaluates the beams which are associated with activated TCI states for L1 event condition fulfilment

Proposal 6: Do not support DG-based first UL transmission for RACH-less conditional LTM.

Support of L3 event based conditional LTM

Proposal 7: Support L3 event (CondEventA3 and CondEventA5) as execution conditions for conditional LTM.

Proposal 7a: It is CU to generate the L3 event based executions condition for conditional LTM.

Proposal 7b: For L3 event based conditional LTM, RRC layer indicates the initiation of LTM cell switch to MAC layer, and then the MAC layer determines whether to perform RACH-less LTM after reception of the indication.

LTM recovery

Proposal 8: Support RACH-less based LTM recovery to a candidate cell of conditional LTM.

# Reference

[1] meeting report of RAN2 #127bis.