3GPP TSG RAN WG2 Meeting #128		R2-2410068

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	8.2.3

Source:	Sharp

Title:	New and retransmitted A-IoT paging indication and device behaviors

Document for:	Discussion and Decision

- Introduction

An AIoT readed can send multiple ambient IoT(A-IoT) paging messages for the same request from the core network. In RAN2 #127bis [1], it was agreed that some information can be used to indicate whether a paging is new or a retransmission, ans the device may skip sending the response to paging based on the information, as shown below.

Agreements on AIoT paging

1	The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

2	Based on this information the device determines whether to skip sending the response to paging.

3	From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

4	If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

In this contribution, as an update of [2], we discuss what information can be indicated to differentiate a supplicate paging from a new paging message, and the triggering conditions for a paging retransmission from the reader to enable devices retransmissions of Msg1 for radom access.

Furthermore, we provide our views on the device behaviors, e.g. how to response to a paging message depending on the conditions of an ongoing paging and/or the device capability.

- New or duplicate indication in a paging message

A paging message is the first message from a reader to initiate any AIoT initial access or data transmission. Different paging functions may be defined for unicast, broadcast, group cast and multicast, etc. At least a 2-step and a 3-step random access (RA) procedures should be supported for “inverntory only” and “inventory and command” use cases, as discussed in RAN2.

A readed can send multiple ambient IoT(A-IoT) paging messages for the same request from the core network, i.e. the same paging process. Therefore, some information is needed indicate the duplicate paging message by the reader, so that the A-IoT devices can detect whether a paging messaage is new paging or a duplicate message of an ongoing paging. Depending on the type of paging, an A-IoT device can determine whether a new random access procedure should be performed or not.

To identify a paging message, as a minimum, a paging ID can be included in a paging PRDCH Msg0. The paging ID can be increased by one for each new paging process and cyclically reused based on the ID length.

If there are multiple AIoT readers, it is possible more than one reader may select the same paging ID at the same time and cause paging ID collision. If the paging ID is big enough, the collision probability can be negligible. To avoid the ambiguity of paging from different readers, a reader ID may be included in the paging RDCH Msg0 together with the paging ID. The combination of paging ID and reader ID may define a unique paging process. An AIoT device can differentiate a different paging process from different readers as well based on the reader ID.

To differentiate an initial paging message from a duplicate paging message, a paging re-transmission counter may be included in the paging message for a given paging ID. The paging re-transmission counter is reset to 0 for each new paging process with a new paging ID, and increased by 1 for each paging message re-transmission for the same paging ID. Alternatively, new and duplicate pagings can be indicated by 1 bit only, e.g. 0 for initial transmission and 1 for retransmission.

An AIoT device may determine the paging message is a re-transmission of an existing paging process by the paging counter. If the counter is 0, the paging Msg0 is a new paging message, and if the counter is greater than 0, the paging message is a re-transmission of the same paging process. Furthermore, the AIoT device may determine whether one or more paging messages is(are) not detected by observing if some paging counters are missed.

Proposal 1: A paging message Msg0 can include a paging ID, a reader ID and a paging retransmission counter for a device to determine if a paging is a new one or a retransmission of an existing paging.

- Triggering conditions for duplicate paging messages

For a paging that triggers contention based random access (CBRA), the AIoT device will perform contention access with slotted Aloha in a contention access region. Each contention slot can be defined as a random access occasion (RO). The reader listens to the channel for Msg1 reception in each RO, and may have three possible observations and reception status:

- Successfully receive and decode a Msg1 with a randomID and/or actual ID

- Detected energy with Msg 1 transmissions, but cannot decoded anything successfully. Thus implies a collision of Msg1 occurs, i.e. more than one AIoT devices transmit Msg1 in the same RO.

- Idle. No energy detected in the RO, i.e. no AIoT device selects this RO for Msg1 transmission.

An AIoT reader may need to send multiple paging messages for the same paging process. The reader may decide to retransmit Msg0 for a variety of reasons. For example, the reader may send multiple paging messages at different directions to cover more AIoT devices, or the reader may simply expect more AIoT responses or doublecheck if there are remaning devices for response.

The reader should be required to send a duplicate paging message if collision is observed in the CBRA procedure of the previous paging message, esp.

- If the reader detects collisions in one or more ROs, a duplicate paging message will allow the AIoT devices involved in the collision to retransmit Msg1 for collision resolution.

- If the reader detects more than one AIoT devices reported the same random ID. A duplicate paging message should be triggered for these devices to retransmit with new random IDs for collision resolution.

Thus, the status of each RO in the contention access region can be observed and obtained at the reader. And the reader can determine whether a duplicate paging message should be transmitted for the same paging process, or a new paging process should be initiated.

Proposal 2: A paging message retransmission should be triggered at least when

- Channel collision is detected in one or more ROs in the contention access region, or

- Same random ID is reported from more than one AIoT devices.

- Device capabilities and behaviors with a paging message detection

With new and re-transmission indication in a paging message, the device can determine whether to response to the paging with a random access procedure. A RA process can be started if a new paging is deteceted. For a retransmitted paging message, the device behavior should be determined based on the conditions of the onging paging process.

Using 3-step RA as an example,

- If the device didn’t receive the previous paging and is not in a onging RA procedure, the device can treat the re-transmitted paging as a new paging and starts a new RA procedure by sending a Msg1.

- If the device is already in a RA process of the same paging and Msg1 is already transmitted but Msg2 is not received, the device should decide wheter it should wait the Msg2 response or re-transmit a Msg1 follwing the paging retransmission, e.g. a Msg2 response window may be defined.

- If the device already transmitted Msg 3 in response to a Msg2 for the same paging, the device may assume that the paging process is completed and ignore the received retransmitted paging message.

Proposal 3: The device behaviors for a retransmitted paging message should be determined based on the conditions of the onging RA process of the same paging.

Additionally, if a new paging is receiced, the AIoT may consider whether there is another ongoing RA procedures in progress for another paging process. The other paging process may be triggered by the same reader or a different reader. In the later case, a reader sending the paging message may not know the existence of other readers and/or another paging processes from other readers.

Thus, the device behavior may be determined by the supported number of simultaneous paging processes by the device.  The number can be defined as a device capability, and should be very limited, e.g. only 1 or 2. In case of multiple paging processes are supported, the AIoT device may maintain multiple paging processes separately, esp. if they are from different readers.

Otherwise, the device should decide which paging process is prioritized between the onging paging and the newly received paging.

If there is no other ongoing RA process for another paging process, the AIoT device should perform RA and transmit Msg1 following the new paging Msg0.

If there is another ongoing RA procedure for another paging process, the device should prioritize an existing ongoing RA procedure over a new paging process. Thus, the AIoT device should ignore the new paging Msg0, and continue the RA procedure for the other paging process. The AIoT device may start the new RA process for the new paging Msg0 after the existing RA procedure is completed or wait for another Msg0 retransmission.

Proposal 4: The device should check if there is(are) ongoing paging(s)/RA process(es), and device behaviors may be different based on the capability of whether simulateneous paging processes is supported.

- Conclusions

In this contribution, we discuss the methods to indicate a new or a retransmitted paging message, and the conditions that a paging message should be retransmitted. We propose:

Proposal 1: A paging message Msg0 can include a paging ID, a reader ID and a paging retransmission counter for a device to determine if a paging is a new one or a retransmission of an existing paging.

Proposal 2: A paging message retransmission should be triggered at least when

- Channel collision is detected in one or more ROs in the contention access region, or

- Same random ID is reported from more than one AIoT devices.

Also, the AIoT device behaviors are discussed for a new or an existing paging process at different conditions of a RA process. We propose

Proposal 3: The device behaviors for a retransmitted paging message should be determined based on the conditions of the onging RA process of the same paging.

Proposal 4: The device should check if there is(are) ongoing paging(s)/RA process(es), and device behaviors may be different based on the capability of whether simulateneous paging processes is supported.

- References

- Draft Report of 3GPP TSG RAN WG2 meeting #127bis, v2

- R2-2408954, Duplicate A-IoT paging message transmission and indication, Sharp Corporation