3GPP TSG-RAN WG2 Meeting #128	           R2-2410104

Orlando, US, Nov 18th – Nov 22nd, 2024

Agenda item:	8.13.2

Source:	China Telecom

Title:	Multi-hop relay discovery and reselection

Document for:	Discussion

- Introduction

RAN2#127 meeting had an initial discussion on terminologies and relay discovery and made some agreements. In this paper, we would like to further discuss the potential enhancements to relay discovery and (re)selection to support additional hops, such as discovery conditions for the intermediate relay UEs, relay (re)selection radio criteria, relay reselection trigger, backward compatibility and so on.

- Discussion

2.1	General

Regarding backward compatibility, in our understanding, in order to make more Remote UEs located in poor coverage able to communicate with the network via Relay UE(s). RAN2 should try to enable Rel-17 U2N Remote UE to communicate with the network via Rel-19 U2N Relay UE(s) when designing multi-hop relay mechanism.

Proposal 1: RAN2 should try to enable Rel-17 U2N Remote UE to communicate with the network via Rel-19 U2N Relay UE(s) when designing multi-hop relay mechanism.

2.2	Relay discovery

RAN2 has the following agreements on discovery conditions [1]:

A candidate last relay UE for discovery or (re)selection can be in any RRC state, but not outside network coverage. FFS if acting as a relay requires it to transition to RRC_CONNECTED. Some or all of the intermediate relay UEs on a multi-hop indirect path can be inside or outside the network coverage. If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18. If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds). FFS discovery conditions for the intermediate relay UEs.

The Rel-17 U2N Relay UE uses a maximum Uu RSRP threshold, a minimum Uu RSRP threshold, or both, which are configured by the network, to determine if it can transmit Relay discovery messages to U2N Remote UE(s). Obviously, this is also applicable for last relay UE (i.e. the U2N relay UE directly connected to the gNB) in Rel-19, which was agreed during RAN2#127 meeting. However, according to the agreements, some or all of the intermediate relay UEs on a multi-hop indirect path can be inside or outside the network coverage. If intermediate Relay UE is out of coverage, the Uu RSRP measurement is unavailable for intermediate Relay UE, such as first relay and second relay. Thus the Rel-17 Uu RSRP threshold can’t be used for OOC intermediate Relay UE.

Take one additional hop relay scenario for example, first relay UE out of coverage can use a maximum SD-RSRP/SL-RSRP threshold, a minimum SD-RSRP/SL-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s). The maximum SD-RSRP/SL-RSRP threshold, and minimum SD-RSRP/SL-RSRP threshold can be configured by the network via RRC signalling. With the extended threshold conditions, the interference introduced by first relay UE is limited while close enough to last relay UE. In addition, the extended threshold conditions also apply to two additional hops relay scenario.

Proposal 2: Intermediate relay UEs (i.e. first relay UE or second relay UE) can use a maximum SD-RSRP threshold, a minimum SD-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

Proposal 3: Intermediate relay UEs (i.e. first relay UE or second relay UE) can use a maximum SL-RSRP threshold, a minimum SL-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

2.3	Relay selection/reselection

The Rel-17 U2N Remote UE performs SL-RSRP/SD-RSRP measurements and uses them for Relay selection and reselection radio criteria. A Rel-17 U2N Relay UE is considered suitable by a Rel-17 U2N Remote UE in terms of radio criteria if the SL-RSRP/SD-RSRP measured by U2N Remote UE towards the U2N Relay UE exceeds configured threshold. The SL-RSRP/SD-RSRP threshold introduced for relay selection/reselection in Rel-17 can be reused for Rel-19 U2N Remote UE to select/reselect multi-hop relay UE, since the expected minimum PC5 signal quality of each hop can be the same.

Proposal 4: the Rel-17 SL-RSRP/SD-RSRP threshold of relay (re)selection radio criteria can be reused for Rel-19 U2N Remote UE (re)selecting multi-hop relay UE.

Rel-17 defined the following cases triggering U2N Relay UE reselection:

-	PC5 signal strength of current U2N Relay UE is below a (pre)configured signal strength threshold;

-	Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by U2N Relay UE via PC5-RRC signalling;

-	When U2N Remote UE receives a PC5-S link release message from U2N Relay UE;

-	When U2N Remote UE detects PC5 RLF;

-	Indicated by upper layer.

However, in terms of the second bullet, for multi-hop relay, there may be some new cases. Single-hop U2N Relay UE communicates with the network directly, which can be considered as last relay UE in multi-hop relay. The first relay UE may perform path switch, such as switch from indirect path to direct path, indirect path to indirect path. In that case, the first relay UE may need to indicate path switch to the connected remote UE, which may trigger the remote UE to perform relay UE reselection. In another case, the first relay may reselect its relay UE, and the reselected relay UE’s serving cell may be different from or the same as the former serving cell. Thus we think Rel-19 need to introduce new indication type to trigger U2N Relay UE reselection, such as path switch.

Proposal 5: introduce new indication type to trigger U2N Relay UE reselection, such as path switch.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: RAN2 should try to enable Rel-17 U2N Remote UE to communicate with the network via Rel-19 U2N Relay UE(s) when designing multi-hop relay mechanism.

Proposal 2: Intermediate relay UEs (i.e. first relay UE or second relay UE) can use a maximum SD-RSRP threshold, a minimum SD-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

Proposal 3: Intermediate relay UEs (i.e. first relay UE or second relay UE) can use a maximum SL-RSRP threshold, a minimum SL-RSRP threshold, or both, to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

Proposal 4: the Rel-17 SL-RSRP/SD-RSRP threshold of relay (re)selection radio criteria can be reused for Rel-19 U2N Remote UE (re)selecting multi-hop relay UE.

Proposal 5: introduce new indication type to trigger U2N Relay UE reselection, such as path switch.

4	References

- [1] R2-2409210, Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands.