3GPP TSG-RAN WG2 Meeting #128	R2-2410118

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.6.2

Source:	MediaTek Inc.

Title:	Leftover issues on Inter-CU LTM

Document for:	Discussion, Decision

Introduction

This contribution further discusses the LS from SA3 and some leftover issues for inter-CU SA LTM.

Discussion

Discussion on the LS from SA3

In this meeting, an LS from SA3 S3-244316 (R2-2409534) is received regarding the security handling for inter-CU LTM in non-DC cases. The LS confirms that the preferred rekeying method among most companies is to send an RRC message after each inter-CU LTM cell switch, ensuring that the NCC is protected by RRC signalling.

S3-244316: The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure.

However, receiving an RRC message after each inter-CU LTM may go against the original design principles of LTM. LTM is designed to perform fast cell switching by lower layer signalling. Introducing RRC signalling after each LTM cell switch would slow down this process. It will even make subsequent LTM useless, as new LTM configuration can be indicated in each RRC signalling after LTM, and pre-configuration become unnecessary.

Observation 1: Receiving an RRC message after each inter-CU LTM go against the original design principles of LTM. It will slow down the cell switch process and make subsequent LTM useless.

Moreover, the definition of LTM cell switch completion is based on the successful reception of the first UL data by the network. The RRCReconfigurationComplete message is encrypted with the target key calculated from the NCC value. Therefore, if another RRC message is used to convey this NCC value, it should be "RRC during LTM cell switch" rather than "RRC after LTM cell switch." Until the UE receives the NCC value, calculates the target key, and uses it to encrypt the first UL data, the LTM cell switch is not considered complete, and the data cannot be transmitted. This will lead to much severe interruption for inter-CU LTM.

Observation 2: The LTM cell switch is not considered complete until the UE receives the NCC value, calculates the target key, and uses it to encrypt the first UL data. Introducing an RRC message to indicate the NCC value will result in significant latency and data interruptions to inter-CU LTM.

This is why SA3 also mentioned the following in the LS:

S3-244316: SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like, - Security threat of the unprotected MAC CE to deliver the NCC value. - Sending NCC value per candidate cell via RRC message during LTM preparation phase.

From the RAN2 perspective, both the MAC-CE and pre-configuration methods are viable. The MAC-CE may raise security concerns in SA3, while pre-configuring NCC values could introduce additional complexity.

Considering the impact to introduce RRC to indicate the NCC value for LTM, it is suggested that RAN2 respond to SA3 to continue to analyse these two methods or other methods without RRC signalling between LTM cell switches.

Proposal 1: RAN2 reply to SA3 that RAN2 prefers to avoid RRC signalling between subsequent LTM and would like to ask SA3 to further exploring alternative methods.

Triggering order in LTM execution

Regarding the order of triggering cell switch command and interaction between the source gNB and target gNB, In Rel-18 LTM, RAN3 clarified that the serving DU may trigger the cell switch command MAC CE first, or inform the target DU before sending the cell switch command to the UE. In the reply, RAN2 agreed that the former approach (serving DU triggering LTM command first and then informing the target DU) should be the baseline to reduce potential latency for LTM execution.

Observation 3: In Rel-18 intra-CU LTM, RAN2 confirms that serving DU triggering LTM command first and then informing the target DU is the baseline.

In an intra-CU scenario, the typical latency for the F1 interface is around 10ms for round trip time. This results in approximately 10ms from the source DU informing the target DU in the same CU. The UE requires more than 10ms to perform TLTM_RRC-processing and TLTM_processing. This time frame allows the network to prepare well before the UE attaches to the target cell.

However, in an inter-CU scenario, it is worth discussing the latency for the source DU to inform the target DU for the initiation of the LTM cell switch. Typically, the latency over F1 interface is 5ms for one shot, and the latency over Xn interface is 10ms. In the last meeting, RAN2 agreed that it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell. So the signalling is send from Source DU->Source CU->Taget CU->Target DU, totally around 20ms.

For UE, the time for Uu interface and MAC-CE decoding is smaller than 4ms. The latency of UE to decode ASN.1 TLTM_RRC-processing can be saved (reduced to zero) by UE’s capability ltm-FastProcessingConfig-r18 (FG 39-6). If UE does not support this capability, the latency is 10ms. The latency for UE to perform LTM processing (TLTM_processing) is also depends on UE’s capability ltm-FastUE-Processing-r18 (FG 39-7), which can be 10~20ms for intra frequency and 20~40ms for inter-frequency cell switch.

Therefore, If the UE supports FG 39-6 and FG 39-7, the UE's total processing time (4+0+10ms) may be shorter than the latency from source DU to target DU (5+10+5ms), potentially leading to the target cell not being well-prepared when the UE finishes the LTM processing. This could lead to more interruptions (rather than latency) during LTM.
Observation 4: If the serving DU triggering LTM command first, and UE support fast RRC processing and fast LTM processing, the target DU may not receive the indication (5+10+5ms) when UE finish the LTM processing (0+10ms), which lead to more interruption.

If we take RACH-less LTM as example:

Figure 1 Potential impact in Intra/inter CU network latency for LTM execution

In such cases, the network may need to inform the target DU earlier than sending LTM CSC MAC-CE to UE to align with the UE's processing time, as latency is anyway better than interruption. This behaviour can be dynamically adjusted according to UE’s capability and implemented by network.

Proposal 2: RAN2 assumes it is up to network implementation to adjust the order between sending LTM CSC MAC CE to UE and indicating target gNB for the cell switch, to always make sure that target cell is well-prepared when UE finishes the LTM processing.

The support of RACH-based LTM

In the RAN2 #126 meeting, both CG-based RACH-less and DG-based RACH-less procedures were agreed.

Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM

However, one point that may have been missed is the support of RACH-based LTM in inter-CU scenario. This is also raised by other companies but was not discussed in the last meeting.

From RAN2 perspective, we see no issue to support RACH-based LTM in inter-CU scenario, as it does not bring extra complexity or strict latency requirement for inter-node coordination. This includes both CBRA and CFRA cases.

Therefore, we propose:

Proposal 3:	RACH-based inter-CU LTM is supported.

Conclusion

In this contribution, the following observations and proposals are made:

LS from SA3

Observation 1: Receiving an RRC message after each inter-CU LTM go against the original design principles of LTM. It will slow down the cell switch process and make subsequent LTM useless.

Observation 2: The LTM cell switch is not considered complete until the UE receives the NCC value, calculates the target key, and uses it to encrypt the first UL data. Introducing an RRC message to indicate the NCC value will result in significant latency and data interruptions to inter-CU LTM.

Proposal 1: RAN2 reply to SA3 that RAN2 prefers to avoid RRC signalling between subsequent LTM and would like to ask SA3 to further exploring alternative methods.

Triggering order in LTM execution

Observation 3: In Rel-18 intra-CU LTM, RAN2 confirms that serving DU triggering LTM command first and then informing the target DU is the baseline.

Observation 4: If the serving DU triggering LTM command first, and UE support fast RRC processing and fast LTM processing, the target cell may not receive the indication (5+10+5ms) when UE finish the LTM processing (0+10ms), which lead to more interruption.

Proposal 2: RAN2 assumes it is up to network implementation to adjust the order between sending LTM CSC MAC CE to UE and indicating target gNB for the cell switch, to always make sure that target cell is well-prepared when UE finishes the LTM processing.

The support of RACH-based LTM

Proposal 3:	RACH-based inter-CU LTM is supported.

References

- R2-2409534      Reply LS on security handling for inter-CU LTM in non-DC cases (S3-244316; contact: Samsung)

- R2_127b_R18 SL_MOB_R19 NES_MOB_Session Notes_EoM

- RP-240299-Revised-WID-R19-Mobility-Enhancements