3GPP TSG-RAN WG2 Meeting #128	R2-2410229

Orlando, USA, Nov. 18th – 22th, 2024

Agenda Item:	8.7.4.2

Source:	ITRI

Title:	Discussion on enhanced DSR for XR

Document for:	Discussion and Decision

1 Introduction

In RAN2 #127bis, some agreements of enhanced DSR had been reached as below.

- We do not change the definition of delay-critical data

- For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s)

- Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc.

There are several issues of enhanced DSR still needed to be clarified, such as need of thresholds configuration constraints (including analysis of impact on DSR triggering/cancellation etc.), inclusion of non-delay critical data, and MAC CE design, interworking with legacy DSR etc.

In this document, we proposed that the configuration of DSR triggering and reporting thresholds should be flexible and network implementation. Then we checked the impact on DSR triggering and cancellation procedures. For the inclusion of non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, we thought that there is no need to discuss it if a flexible configuration of DSR triggering and reporting thresholds can be approved.

2 Discussion

2.1 Configuration of DSR triggering and reporting thresholds

From our point of view, the configuration of DSR triggering and reporting thresholds should be flexible and network implementation. Firstly, DSR is for urgent data, so the DSR should not be triggered very frequently in a normal condition. Secondly, the multiple reporting thresholds in the DSR agreed in Rel-19 are used to provide more buffer information with delay status, so the gNB can proceed the UL resource allocation for multiple UEs in the future more efficiently. Therefore, the single triggering threshold can assumed to be a smaller value compared to the multiple reporting thresholds of the LCG. For example, the multiple reporting thresholds may be 20ms, 40ms, 60ms, and so on, and the single triggering threshold can be the smallest value of threshold for DSR reporting of an LCG, e.g, 40ms.

Based on the above discussion, we clarify that only the single triggering threshold is related to the delay-critical data definition and DSR triggering procedure, and the multiple reporting thresholds are related to detailed buffer information with delay status in DSR. Therefore, the configuration of DSR triggering and reporting thresholds should be flexible and network implementation.That means both delay critical data (i.e., the data that has already been delay-critical) and non-delay critical data (i.e., the data that is going to be delay-critical) are included in DSR.

Proposal 1: the configuration of DSR triggering and reporting thresholds should be flexible and network implementation.

In the following, we check whether Proposal 1 causes the impact on specifications or not, i.e., DSR triggering and cancellation.

DSR triggering:

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG:

1>	if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and

1>	if there is no DSR pending for the logical channel:

2>	trigger a DSR for the logical channel.

Since the above DSR triggering procedure depends on the single triggering threshold (i.e., remainingTimeThreshold of the LCG), we do not see any modifications needed for DRS triggering under the proposition of proposal 1. That means triggering of DSR depends on the single triggering threshold, and there is no impact on specifications of DSR triggering.

DSR cancellation:

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

Since both delay critical data (i.e., the data that has already been delay-critical) and non-delay critical data (i.e., the data that is going to be delay-critical) are included in DSR under the proposition of proposal 1, for the DSR cancellation, we have to modify the “all the PDCP SDUs” to “all the delay-critical PDCP SDUs”.

Proposal 2: if the flexible configuration of DSR triggering and reporting thresholds in Rel-19 is approved, the DSR cancellation is suggested to be modified as bellow,

The MAC entity shall cancel a pending DSR, when all the delay-critical PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the delay-critical PDCP SDUs associated with the DSR.

2.2 Discussion on inclusion of non-delay critical data ahead of delay critical data

Some companies proposed to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. From our point of view, although the logical channel buffer at MAC layer is FIFO (first in, first out) as default, it is reasonable that the packets can be assumed to be sorted properly by UE implementation for XR application. Also, if the configuration of single  triggering threshold and multiple reporting thresholds is approved to be flexible and both delay critical data and non-delay critical data are included in DSR, we think that there is no need to discuss whether to include non-delay critical data ahead of delay critical data in the buffer size calculation or not.

Proposal 3: No need to discuss the inclusion of non-delay critical data ahead of delay critical data in the buffer size calculation for DSR.

3 Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: the configuration of DSR triggering and reporting thresholds should be flexible and network implementation.

Proposal 2: if the flexible configuration of DSR triggering and reporting thresholds in Rel-19 is approved, the DSR cancellation is suggested to be modified as bellow,

The MAC entity shall cancel a pending DSR, when all the delay-critical PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the delay-critical PDCP SDUs associated with the DSR.

Proposal 3: No need to discuss the inclusion of non-delay critical data ahead of delay critical data in the buffer size calculation for DSR.

4 References

- R2_127bis R18 MBS, R18 QoE and R19 XR session notes (Dawid)_EOM.docx