3GPP TSG-RAN WG2 Meeting #128	R2-2410240

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.7.6

Source:	OPPO

Title:	Discussion on XR rate control

Document for:	Discussion, Decision

Introduction

The revised WID of Rel-19 XR approves a new objective as follows [1]:

- Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

This contribution shares our opinions on this matter.

Discussion

The newly approved Rel-19 XR objective aims to dynamically adjust the UL XR application codec bit rate and encoding parameters to provide a seamless XR user experience during times of network congestion. This adaptation of the data rate is initiated by a DL MAC CE from the network.

In Rel-18, RAN2 confirms that the NG-RAN can estimate congestion for UL[2].

- It is feasible for RAN to estimate the congestion information per-QoS flow and per-DRB in downlink and uplink directions. - It is feasible for RAN to estimate the congestion information per-QoS flow and per-DRB in uplink without UE impacts.

When congestion occurs, the NG-RAN is able to inform the UE about the congestion situation for each DRB or QoS flow. The ongoing debate is whether the congestion control indication should be provided at the QoS flow level or the DRB level and RAN2 leaves an FFS on this topic in RAN2#127bis.

Agreements on XR rate control - FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc.

From our perspective, a per-DRB indication is simple, adequate and effective for the following reasons:

- Congestion mitigation

In the current spec, data transmission through the Uu interface is typically managed on a per LCH or DRB basis. This means that when there is data buffered for a specific DRB, the UE will always utilize the allocated UL grant to transmit that data. Importantly, the UE does not distinguish which data corresponds to which QoS flow linked to this DRB. In the event of congestion, even though more detailed congestion information is available via QoS level indication – this could help the application layer perform more precise frame rate adaptation for specific QoS flows – reducing the data rate for the entire DRB is a more effective way to promptly control and alleviate the congestion.

- Aware of which QoS flow supports rate adaption

Adjusting the XR application rate can help alleviate RAN congestion, thereby enhancing the overall QoS and user experience. However, the extent to which the codec bit rate of a specific QoS flow can be adjusted as well as which QoS flow can be adapted should be determined by the UE implementation. This is because the gNB currently lacks information about the type of traffic associated with each specific QoS flow. As a result, it would be challenging for the gNB to provide accurate data rate.

- QoS enforcement

In legacy, guaranteeing QoS requirements depends on the implementation of the gNB. This involves determining the mapping relationship between QoS flows and DRBs, as well as allocating resources for these DRBs and LCGs. It's important to note this method goes beyond simply controlling the source codec rate during congestion. Since the gNB knows which QoS flows and DRBs are experiencing congestion, it can remap a congested QoS flow to an alternative DRB. Resultantly, the congestion associated with that QoS flow can be alleviated.

- Interworking with L4S

Some companies argue that the QoS-level indication complements L4S as L4S operates on a per-QoS flow basis. However, we believe that per-DRB congestion indication can also work with L4S effectively. In one implementation, if the UE knows which QoS flow experiencing congestion by using L4S marking and receives a DRB-level congestion indication, the UE can derive which QoS flow requires the rate control as the UE knows how each QoS flow maps to a specific DRB. As a result, the UE can appropriately adjust the codec bit rate and encoding parameters for the identified QoS flow.

Therefore, per-DRB MAC CE signalling from the network is preferred to enable uplink rate adaption for congestion.

Per-DRB MAC CE signalling from the gNB is used to enable uplink rate adaption.

If Proposal 1 is accepted, we prefer to use the existing Recommended Bit Rate MAC CE format for uplink rate adaption because

- The recommended bit rate reflects the congestion situation. The worse the congestion situation/status, the lower the recommended bit rate.

- Usually, one RLC/logical channel is associated with one DRB. The UE can easily identify which DRB is experiencing congestion by the indication of LCID of that logical channel.

- Avoid the effort of discussing/designing the format for the new MAC CE. However, it is still necessary to introduce the usage description and the new LCID for this new MAC CE in the MAC/Stage-2 spec accordingly.

RAN2 aims at using the format of the existing Recommended Bit Rate MAC CE for uplink rate adaption.

In RAN2#127bis, RAN2 outlines several candidates to improve the indication of bit rate values.

Agreements on XR rate control - RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - Extend the Bit Rate field - Define a new bit rate table to provide sufficient granularity for XR traffic - Introduce new values for the bitRateMultiplier

Introducing a new bit rate table involves a significant workload for RAN2 in terms of designing and discussing the details, such as the number of code points and the specific value assigned to each code point. In contrast, the other two approaches seem simpler from the workload perspective. However, the choice of the most suitable approach depends on SA4's response regarding the appropriate range and granularity of uplink bit rates to be supported. Therefore, we recommend that RAN2 waits for SA4’s input before making any decisions.

RAN2 waits for SA4's feedback on the appropriate range and granularity of uplink bit rates before proceeding with how bit rate values are indicated for XR.

Conclusion

We have the following proposals:

Proposal 1	Per-DRB MAC CE signalling from the gNB is used to enable uplink rate adaption.

Proposal 2	RAN2 aims at using the format of the existing Recommended Bit Rate MAC CE for uplink rate adaption.

Proposal 3	RAN2 waits for SA4's feedback on the appropriate range and granularity of uplink bit rates before proceeding with how bit rate values are indicated for XR.

Reference

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur)

[2] R2-2213226 Reply LS to SA2 on XR, vivo, Huawei