3GPP TSG-RAN WG2 Meeting #128	R2-2410287

Orlando, USA, Nov. 18th – 22nd , 2024

Agenda item:		8.7.5

Source:		Lenovo

Title:		AM RLC enhancement

Document for:		Discussion and Decision

- Introduction

In this contribution we discuss RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

- Discussion

The Rel-19 Work Item on XR Phase 3 was agreed in RP-240791.

The Rel-19 XR Phase 3 objectives are as follows: Specify Enhancements for Scheduling, as follows: - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

In RAN2#127, the following discussion on RLC related enhancements for Rel-19 took place with the following agreements:

- Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances. FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first. - In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer. - In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where: Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach. Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach.

In RAN2#127bis, the following agreements have been achieved:

RLC timely retransmissions - RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay Avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

2.1 Avoiding Unnecessary Retransmissions

In the last meeting, RAN2 has agreed a combined Rx and Tx approach for avoiding unnecessary RLC retransmissions.

- It was also agreed that the RLC receiver can maintain an additional timer that dictates how long the RLC AM receiver needs to try to receive/ recover a packet. The timer can be started when the RLC receiver delivers a gap (i.e., a RLC SDU corresponding to a PDCP SDU out of order) to PDCP. The intention is to maintain the synchronization between the start of the new RLC timer and the PDCP reordering timer. Upon expiry of the timer, the RLC receiver transmits a status report to its peer entity containing an ACK for those SDUs that are abandoned. The RLC receiver then updates its reception window, state variables and timers as per legacy procedure. The RLC transmitter, upon reception of PDCP discard indication, stops transmissions of an outdated SDU. Upon reception of the status report, the RLC transmitter, discards the SDUs indicated as ACKed and updates its transmission window as per legacy procedure.

Proposal 1. The new local timer in the RLC receiver (when configured) is started when the RLC receiving entity delivers a RLC PDU corresponding to a PDCP SDU out of order to PDCP.

The assumed “combined” approach poses a risk of abandonment of RLC SDUs corresponding to PDCP Control PDUs based on the legacy status report. The RLC receiver has no information of the RLC PDUs associated with a PDCP Control PDU, so the receiver window will be updated regardless of such PDUs according upon the expiration of the new timer and notify the RLC transmitter with a status report. The RLC transmitter then considers the RLC SDU associated with the PDCP control PDU as successfully received based on the report, which causes the RLC transmitter to discard the RLC SDU associated with the PDCP control PDU. Furthermore, if RLC receiver starts the new timer when the RLC receiver delivers a RLC SDU corresponding to a PDCP control PDU, while the PDCP receiver does not start the reordering timer, it may cause the RLC receiver potentially unnecessarily moves the receiver window earlier than the PDCP receiver window.

Observation 1. The assumed “combined” approach poses a risk of abandonment of RLC SDUs corresponding to PDCP Control PDUs since the receiver window will be updated regardless of such PDUs upon the expiration of the new timer.

Observation 2. If RLC receiver starts the new timer when the RLC receiver delivers a RLC SDU corresponding to a PDCP control PDU, while the PDCP receiver does not start the reordering timer, it may cause the RLC receiver potentially unnecessarily moves the receiver window earlier than the PDCP receiver window.

Hence, we think that there needs to be a mechanism for the RLC Tx to provide RLC Rx with some information. One straight way is to avoid the discard of RLC SDUs associated with PDCP Control PDUs, the RLC receiver notifies the abandoned SN gap report to the RLC transmitter (the abandoned SN gap report can be different from the legacy status report, i.e., indicating those packets that are not successfully received but for which the RX no longer expects further reception (abandoned), e.g., due to expiry of the local timer and corresponding update of reception window). The RLC transmitter can identify whether the RLC SDU is abandoned due to the expiry of the local timer at RLC RX side or successfully received by the RLC receiver. If a RLC SDU is abandoned by the RLC RX side, the RLC transmitter should still continue (re)transmitting an associated PDCP control PDU for which no discard timer expiry indication was received by PDCP Tx side, e.g., using a new RLC SDU. Alternatively, if the RLC transmitter determines that some PDUs are not yet outdated although it was abandoned by the receiver, it is beneficial to notify the RLC receiver of the RLC SNs that are not yet outdated (e.g., associated with a PDCP Control PDU). This can help the RLC receiver to not discard the RLC SDU even it is received after the receiver window is updated, e.g. RLC receiver delivers a RLC SDU to PDCP layer even though the corresponding RLC PDU was received outside the RLC receiving window.

Proposal 2. To avoid discard of RLC SDUs associated with PDCP control PDUs, RAN2 to discuss the following:

- the RLC receiver notifies the abandoned SNs to the RLC transmitter in order to identify whether the RLC SDU associated wtith a PDCP control PDU is abanoned or not. The RLC transmitter may then transmit the PDCP control PDU e.g., using a new RLC SDU.

- the RLC transmitter notifies the SNs of the RLC PDUs not yet outdated among the abandoned PDUs. The RLC receiver then does not discard these PDUs even if received outside the receiving window.

2.2 Timely Retransmissions

RLC-AM feedback based on current polling parameters are not well adapted for short packet delay budgets applicable to XR traffic. The data volume of PDU set of XR traffic is variable, i.e., the number of PDUs or the size of the PDUs of a PDU set is variable. One possibility, the polling parameter and t-PollRetransmit is set to small value for triggering more frequent polling to the RLC receiver, and gNB frequently transmits status report, which can cause higher overhead.

In legacy, a polling can be triggered if both the transmission buffer and the retransmission buffer become empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU, or if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling). However, if there are multiple PDU sets in transmission or retransmission buffer, the corresponding polling may not be triggered after the transmission of a PDU set.

Hence, the RLC retransmission based on current RLC-AM feedback mechanism may not be timely triggered, which causes the packets to arrive too late at the receiver, thus impacting the user experience.

Observation 3. It is difficlut to set the polling related parameters and status prohibit timer for a timely status report, to adapt for XR traffic with short packet delay budgets and variable data volume of PDU set, without significantly increasing the overhead.

Observation 4. If there are multiple PDU sets in transmission or retransmission buffer, the polling may not be triggered after the transmission of a PDU set.

To avoid the late RLC retransmission, RLC transmitter may trigger early RLC retransmission for data based on the delay status, provided the transmitting entity did not receive a status report, assuming polling has been transmitted to the peer RLC entity. For example, the PDCP entity delivers a delay-critical indication to the corresponding AM RLC entity, and the delay critical data may be determined based on a separately configured threshold, different than R18 value.

Additionally, another condition that can be considered to enable autonomous retransmissions is the PSI value. For example, only those packets with PSI value greater than a preconfigured threshold are eligible for autonomous retransmissions. Furthermore, the cancellation of an autonomous retransmission can be considered when such a retransmission was triggered and an ACK (e.g., HARQ ACK or a RLC status-report containing an ACK for the corresponding PDU) is received before it is submitted to lower layers. Lastly, RAN2 can also look into scheduling enhancements (e.g., LCP enhancements) for packets that are autonomously retransmitted to avoid any delays in transmission of such data before the discardTimer expires.

In addition, the enhancement of polling mechanism may be considered for the early RLC retransmission. For example, the transmitter RLC entity can trigger a new polling based on the end of PDU set, and if the remaining discard time of the data in the PDU set is below a threshold. Another condition could be that there is more than one PDU set in transmission buffer after transmission of a PDU set, in which case, the AM RLC transmitter entity may poll its peer AM RLC receiver entity in order to trigger STATUS reporting at the peer AM RLC entity. And if gNB receives the polling and a t-StatusProhibit is running, it may ignore the running timer and send a status report to the peer AM RLC receiver entity at least when the status report is triggered by this new polling condition, and if there is a NACK status report to transmit.

Proposal 3. To achieve timely retransmissions on RLC layer for XR traffic, RAN2 to consider the following:

- RLC transmitter triggers a new polling based on PDU set for data with remaining delay below a threhold.

- RLC transmitter triggers autonomous RLC retransmission for data with remaining delay below a threhold.

- Additional conditions such as PSI-based threshold for triggering autonomous RLC retransmissions.

Autonomous retransmissions can congest the link if configured poorly, due to excessive transmissions of redundant RLC PDUs. Hence, autonomous retransmission can be network controlled to provide additional control when this function is used, e.g., via RRC configuration for each RLC entity. We think both enhancements can be used together without it conflicting the other, each may use a different threshold. The new polling trigger may avoid/reduce the number of early RLC retransmission.

Proposal 4. The enhanced option(s) to achieve timely retransmissions on RLC layer is configured per RLC entiy by RRC.

- Conclusion

In this contribution, we discuss the RLC AM enhancement for XR traffic with small delay budget. We have the following observation and proposals:

Observation 1. The assumed “combined” approach poses a risk of abandonment of RLC SDUs corresponding to PDCP Control PDUs since the receiver window will be updated regardless of such PDUs upon the expiration of the new timer.

Observation 2. If RLC receiver starts the new timer when the RLC receiver delivers a RLC SDU corresponding to a PDCP control PDU, while the PDCP receiver does not start the reordering timer, it may cause the RLC receiver potentially unnecessarily moves the receiver window earlier than the PDCP receiver window.

Observation 3. It is difficlut to set the polling related parameters and status prohibit timer for a timely status report, to adapt for XR traffic with short packet delay budgets and variable data volume of PDU set, without significantly increasing the overhead.

Observation 4. If there are multiple PDU sets in transmission or retransmission buffer, the polling may not be triggered after the transmission of a PDU set.

Proposal 1. The new local timer in the RLC receiver (when configured) is started when the RLC receiving entity delivers a RLC PDU corresponding to a PDCP SDU out of order to PDCP.

Proposal 2. To avoid discard of RLC SDUs associated with PDCP control PDUs, RAN2 to discuss the following:

- the RLC receiver notifies the abandoned SNs to the RLC transmitter in order to identify whether the RLC SDU associated wtith a PDCP control PDU is abanoned or not. The RLC transmitter may then transmit the PDCP control PDU e.g., using a new RLC SDU.

- the RLC transmitter notifies the SNs of the RLC PDUs not yet outdated among the abandoned PDUs. The RLC receiver then does not discard these PDUs even if received outside the receiving window.

Proposal 3. To achieve timely retransmissions on RLC layer for XR traffic, RAN2 to consider the following:

- RLC transmitter triggers a new polling based on PDU set for data with remaining delay below a threhold.

- RLC transmitter triggers autonomous RLC retransmission for data with remaining delay below a threhold.

- Additional conditions such as PSI-based threshold for triggering autonomous RLC retransmissions.

Proposal 4. The enhanced option(s) to achieve timely retransmissions on RLC layer is configured per RLC entiy by RRC.

- References

TS 38.322 v 18.0.0

R2-2408308, AM RLC enhancement, Lenovo