3GPP TSG-RAN WG2 #128	R2-2410311

Orlando, US, 18th – 22nd, November 2024

Source:	NTT DOCOMO, INC.

Title:	Remaining issues on study of AIoT functionalities

Document for:	Discussion and decision

Agenda Item:	8.2.2

- Introduction

The SI: FS_ Ambient_IoT_Solutions_RAN is planned to be closed in this meeting [1]. This contribution provides our views on remaining issues to conclude the study on functionality aspects of Ambient IoT.

- Discussion

  - AS ID

In the last meeting, an FFS is left on whether the “AS ID” can be based on a part of upper layer device ID (i.e., permanent device ID).

RAN2#127-bis [2] RAN2 assumes that if “AS ID” is defined it is used at least for purpose of D2R scheduling and R2D reception. Up to RAN1 to decide whether a “AS ID” is defined. RAN2 assumes this “AS ID” should be a short AS layer ID, rather than the full upper layer device ID. FFS on the length. FFS if AS ID can be based on partial upper layer device ID.

We have a strong concern on allowing to use a permanent device ID for AS ID. There are two reasons:

- Security concern. AS ID is used for purpose of D2R scheduling and R2D reception; this means that the AS ID is attached with D2R and R2D messages in A-IoT air interface. Considering that we do not assume AS security in A-IoT protocol stack, AS ID is not security protected either, thus any receiver can intercept AS ID without decryption. If partial upper layer device ID is used as an AS ID, the problem is that even a part of the upper layer device ID does not change permanently. This is an issue that allows any malicious third party, or any competitor against our customers who purchased A-IoT system, can physically trace Ambient IoT device.

- Permanent device ID is unique, but a part of it is not unique. We do not see a reason to utilize permanent numbers that can collide each other for AS IDs. Especially when the two proximate A-IoT devices have the same permanent AS ID, the collision cannot be resolved, i.e., unless the devices distance themselves from each other enough, all R2D messages to either of them is received by both devices while the reader cannot make sure which of them sent any D2R messages which is sent by either of them.

Therefore, “AS ID” should be a temporary ID generated in AS layer.

Proposal 1.	“AS ID” is a temporary ID which is generated in AS layer.

There are two options which node generates an AS ID.

RAN2#127-bis [2] From RAN2 perspective, following options are possible for “AS ID”: - Option 1: a random ID if used in Msg1 can be reused; - Option 2: reader assigns this “AS ID”. FFS via which R2D message.

For Option 1, we doubt whether the random ID is long enough not to often collide. We already agreed that the random ID is 16 bits. The length is agreed intending to let the random ID be unique within devices which collided on one access occasion for Msg1 in A-IoT random access. However, the AS ID should be unique within all devices that is in distance to communicate with an A-IoT reader. Since we are studying indoor use cases, it would be possible that 100 to 1,000 of A-IoT devices are located in coverage of an A-IoT reader, where the random ID (16 bit => 65,536) is not long enough for AS ID.

On the other hand, Option 2 allows the reader to assign AS ID without collision because the reader can make sure that the AS ID is unique within all devices that is in distance to communicate with an A-IoT reader. The reader should assign AS ID to the device via the first R2D message dedicated to the device as depicted in Figure 1, i.e., Msg2 in 3-step CBRA, success indication in 2-step CBRA and CFRA.

Figure 1. First R2D messages dedicated to a device.

Proposal 2.	“AS ID” is generated by A-IoT reader and allocated to each A-IoT device via Msg2 in 3-step CBRA, success indication in 2-step CBRA and CFRA.

  - Energy status report

RAN2 has agreed to capture the energy status reporting in TR. Energy status reporting is indicated when the device is aware of its low-battery status, and it cannot continue the following procedure after the next D2R message.

RAN2#127-bis [2] Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1,MSG3 and Command Response message), if the device can. 1 bit indication can be captured in the TR. We will capture the use case in the TR. FFS whether it is reader controlled.

Energy status reporting feature helps the reader to be aware of a termination of the procedure without detecting D2R message failure. On the other hand, implementing this feature impels some requirements to the device, e.g.,

- The device can monitor the level of its own battery level.

- The device can recognize how much power is consumed to transmit a D2R message.

- The device can determine whether to trigger energy status report based on its knowledge on above two.

However, some A-IoT devices (especially device type 1) are expected to be simple designs, thus incorporating the energy status report feature in such devices may impose implementation restrictions and increase manufacturing costs. Therefore, the energy status report should be an optional feature. It should be an FFS in normative phase whether to introduce device capability signalling to report supporting energy status reporting.

Proposal 3.	A-IoT device optionally supports energy status reporting. FFS whether device capability is needed.

As mentioned above, not all devices should support energy status reporting. Moreover, the device may fall into low battery without triggering energy status report (e.g. due to mis-expectation of its own battery level because of deterioration of battery components over time).

Therefore, a procedure that works without energy status reporting (e.g., declaring failure if a D2R message is not received) has to be supported by the network anyway. In other words, from network perspective, terminating ongoing procedure upon detecting energy status reporting is an optimized feature for such devices capable of the reporting.

From cost point of view, a network implementation or OAM not supporting the alternated procedure based on energy status reporting should be allowed, and then it would be efficient to prevent the device from monitoring its battery to transmit energy status report to the reader, since the report does not help such network anymore. That is why we think it is beneficial to introduce an R2D indication to prevent energy status reporting.

Proposal 4.	A-IoT reader can indicate for A-IoT devices not to trigger energy status reporting via A-IoT paging.

- Summary and proposal

Proposal 1.	“AS ID” is a temporary ID which is generated in AS layer.

Proposal 2.	“AS ID” is generated by A-IoT reader and allocated to each A-IoT device via Msg2 in 3-step CBRA, success indication in 2-step CBRA and CFRA.

Proposal 3.	A-IoT device optionally supports energy status reporting. FFS whether device capability is needed.

Proposal 4.	A-IoT reader can indicate for A-IoT devices not to trigger energy status reporting via A-IoT paging.

- References

[1]	RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR,” RAN#103.

[2]	Chair’s notes, RAN2#127-bis.