3GPP TSG-RAN WG2 #128	R2-2410312

Orlando, US, 18th – 22nd, November 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on RA type indication in AIoT paging

Document for:	Discussion and decision

Agenda Item:	8.2.3

- Introduction

The SI: FS_ Ambient_IoT_Solutions_RAN is planned to be closed in this meeting [1]. This contribution provides our views on a remaining issue; RA type indication in AIoT paging.

- Discussion

In the last meeting, we concluded that the A-IoT paging message provides some information from which the device determines the random access type. It is still FFS whether the information is explicit or implicit, and whether to unify 2-step CBRA and 3-step CBRA.

RAN2#127-bis [2] For all use cases, device determines the random access type from the paging message. FFS whether it is explicitly or implicitly. FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

Unify 2-step and 3-step or not

As already agreed, the contents of Msg1 are different between 2-step CBRA and 3-step CBRA: In 3-step CBRA Msg1 only includes random ID, while in 2-step CBRA upper layer data is included in addition to random ID. Therefore, 2-step CBRA and 3-step CBRA obviously require different sizes of resource for Msg1 transmission, with Msg1 for 2-step requiring a larger resource size than 3-step CBRA.

Then if the reader indicates CBRA without distinguishing 2-step and 3-step, the device down-selects RA type to apply. However, whichever RA type the device chooses, the reader must always allocate resources large enough to accommodate Msg1 for 2-step CBRA for each access occasion. Therefore, even if the device consequently chooses the 3-step CBRA, the reader must always indicate a large access occasion enough for the 2-step CBRA, which is not resource efficient.

Observation 1.	If the reader indicates CBRA without distinguishing 2-step and 3-step, the reader has to allocate resources large enough to accommodate Msg1 for 2-step CBRA even if the device chooses 3-step CBRA.

Explicit or implicit indication

Some companies proposed to implicitly indicate RA type. Now we consider the way how to indicate each of RA type in A-IoT message.

- 3-step CBRA: A-IoT paging message indicates resources for multiple transmission occasions for Msg1. Resources with smaller size is allocated for each access occasion than 2-step CBRA.

- 2-step CBRA: A-IoT paging message indicates resources for multiple transmission occasions for Msg1. Resources with larger size is allocated for each access occasion than 3-step CBRA.

- CFRA: A-IoT paging message indicates resources for one transmission occasion for Msg1.

Although CFRA can be easily implicitly indicated in paging message, it looks difficult for devices to distinguish between 3-step CBRA and 2-step CBRA without explicit indication.

Observation 2.	It is difficult for devices to distinguish between 3-step CBRA and 2-step CBRA if RA type is implicitly indicated in A-IoT paging message.

Proposal 1.	Ambient IoT reader explicitly indicates random access type (3-step CBRA, 2-step CBRA or CFRA) in A-IoT paging message.

- Summary and proposal

Observation 1.	If the reader indicates CBRA without distinguishing 2-step and 3-step, the reader has to allocate resources large enough to accommodate Msg1 for 2-step CBRA even if the device chooses 3-step CBRA.

Observation 2.	It is difficult for devices to distinguish between 3-step CBRA and 2-step CBRA if RA type is implicitly indicated in A-IoT paging message.

Proposal 1.	Ambient IoT reader explicitly indicates random access type (3-step CBRA, 2-step CBRA or CFRA) in A-IoT paging message.

- References

[1]	RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR,” RAN#103.

[2]	Chair’s notes, RAN2#127-bis.