3GPP TSG-RAN WG2 Meeting #128	R2-2410320

# Orlando, USA, Nov. 18th – 22nd, 2024

# Agenda item:	8.5.2

# Source:	CMCC

# Title:	Discussion on on-demand SSB for SCell

# Document for:	Discussion, Decision

- Introduction

In RAN2#127bis meeting, RAN2 reached the following agreements on on-demand SSB for SCell [1]:

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this paper, we further discuss open issues related to OD-SSB transmission configuration/indication and measurement considering OD-SSB.

- Discussion

  - Configuration and Indication of OD-SSB transmission

In last meeting, RAN2 reached the agreement that RRC signalling can be used to indicate OD-SSB activation/deactivation, and this is not limited to initial state. In addition to the activation/deactivation state, there are necessary parameters to be configured in RRC signalling. Following are the RAN1’s agreements on OD-SSB configuration：

RAN1#117 meeting Agreement[2] - For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused

RAN1#118bis meeting Agreement[3] For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1. - Sub-carrier spacing of the on-demand SSB - FFS if this can be absent - Physical Cell ID of the on-demand SSB - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index - Downlink transmit power of on-demand SSB - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters Agreement The previous RAN1 agreement is partly confirmed and further revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Based on RAN1 progress, the following parameters can be configured in RRC signalling:

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - Sub-carrier spacing of the on-demand SSB

  - Physical Cell ID of the on-demand SSB

  - Downlink transmit power of on-demand SSB

  - SSB time domain positions of on-demand SSB burst

Proposal 1: The following parameters can be configured in RRC signalling:

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - Sub-carrier spacing of the on-demand SSB

  - Physical Cell ID of the on-demand SSB

  - Downlink transmit power of on-demand SSB

  - SSB time domain positions of on-demand SSB burst

As to the signalling to configure OD-SSB, one straight forward way is to include OD-SSB configurations as a part of SCell configuration. However, considering that there could be common configuration parameters among SCells and there could be multi SCells configured to a UE, and there’s discussion on supporting multiple OD-SSB configuration in RAN1, OD-SSB configuration can be independent to SCell configuration.

Proposal 2:  OD-SSB configuration can be independent to SCell configuration, to reduce the avoid huge signalling overhead.

Though RAN2 only agreed to use a new MAC CE for OD-SSB transmission indication, RAN1 discussed to indicate more parameters via RRC or MAC CE:

RAN1#118 meeting Agreement [4] For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

RAN1#118bis meeting Agreement For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options

Observation 1: based on RAN1’s agreements, periodicity of the on-demand SSB can be indicated in OD-SSB transmission indication MAC CE, and whether the number N of on-demand SSB bursts and  the duration of on-demand SSB transmission window can be indicated in MAC CE is under discussion.

From RAN2 perspective, including more parameters in OD-SSB transmission indication MAC CE may enlarge the size of the MAC CE, we prefer to configure these parameters in RRC configuration. And for compromise, if multiple-configuration is supported, the configuration index can be indicated in OD-SSB transmission indication MAC CE. RAN2 should discuss whether to postpone the MAC CE design until RAN1 has more progress to avoid duplicate discussion.

Proposal 3: RAN2 should discuss whether to postpone the MAC CE design until RAN1 has more progress to avoid duplicate discussion.

Proposal 4: If multiple-configuration of OD-SSB transmission is supported, the index of applicated configuration can be indicated in OD-SSB transmission indication MAC CE.

  - Measurement considering OD-SSB

In last RAN2 meeting, two options related to OD-SSB measurement were listed for discussion:

-	Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB

-	Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

During the discussion, one issue mentioned by companies was whether OD-SSB and always-on SSB are on the same frequency. Similar issue was discussed in RAN1. Following is the cases supported by RAN1, in our view that UE’s measurement behaviour need to be discussed case by case.

RAN1#118bis meeting Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

Observation 2: Based on RAN1’s agreements, on-demand SSB and always-on SSB may have time or frequency domain overlapping or not.

Based on RAN1 agreements, the measurement considering OD-SSB and always-on SSB can be clarified as the following sub-cases and depicted in Figure 1.

Figure 1 Different cases of OD-SSB multiplex with always-on SSB

For Case A, always-on SSB and OD-SSB are on the different frequency, no matter there’s time domain overlapping between them or not, Option-1 to only measure OD-SSB if transmitted is a simpler way for UE.

For Case B, since always-on SSB and OD-SSB are on the same frequency and there’s time domain overlapping, UE can measure based on OD-SSB, which can be seen as an implementation of Option 1.

For Case C, though always-on SSB and OD-SSB are on the same frequency, there’s no time domain overlapping. In this case, Option 1 can be used as a baseline to achieve a unified solution for different cases. However, if always-on SSB is also measured, it may reduce the measurement latency, and maybe the UE can provides more information to the network, Besides, the option selection in this case may be affected by the discussion of L3 measurement framework.

Proposal 5: For the cases that always-on SSB and OD-SSB are on the different frequency and always-on SSB and OD-SSB are on the same frequency and there’s time domain overlapping, UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB.

Proposal 6: For the case that always-on SSB and OD-SSB are on the same frequency and there’s no time domain overlapping, at least OD-SSB is measured, while whether to measure always-on SSB may depend on the discussion of L3 measurement framework.

As to the L3 measurement framework, there’s also two options listed in last RAN2 meeting for discussion:

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

In current mechanism, the periodicity and duration of SMTC are configured, the other words, there’s fixed periodic start point and duration window for UE to perform measurement. But when it comes to OD-SSB,  as we agreed that the transmission of OD-SSB can be indicated by RRC signalling or MAC CE, though the timing relationship of MAC CE and OD-SSB transmission is defined in RAN1, but the time to send MAC CE is random, it cannot assure there’s SSB in the adjacent SMTC window, even if it’s another SMTC configuration for OD-SSB, which may result in measurement delay. Besides, since whether to support multiple configuration of OD-SSB is still FFS, if supported, Option 1 may increase the signalling overhead for different OD-SSB configurations. What’s more, since OD-SSB is mainly for RRC_CONNECTED UE’s serving cell measurement, it is not related to neighbour cell measurement, current L1 measurement of serving cell based on SSB can be considered. Therefore, it’s preferred to use Option 2 for OD-SSB measurement.

Observation 3: Option 1 with SMTC mechanism may result in measurement delay and huge signalling overhead, especially when multiple OD-SSB configuration is supported.

Observation 4: OD-SSB is mainly used for serving cell measurement.

Proposal 7: The UE measurements OD-SSB based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted.

If Option 2 is adopted for OD-SSB, we need to discuss whether a separate reporting of OD-SSB measurement results is needed, considering that OD-SSB and always on SSB may have frequency/ time domain even power domain difference.

Proposal 8: RAN2 should discuss whether a separate reporting is needed for measurement based on OD-SSB.

- Conclusion

In this contribution, we discuss configuration/indication of OD-SSB transmission and measurement related issues. Following are the proposals and observations made in this contribution:

Observations:

Observation 1: based on RAN1’s agreements, periodicity of the on-demand SSB can be indicated in OD-SSB transmission indication MAC CE, and whether the number N of on-demand SSB bursts and  the duration of on-demand SSB transmission window can be indicated in MAC CE is under discussion.

Observation 2: Based on RAN1’s agreements, on-demand SSB and always-on SSB may have time or frequency domain overlapping or not.

Observation 3: Option 1 with SMTC mechanism may result in measurement delay and huge signalling overhead, especially when multiple OD-SSB configuration is supported.

Observation 4: OD-SSB is mainly used for serving cell measurement.

Proposals:

Proposal 1: The following parameters can be configured in RRC signalling:

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - Sub-carrier spacing of the on-demand SSB

  - Physical Cell ID of the on-demand SSB

  - Downlink transmit power of on-demand SSB

  - SSB time domain positions of on-demand SSB burst

Proposal 2:  OD-SSB configuration can be independent to SCell configuration, to reduce the avoid huge signalling overhead.

Proposal 3: RAN2 should discuss whether to postpone the MAC CE design until RAN1 has more progress to avoid duplicate discussion.

Proposal 4: If multiple-configuration of OD-SSB transmission is supported, the index of applicated configuration can be indicated in OD-SSB transmission indication MAC CE.

Proposal 5: For the cases that always-on SSB and OD-SSB are on the different frequency and always-on SSB and OD-SSB are on the same frequency and there’s time domain overlapping, UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB.

Proposal 6: For the case that always-on SSB and OD-SSB are on the same frequency and there’s no time domain overlapping, at least OD-SSB is measured, while whether to measure always-on SSB may depend on the discussion of L3 measurement framework.

Proposal 7: The UE measurements OD-SSB based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted.

Proposal 8: RAN2 should discuss whether a separate reporting is needed for measurement based on OD-SSB.

- References

- Report of 3GPP TSG RAN WG2 meeting #127bis;

- Report of 3GPP TSG RAN WG1 meeting #117;

- Report of 3GPP TSG RAN WG1 meeting #118bis;

- Report of 3GPP TSG RAN WG1 meeting #118.