3GPP TSG-RAN WG2 Meeting #128	R2-2410323

# Orlando, USA, Nov. 18th – 22nd, 2024

# Agenda item:	8.6.2

# Source:	CMCC

# Title:	Discussion on inter-CU LTM

# Document for:	Discussion, Decision

- Introduction

In RAN2#127bis meeting, RAN2 reached the following agreements on inter-CU LTM [1]:

Agreements on inter-CU LTM

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

- The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

5.	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7.	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

9.	Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

10.	Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

11.	RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

In this paper, we further discuss the open issues of inter-CU SCG LTM and the coexistence of different LTM types.

- Discussion

  - Inter-CU SCG LTM

    - Reference configuration

In last meeting, agreements related to (SN-initiated) inter-CU SCG LTM procedure were achieved, but there’s FFS on the reference of inter-CU SCG LTM.

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

Reference configuration is the configuration provided by the network to the UE that is common, within the same cell group, to a group of configured non-complete candidate configurations. And during the LTM preparation phase, it is sent by MN to the candidate cells to configure non-complete candidate configurations. For inter-CU  SCG LTM, it can be treated as a kind of variant of SN change/CPAC/SCPAC, and RAN2 agreed to use Rel-18 SCPAC principles as baseline for inter-CU SCG LTM. For SCPAC, SN addition procedure is used in the preparation phase. And mcg-RB-Config can be included in CG-ConfigInfo used by the SN to support delta configuration to UE. Besides, the reference configuration generated by the source or candidate SCG for SCPAC may have effect on MCG configuration. Therefore, we think there’s no reason to limit that reference configuration of inter-CU SCG LTM only contains SCG part.

Observation 1: In legacy SN Change procedure, mcg-RB-Config can be included in CG-ConfigInfo used by the SN to support delta configuration to UE.

Observation 2: The SCG part of SCG LTM generated by source or candidate SCG may have effect on MCG configuration.

Proposal 1: Reference configuration of inter-CU SCG LTM can include MCG part.

It was agreed that	 for the inter-CU SCG LTM configuration, SN generates SCG part configuration, and MN includes it into its MN RRC configuration message. But for the SCG part generated by candidate SCG, it cannot be modified by MN, then it is delivered to UE in a RRC container of MCG. And for MCG part, it’s an optional part. Therefore, we don’t think that network cannot ensure that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations. And it’s UE implementation when to perform compliance check of LTM configuration. If the LTM configuration is invalid after compliance check, UE can trigger SCGFailureInformation reporting.

Observation 3: Since MN cannot modify the configuration provided by SN and it’s optional to include MCG part by MN, but delivery to the UE in a RRC container, therefore, the network cannot ensure that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

Proposal 2: The network, i.e., MN is not responsible to ensure that the configuration generated by UE contains both MCG and SCG part configurations.

Proposal 3: If the inter-CU SCG LTM configuration is invalid, for example, MCG/SCG part is missing, the UE can trigger SCGFailureInformation reporting.

    - Co-existence of different types of LTM

For the coexistence of different types of LTM, RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

Inter-MN LTM and intra-SN LTM

If inter-MN LTM is performed first, there could be two cases after one LTM cell switch:

- Source SN is released or changed

- Source SN is kept

For case 1, source SN is released or changed after one MCG LTM, the intra-SN LTM configuration can be kept for subsequent LTM if the source MN is configure as a candidate cell of MCG LTM.

As we agreed that upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied). For case 2, if SN is kept, intra-CU SCG LTM configuration should be updated, for example, to reconfigure the Rel-18 LTM NoReset ID.

Proposal 4: In case inter-MN LTM and intra-SN LTM is configure, if inter-MN LTM is performed first:

- If the source SN is released or changed after one MCG LTM, the intra-SN LTM configuration can be kept for subsequent LTM if the source MN is configure as a candidate cell of MCG LTM.

- If the source SN is kept after one inter-MN LTM, intra-SN LTM configuration could be updated by the network.

If intra-SN LTM is performed first, it means the PsCell of the SCG is changed, and it may come with SCG bearer or SCG leg release, which may introduce involvement of MCG. And we think the potential effect on inter-MN LTM configuration is the same with its effect on intra-MN LTM configuration when configured together as discussed in Rel-18. While  in Rel-18, only intra-MN LTM and inter-SN LTM without MN involvement can be configured together. The same principle can be reused, in other words, only to support only intra-CU SN LTM without MCG involvement configured with inter-MN LTM.

Observation 4: In Rel-18, intra-CU LTM is configured with intra-SN LTM without MN involvement.

Proposal 5: It’s proposed to support inter-MN LTM configured with intra-SN LTM without MN involvement.

Inter-SN LTM and intra-MN LTM

Similar to the discussion above, in some case, the execution of LTM in node may introduce impact to the configuration of the LTM configuration in the other node.

If intra-MN LTM is triggered first, whether inter-SN LTM configuration can be valid in some cases, i.e., there’s only PCell change, but not UP configuration change like bearer type change. There’s similar discussion for Rel-18 S-CPAC. Following is the agreements made to considering the impact of PCell change to SCPAC:

RAN2#121 meeting agreements [2]: - In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case). RAN2#123bis meeting agreements [3]: P2: Upon intra-MN PCell change, it’s up to the NW decision to maintain/modify/release the subsequent CPAC configuration.

The same principle can be applied to intra-MN LTM and inter-SN LTM coexistence case, once intra-MN LTM is triggered, it’s up to the NW decision to maintain/modify/release inter-SN LTM configuration.

In our view, since inter-SN LTM takes SCPAC as baseline, and it will change SN node, MN involvement cannot be avoided, but whether to modify intra-MN LTM configuration after inter-SN LTM can be up to network implementation, since both intra-MN LTM configuration and inter-SN LTM configuration are generated by MN.

Proposal 6: For inter-SN LTM and intra-MN LTM co-existence case, it’s up to the MN decision to maintain/modify/release one node’s LTM configuration after the other node’s LTM is performed.

For intra/inter-MN LTM, it’s DU of MN to send LTM cell switch MAC CE to the UE, while for intra/inter-SN LTM, it’s SN to send LTM cell switch MAC CE to the UE, to avoid simultaneous execution of MCG and SCG LTM,  it’s better to introduce Xn interaction, for example, when MN decides to trigger LTM, it indicates to SN to avoid simultaneous execution, and vice versa.

Proposal 7: Xn indication can be considered between MN and SN to indicate LTM in one node to the other one to avoid simultaneous execution in MN and SN.

    - Rel-19 ID for L2 handling

RAN2 agreed to use a new Rel-19 ID in RRC for the UE to identify whether security key updating and the related L2 handling including MAC reset, RLC re-establishment and PDCP re-establishment are required [127]. And in last meeting, the UE behaviour considering both Rel-18 LTM-NoResetID and the new Rel-19 ID is discussed. General understanding  of the granularity of the new Rel-19 ID is a cell, same with Rel-18 LTM-NoResetID. But in the discussion, some company mentioned that security information updating may be not needed in all DC cases, i.e., for MCG terminated SCG bearers, inter-CU SCG LTM may not cause the change of the termination point. Therefore, it is proposed to set the new Rel-19 ID per RB level. But it should be noticed that, based on TS 37.340, No L2 handling case only happens when there’s no bearer type change and no termination change. In Rel-18 SCG LTM, though it is for intra-CU case, but it doesn’t mean there’s no bearer type change. But Rel-18 LTM-NoReset ID is still applied to a cell. Considering the complexity, we prefer that the new Rel-19 ID for DC is configured per cell as the new Rel-19 new ID for non-DC case.

Proposal 8: The same principle is applied to the new Rel-19 ID for non-DC case and DC case, that  the new Rel-19 ID is configured per cell.

  - Inter-CU MCG LTM

    - SCG handling in inter-CU MCG LTM

In RAN2#127 meeting [4], it was agreed that SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). One possible cased mentioned in [] is that SCG is release in one hop of inter-CU MCG LTM, while in the following LTM, candidate MCG may work with SCG, therefore, it seems SN addition procedure is performed. In our view, since whether to keep/change or release SCG is decided by the candidate MN, if the candidate MN decides to keep/change a SN, it will trigger SN addition procedure in LTM preparation phase, similarly to the inter-Master node handover with or without SCG change. In other words, SN addition is performed from network perspective, irrespective whether LTM is performed or not. From UE perspective, since SCG configuration is part of  MCG LTM configuration, SN addition is performed only when inter-CU MCG LTM configuration applied.

Proposal 9: SN addition procedure is supported:

- From network perspective, SN addition is performed when the candidate MCG decides to add one SCG;

- From UE perspective, SN addition is performed when LTM cell switch happens and the related configuration applied.

As to the configuration, since subsequent LTM is supported for inter-CU MCG LTM, therefore, the configuration of the source MCG (with or without SCG) and candidate MCG are not released without explicit network indication. Therefore, even if SCG is released in one hop of LTM, and UE access to one candidate MCG with SCG using delta configuration, valid configuration can be achieve.

Proposal 10: Both delta configuration and full configuration can be used for inter-CU MCG LTM with SCG kept or released.

    - UE-based TA measurement

In Rel-18 intra-CU LTM, except L2 handing, the UE’s execution of UE-based TA measurement is also based on the related ID comparison of the serving cell and candidate cell. In the current spec, it’s UE implementation on how to measure TA when this way is configured by the network, and there’s no restrictions on the scenarios  in which UE can be configured with UE-based TA measurement. In other words, whether to configure UE-based TA measurement to the UE is a kind of network implementation, maybe it is decided based on the prior information in the serving cell.

Observation 5: In Rel-18 intra-CU LTM, whether to configure UE-based TA measurement is up to network implementation.

When it comes to Rel-19 inter-CU LTM, we think the same principle can be applied, the serving cell can configure UE-based TA measurement to the UE. If UE cannot access to the target cell successfully, it can fallback to legacy RACH procedure.

Proposal 11: UE-based  TA  measurement can be supported in Rel-19 inter-CU LTM.

    - L3 measurement results triggered inter-CU LTM

In the discussion of decision node of inter-CU LTM in RAN2#127 meeting, L3 measurement results triggered LTM is mentioned, which is also discussed for Rel-18 intra-CU LTM. The reason to support L3 measurement results triggered LTM is that the UE capability of LTM and L1 measurement reporting is decoupled, so that UE supporting LTM may have no L1 measurement results. To progress L3 measurement results triggered LTM, an LS was sent to RAN3 [5]. However, since it is triggered by intra-CU LTM, it’s not clear whether L3 measurement results triggered inter-CU LTM  is supported. In our view, since RAN2 agreed to support the coexistence of intra-CU LTM and inter-CU LTM in a UE, there’s no need to preclude the LTM supporting UE without L1 measurement capability. Therefore, we think inter-CU LTM can be triggered by L3 measurement results.

Proposal 12: RAN2 clarifies that inter-CU LTM can be triggered by L3 measurement results.

For L1 measurement results triggered LTM, the gNB-DU sends the DU-CU CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch command to the UE including the target cell ID and the TCI state ID after sending LTM cell switch command to the UE.

Observation 6: In current LTM procedure, the gNB-DU sends the DU-CU CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch command to the UE including the target cell ID and the TCI state ID after sending LTM cell switch command to the UE.

However, when it comes to L3 measurement result triggered LTM, it’s rational that gNB-CU only indicates the target cell ID and related information but not L3 measurement results, in this case, there’s no need for the gNB-DU to send target cell ID to the gNB-CU again.

Proposal 13: For L3 measurement results triggered LTM, it’s not needed for the source gNB-DU to indicate cell ID to the source gNB-CU after it sends LTM cell switch command to the UE.

The motivation to support L3 measurement based LTM is for the UE without L1 measurement capability. But when it comes to the UE with L1 measurement capability, it’s not clear, whether the gNB-CU can request the gNB-DU to send LTM cell switch command based on L3 measurement results. If the gNB-CU can request the gNB-DU to send LTM switch command for the UE with L1 measurement capability, it may cause conflict between the gNB-CU and the gNB-DU on the target cell selection. In our opinion, LTM is to take advantage of L1 measurement, therefore, for  the UE with L1 measurement capability, the gNB-CU should not request LTM based on L3 measurement results.

Proposal 14: For the UE with capability of L1 measurement, the source gNB-CU cannot request the source gNB-DU to trigger LTM cell switch command based on L3 measurement results.

    - Inter-CU LTM security updating

SA3 send an LS to RAN2 to reply security handling for inter-CU LTM in non-DC case[6]:

# 1 Overall description SA WG3 thanks RAN WG2 for the LS on the security handling for inter-CU LTM in non-DC cases. SA WG3 has been considering the security mechanisms proposed for the inter-CU LTM feature. For this feature, SA WG3 want to preserve the principle that when there is a PDCP anchor change, then it is necessary to perform a key change (i.e. for solutions or handovers that do not require a PDCP anchor change, there is no need to provide rekeying). The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure. SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like, - Security threat of the unprotected MAC CE to deliver the NCC value. - Sending NCC value per candidate cell via RRC message during LTM preparation phase. SA WG3 will keep RAN WG2 and RAN WG3 informed as we make further progress on this topic. # 2 Actions To RAN2: ACTION: SA3 kindly asks RAN WG2 to take the above information into account and provide feedback to SA WG3.

From SA3 perspective, it’s preferred to use RRC signalling to provide NCC value after every inter-CU LTM cell switch execution. And SA3 understands RAN2’s motivation to avoid RRC reconfiguration between LTM cell switches and looks forward to RAN2’s reply on the necessity of avoiding RRC reconfiguration. In our view, though RRC reconfiguration for non-LTM purpose cannot avoided, it’s better to avoid RRC reconfiguration for LTM, RAN2 should confirm our initial intention. As to the mechanism, we think sending NCC value per candidate cell via RRC message during LTM preparation phase can be further discussed, considering that it’s safer than MAC CE deliver way. And it can be applied to inter-CU conditional LTM if supported in the further. Besides, for DC case, SCPAC security key update mechanism with NCC value pre-configured is adopted as baseline[7]. With this way, a unified solution can be achieved , and the workload for SA3 may be reduced.

Observation 7: Compared to using MAC CE for NCC delivery, providing NCC value in LTM configuration is safer and is more suitable to inter-CU conditional LTM (if supported in the further) and it can achieve a unified solution for   with DC case.

Proposal 15: RAN2 confirms the necessity of avoiding RRC reconfiguration between LTM cell switches.

Proposal 16: It’s proposed to use sending NCC value per candidate cell via RRC message during LTM preparation phase for non-DC inter-CU LTM and inform RAN2 intention to SA3 for final decision.

- Conclusion

In this contribution, we discuss inter-CU LTM related issue. Following are the proposals and observations made in this contribution:

Observations:

Observation 1: In legacy SN Change procedure, mcg-RB-Config can be included in CG-ConfigInfo used by the SN to support delta configuration to UE.

Observation 2: The SCG part of SCG LTM generated by source or candidate SCG may have effect on MCG configuration.

Observation 3: Since MN cannot modify the configuration provided by SN and it’s optional to include MCG part by MN, but delivery to the UE in a RRC container, therefore, the network cannot ensure that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

Observation 4: In Rel-18, intra-CU LTM is configured with intra-SN LTM without MN involvement.

Observation 5: In Rel-18 intra-CU LTM, whether to configure UE-based TA measurement is up to network implementation.

Observation 6: In current LTM procedure, the gNB-DU sends the DU-CU CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch command to the UE including the target cell ID and the TCI state ID after sending LTM cell switch command to the UE.

Observation 7: Compared to using MAC CE for NCC delivery, providing NCC value in LTM configuration is safer and is more suitable to inter-CU conditional LTM (if supported in the further) and it can achieve a unified solution for   with DC case.

Proposals:

Proposal 1: Reference configuration of inter-CU SCG LTM can include MCG part.

Proposal 2: The network, i.e., MN is not responsible to ensure that the configuration generated by UE contains both MCG and SCG part configurations.

Proposal 3: If the inter-CU SCG LTM configuration is invalid, for example, MCG/SCG part is missing, the UE can trigger SCGFailureInformation reporting.

Proposal 4: In case inter-MN LTM and intra-SN LTM is configure, if inter-MN LTM is performed first:

- If the source SN is released or changed after one MCG LTM, the intra-SN LTM configuration can be kept for subsequent LTM if the source MN is configure as a candidate cell of MCG LTM.

- If the source SN is kept after one inter-MN LTM, intra-SN LTM configuration could be updated by the network.

Proposal 5: It’s proposed to support inter-MN LTM configured with intra-SN LTM without MN involvement.

Proposal 6: For inter-SN LTM and intra-MN LTM co-existence case, it’s up to the MN decision to maintain/modify/release one node’s LTM configuration after the other node’s LTM is performed.

Proposal 7: Xn indication can be considered between MN and SN to indicate LTM in one node to the other one to avoid simultaneous execution in MN and SN.

Proposal 8: The same principle is applied to the new Rel-19 ID for non-DC case and DC case, that  the new Rel-19 ID is configured per cell.

Proposal 9: SN addition procedure is supported:

- From network perspective, SN addition is performed when the candidate MCG decides to add one SCG;

- From UE perspective, SN addition is performed when LTM cell switch happens and the related configuration applied.

Proposal 10: Both delta configuration and full configuration can be used for inter-CU MCG LTM with SCG kept or released.

Proposal 11: UE-based  TA  measurement can be supported in Rel-19 inter-CU LTM.

Proposal 12: RAN2 clarifies that inter-CU LTM can be triggered by L3 measurement results.

Proposal 13: For L3 measurement results triggered LTM, it’s not needed for the source gNB-DU to indicate cell ID to the source gNB-CU after it sends LTM cell switch command to the UE.

Proposal 14: For the UE with capability of L1 measurement, the source gNB-CU cannot request the source gNB-DU to trigger LTM cell switch command based on L3 measurement results.

Proposal 15: RAN2 confirms the necessity of avoiding RRC reconfiguration between LTM cell switches.

Proposal 16: It’s proposed to use sending NCC value per candidate cell via RRC message during LTM preparation phase for non-DC inter-CU LTM and inform RAN2 intention to SA3 for final decision.

- References

- Report of 3GPP TSG RAN WG2 meeting #127bis;

- Report of 3GPP TSG RAN WG2 meeting #121;

- Report of 3GPP TSG RAN WG2 meeting #123bis;

- Report of 3GPP TSG RAN WG2 meeting #127;

- R2-2407771; L3 measurement based LTM support over F1; Source: RAN2; To: RAN3;

- R2-2409534; Reply LS on security handling for inter-CU LTM in non-DC cases; Source: SA3; To: RAN2;

- R2-2409535; Reply LS on security key update of inter-CU SCG LTM; Source: SA3; To: RAN2.