3GPP TSG RAN WG2 Meeting #128	R2-2410331

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.8.4

Source:	CMCC

Title:	Considerations on broadcast service for NR NTN

Document for:	Discussion, Decision

Introduction

In last RAN2 meeting, there were some discussions on support of broadcast service and some agreements were achieved as follows [1]:

Agreements:

1.	For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

In this contribution, we would like to provide some further considerations on the broadcast service for NTN system.

Discussion

Intended service areas (and related IDs) provision

- In last meeting, we agreed to include one more intended service area IDs into MCCH. However, the specific provision solution of intended service areas with corresponding area IDs is FFS. And alternatives are MCCH and a new or existing SIB.

- As companies comments, the remaining size of existing SIB is insufficient to include the intended service areas information. Then the MCCH has no size limitation, which already carries the MBSBroadcastConfiguration message which indicates the MBS broadcast sessions that are provided in the cell as well as the corresponding scheduling related information for these sessions. Further, using MBSBroadcastConfiguration could provide both service area information and association between service area and MBS session. Specifically, include association (e.g. one to one, one to multiple, multiple to one) between intended service area ID and MBS session ID. And a new SIB should be considered last, unless the MCCH based solution is determined unworkable.

- Proposal 1: Using MCCH based solution to include the intended service areas (and related IDs) information.

- Proposal 2: A new SIB should be considered last, unless the MCCH based solution is determined unworkable.

- Then for the solution to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service. Per the current TS 38.331, it is up to UE implementation to use the cell/tracking area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service. Similarly, for support broadcast service in NTN, it could be also up to UE implementation to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service based on the intended service areas provision by network.

- Proposal 3: It could be also up to UE implementation to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service based on the intended service areas provision by network.

MBS service announcement was raised to indicate the intended service area in last meeting.

R2-2409184 Support for broadcast services in NR NTN Ericsson discussion Rel-19 NR_NTN_Ph3-Core Proposal 1 The intended service area of an MBS broadcast service is not provided in system information. FFS on other broadcast services (e.g., ETWS). Proposal 2 MBS Service Announcement is used to indicate the intended service area for MBS broadcast in NTN cells. - CATT wonders if we can reuse the MBS service announcement as is or if we need to ask SA2 to introduce changes. Ericsson confirms we need to ask SA2 to perform changes. - LG supports the Ericsson proposal as this would be the cleanest way and thinks we can send an LS to SA2. - Apple thinks we cannot exclude enhancements in AS layer if the information that SA2 can provide is at cell level. - Samsung wonders if this would be consistent with the WI objective - CMCC wonders if SA2 would have time to work on this in Rel-18 - Nokia wonders if the issue is so severe that we cannot solve it in RAN2 and need to involve other groups - Lenovo thinks it’s not a good idea to go for a NAS solution, and we would still need to address NAS/AS interaction. Oppo agrees - Thales thinks a NAS solution would probably work but the WID objective is to work on a AS solution. - Ericsson wonders if we can work on both approaches - Samsung can accept to send an LS to ask - Apple thinks it’s not clear how we could test the UE behaviour if we only rely on a NAS based solution.

From our perspective, providing intended service area via MBS service announcement is not align the WID scope (as shown below) which determines to specify SIB signaling to indicate the intended service area.

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

And MBS service announcement solution would have impact on SA2, while they only has limited remaining time for R19, which means SA2 is most likely running out of time to discuss and solve the issue. Therefore, MBS service announcement solution should not be pursued in R19.

Proposal 4: Not to pursue the MBS service announcement solution in R19.

MCCH information update

- As in current TS 38.331, the MCCH information validity and notification of changes are defined as following:

5.9.1.3 MCCH information validity and notification of changes Change of MCCH information only occurs at specific radio frames, i.e. the concept of a modification period is used. Within a modification period, the same MCCH information may be transmitted a number of times, as defined by its scheduling (which is based on a repetition period). When the network changes (some of) the MCCH information, it notifies the UEs about the change starting from the beginning of the MCCH modification period via PDCCH which schedules the MCCH in every repetition in that modification period. Upon receiving a change notification, a UE receiving or interested to receive MBS services transmitted using MBS broadcast acquires the new MCCH information starting from the same slot. The UE applies the previously acquired MCCH information until the UE acquires the new MCCH information. The notification is transmitted with a 2-bit bitmap, see TS 38.212 [17] clause 7.3.1.5.1. The MSB in the 2-bit bitmap, when set to '1', indicates the start of new MBS service(s). The LSB in the 2-bit bitmap, when set to '1', indicates modification of MCCH information other than the change caused by start of new MBS service(s), e.g. modification of a configuration of an on-going MBS session(s), MBS session(s) stop or neighbouring cell information modification.

- Then for MCCH update due to service area information changing, in order to reduce impact on RAN1, we could consider to leverage the existing mechanism and just add some clarification to include the service area information changing. Specifically, utilize the LSB in the 2-bit bitmap, and just add description to include the service area information changing (e.g. service areas modification), in the “e.g. modification of a configuration of an on-going MBS session(s),...” part.

Proposal 5: For MCCH update due to service area information changing, we could consider to leverage the existing mechanism and just add some clarification under “The LSB in the 2-bit bitmap” part to include the service area information changing (e.g. service areas modification).

Service continuity

In last meeting, we have discussed on service continuity as following:

Proposal 4: Service continuity can be enhanced in two places by considering intended service area. • In MBSBroadcastConfiguration, the intended service area within each neighbor cell is provided for each MBS broadcast service session. • In SIB21, the intended service area is provided for each MBS broadcast service session, or for each FSAI (depending on common understanding selected). - Ericsson thinks we should focus on the scenario where the quasi-Earth-fixed cell is replaced due to satellite movement - ZTE thinks we currently support both and then supports p4 - QC also thinks both scenarios should be supported. - HW thinks this covers the case of UE movement. For the satellite movement there would be no difference wrt previous releases. HW is ok to reflect the second part of p4 - Ericsson disagrees with the proposal as there is no need for service continuity for this. Apple thinks this is needed for cell reselection. - CATT wonders whether we really need to provide the service area for service continuity, in a per neighbour cell way. - Ericsson thinks we need to clarify the UE behavior when receiving this information Come back in the next meeting

From our perspective, providing service area information for each MBS broadcast session of neighbour cells has some benefit for reselection and handover, it could help UE to select a more suitable cell to continue receiving the same broadcast services. As known, MCCH may also contain a list of neighbour cells providing the same broadcast MBS service(s) as provided in the current cell. In order to guarantee service continuity better, providing service area information per neighbour cell in the MBSBroadcastConfiguration could help UE to reselect or handover to a neighbour cell including service areas association with the MBS session UE interested in or providing same broadcast service which UE is receiving currently.

Then SIB21 provides information related to service continuity, which contains the mapping between the current and/or neighbouring carrier frequencies and MBS Frequency Selection Area Identities (FSAI). However, considering the different understanding about the relation between FSA and MBS session (i.e. one to one, one to multiple), to enhance service continuity, we could just consider to introduce intended service area for each MBS broadcast service session in SIB21.

Proposal 6: It is proposed to provide intended service area within each neighbor cell for each MBS broadcast service session in MBSBroadcastConfiguration.

Proposal 7: It is proposed to provide intended service area for each MBS broadcast service session in SIB21.

MBS interest indication

We discussed that whether a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area without convergence.

As current specification, the MBS interest indication procedure is to inform the network that the UE in RRC_CONNECTED is receiving or is interested to receive MBS broadcast service(s) and/or to inform the network about the priority of MBS broadcast versus unicast and multicast MRB reception. MBS Interest Indication can only be sent after AS security activation. And the trigger condition is described in section 5.9.4.2 of TS 38.331.

- 5.9.4.2 Initiation An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell. If the UE does not have the CFR information and subcarrier spacing for MBS broadcast reception on a non-serving cell at the time it sends the MBS Interest Indication, the UE sends another MBS Interest Indication after it has acquired this information from the non-serving cell.

In the contribution [2], it is mentioned that initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area could help NW to choose a suitable cell providing the broadcast service for this UE. However, per the exiting mechanism, UE already supports to initiate the MII procedure upon handover. And in NTN, UE is able to determine whether in the intended service area based its own GNSS and service area information provided by network. Further, network may have no acknowledge of the UE location, i.e., UE is in which service area may not be clear at network side. Then whether initiate the MII procedure could be just up to UE itself. Therefore, further enhancement for MBS Interest Indication initiation is not needed.

Proposal 8: It is proposed not to pursue that a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area.

Signaling format of service area

Regarding the service area indication, we agreed that the intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

- For the former solution, similar to TN coverage information introduced in NTN R18 to assist TN neighbour cell measurements, we could consider to introduce a new IE (e.g. IntendedServiceAreaInfo-r19) as follows to provide intended service area ID, service area reference location and distance radius information to UE. Then UE receive the broadcast service based on whether in the service area or whether interested in the service.

IntendedServiceAreaInfo-r19 ::=       SEQUENCE {

intendedServiceAreaId-r19                  IntendedServiceAreaId-r19,

intendedServiceArea-ReferenceLocation-r19       ReferenceLocation-r17,

intendedServiceArea-DistanceRadius-r19          INTEGER(0..65536)

Proposal 9: For the referenceLocation + radius solution, the following new IE format could be discussed:

IntendedServiceAreaInfo-r19 ::=       SEQUENCE {

intendedServiceAreaId-r19                  IntendedServiceAreaId-r19,

intendedServiceArea-ReferenceLocation-r19       ReferenceLocation-r17,

intendedServiceArea-DistanceRadius-r19          INTEGER(0..65536)

Then for the latter solution (polygon), it is already captured in section 5.4 of TS 23.032.

- 5.4 Polygon A polygon is an arbitrary shape described by an ordered series of points (in the example pictured in the drawing, A to E). The minimum number of points allowed is 3, and the maximum number of points allowed is 15. The points shall be connected in the order that they are given. A connecting line is defined as the line over the ellipsoid joining the two points and of minimum distance (geodesic). The last point is connected to the first. The list of points shall respect a number of conditions: - a connecting line shall not cross another connecting line; - two successive points must not be diametrically opposed on the ellipsoid. The described area is situated to the right of the lines with the downward direction being toward the Earth's centre and the forward direction being from a point to the next. NOTE: This definition does not permit connecting lines greater than roughly 20 000 km. If such a need arises, the polygon can be described by adding an intermediate point. Computation of geodesic lines is not simple. Approximations leading to a maximum distance between the computed line and the geodesic line of less than 3 metres are acceptable. Figure 3: Description of a Polygon

To support both the solutions, a “CHOICE” signaling structure could be considered. And based on NW configuration to provide the intended service area information

Proposal 10: It is proposed to use “CHOICE” signaling structure to support both the service area format solutions.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on MBS broadcast enhancements for NTN and have the following proposals:

- Proposal 1: Using MCCH based solution to include the intended service areas (and related IDs) information.

- Proposal 2: A new SIB should be considered last, unless the MCCH based solution is determined unworkable.

- Proposal 3: It could be also up to UE implementation to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service based on the intended service areas provision by network.

Proposal 4: Not to pursue the MBS service announcement solution in R19.

Proposal 5: For MCCH update due to service area information changing, we could consider to leverage the existing mechanism and just add some clarification under “The LSB in the 2-bit bitmap” part to include the service area information changing (e.g. service areas modification).

Proposal 6: It is proposed to provide intended service area within each neighbor cell for each MBS broadcast service session in MBSBroadcastConfiguration.

Proposal 7: It is proposed to provide intended service area for each MBS broadcast service session in SIB21.

Proposal 8: It is proposed not to pursue that a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area.

Proposal 9: For the referenceLocation + radius solution, the following new IE format could be discussed:

IntendedServiceAreaInfo-r19 ::=       SEQUENCE {

intendedServiceAreaId-r19                  IntendedServiceAreaId-r19,

intendedServiceArea-ReferenceLocation-r19       ReferenceLocation-r17,

intendedServiceArea-DistanceRadius-r19          INTEGER(0..65536)

Proposal 10: It is proposed to use “CHOICE” signaling structure to support both the service area format solutions.

Reference

- R2-127bis NR-NTN-IoT-NTN (Sergio)_ EOM

- R2-2408016	Further Discussion on MBS Broadcast Provision in NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core