3GPP TSG-RAN WG2 Meeting #128       	                      R2-2410342

Orlando, US, November 18th-22nd, 2024

Agenda item:	8.1.2.2

Source:	CMCC

Title:	Discussion on LCM for UE-sided model for BM

WID/SID:	NR_AIML_air-Core

Document for:	Discussion

- Introduction

In last meeting [1], RAN2 discussed LCM for UE-sided model for BM and achieved the following agreements.

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- FFS if inference configuration can be signalled in step3.

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

- Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

- For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1).

- For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models.  Can come back to this discussion later.

This contribution will discuss the remaining issues on UE-sided model LCM for beam management use case.

- Discussion

- 2.1 Applicable functionality reporting

- In last meeting, RAN2 agreed UE can report to the network when an applicable AI functionality becomes non-applicable, and FFS how this is signaled (e.g. explicitly/implicitly). We understand that a smart network can be aware of the non-applicable functionality based on applicable functionality and supported functionality reported by the UE. There is no need to report the non-applicable functionality explicitly.

- Proposal 1: There is no need to signal non-applicable functionality to the network, since it can be indicated implicitly based on applicable functionality and supported functionality reported by UE.

- 2.2 Functionality identification

- During R18 study phase, RAN1 agreed to take the UE capability reporting as starting point for functionality identification and the UE can report the applicable functionality after functionality identification, which were captured in TR 38.843 [2].

- 4.2.1 LCM Flavours The LCM procedure is studied for the case that an AI/ML model has a model ID with associated information and/or for the case that a given functionality is provided by some AI/ML operations. Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion. From RAN1 perspective, an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation. When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model. For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Legacy 3GPP framework of feature is taken as a starting point. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. <omitted> After functionality identification, necessity, mechanisms, for UE to report updates on applicable functionality(es) among functionality(es) are studied, where the applicable functionalities may be a subset of all functionalities. Applicable functionalities can be reported by the UE. <omitted>

- In R19, RAN2 discussed the basic procedure of applicable functionality reporting which includes the UE capability reporting containing the supported functionalities in UE side. In our understanding, functionality identification is achieved in step 1 and step 2. Therefore, we think there is no need to introduce an additional definition and procedure for functionality identification.

- Proposal 2: There is no need to introduce the definition and procedure for functionality identification since it is achieved in UE capability reporting (i.e. step1/2 of applicable functionality reporting).

- 2.3 Data collection for training

- For UE side data collection for model training, RAN2 discussed the following options but had no consensus.

Option 1a. UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

Option 1b. UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO).

Option 2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

Option 3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

- In RAN#105 meeting, RAN plenary sent a LS [3] on data collection to SA and expected at least SA2 to provide inputs on solution 1b, 2 and 3 by RAN#106. After that, RAN2 will continue to down select among the potential options.

- In last meeting, RAN2 discussed the mechanism for data collection, and agreed that data collection initiation and configuration for data collection is under network control, and FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server).

- Since the data for training is terminated in OTT server, we think it is straightforward that OTT server requests which type of data is expected to the network (e.g. CN, OAM, RAN). And then the network will send the configuration to the UE for data collection. We don’t see the motivation for UE to request the data collection configuration.

- Proposal 3: OTT server can request which type of data is expected to the network, since the data collection for training is terminated in OTT server.

- In our understanding, the network can configure the UE based on UE capability, which is similar to SON/MDT. That is, if the UE supports the data collection for model training, the network can initiate the data collection by transmitting the configuration (e.g. Set A and/or Set B) to the UE and when to initiate is up to network implementation.

- Proposal 4: The network can configure the UE to collect data based on UE capability, and when to initiate data collection is up to network implementation.

# 3	Conclusion

Here are the observations and proposals for LCM for UE-sided model for beam management.

Applicable functionality reporting

Proposal 1: There is no need to signal non-applicable functionality to the network, since it can be indicated implicitly based on applicable functionality and supported functionality reported by UE.

Functionality identification

- Proposal 2: There is no need to introduce the definition and procedure for functionality identification since it is achieved in UE capability reporting (i.e. step1/2 of applicable functionality reporting).

Data collection for training

- Proposal 3: OTT server can request which type of data is expected to the network, since the data collection for training is terminated in OTT server.

Proposal 4: The network can configure the UE to collect data based on UE capability, and when to initiate data collection is up to network implementation.

# 4	Reference

- Draft_R2_127bis_Meeting_Report_v1

- TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- RP-242389 LS on AIML data collection To:	TSG SA, WG SA2, WG SA3, WG SA5