3GPP TSG-RAN WG2 Meeting #128		R2-2410403

Orlando, USA, 18th – 22nd November, 2024

Source: Vodafone, Huawei, CMCC, Telit Communications S.p.A., CATT, Nokia

Title:	CBRA type down-selection

Document for: Decision

Agenda Item: 8.2.4

## Introduction

There are two candidates for the contention based random access in the TR: i.e., 2step CBRA and 3step CBRA

- Step 2: Contention resolution of contention-based random access: - There are two candidate solutions being studied for the contention resolution, as below (it can be further discussed on the down selection and/or the unified design): - Solution 1: A-IoT Msg1 without data - A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader. - A-IoT Msg2: The reader responds with the successfully received random ID. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. Solution 2: A-IoT Msg1 with data - A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends the A-IoT Msg1 including the upper layer data, which can be the device ID and/or any other upper layer data, in addition to one 16-bit random ID generated by the A-IoT device to the reader. - A-IoT Msg2: The reader may respond with the successfully received random ID. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. If the A-IoT Msg2 is not received by the device, the re-access is not autonomously performed while the re-access is always controlled by the reader.

## Discussion

A-IoT devices are very cost effective devices where any additional complexity leading to increase costs might not be accepted by the market, it should be the quite simple device which should not support two redundant CBRA type.

Observation 1: A-IoT device should only support one simple CBRA type.

In 2step RA, A-IOT device combines MSG3 comprising device ID (more than 100 bits) with a MSG1 resulting in higher payload within Msg1. It may be more efficient in case the Msg1 can be successfully received, but there are two aspects limits the Msg1 successful probability: 1) radio link condition for a certain targeted coverage; 2) contention failure probability.

Observation 2: 2step CBRA has its potential benefit only in case of high Msg1 transmission success rate.

The key information to be delivered during the inventory procedure is a device ID. The device ID will be delivered in msg 1 in in 2step CBRA (i.e. 16bits random ID and about one hundred bits of device ID) and in the Msg3 in 3step CBRA (i.e. about one hundred bits of device ID).

The larger size of D2R transmission, the smaller coverage it can support for the same BLER in physical layer. Therefore, in the same target coverage, Msg1 in 2step CBRA suffers larger transmission failure rate compared to the Msg3 in 3step CBRA.

Observation 3a: At the same uplink target coverage, Msg1 in 2step CBRA suffers larger transmission failure rate, compared to the Msg3 in 3step CBRA. Hence, 2step CBRA is only suitable in the better coverage case, i.e. only using 2step CBRA will reduce/limit the uplink coverage.

In the slotted aloha procedure, the contention resolution failure will cause the failure of device ID transmission in 2step CBRA (i.e. two devices use the same access occasion to send their own Msg1). Therefore, the Msg1 collision probability impacts the performance of 2step CBRA. In order to reduce the probability of Msg1 collision, the reader needs to allocate more number of access occasions (i.e. more resources), which will result in more unselected access occasions (i.e. the access occasion not selected by any device). It will be the trade-off between access occasion waste and Msg1 collision. Some simulation analyses are referred in [1].

Observation 3b:  The 2step CBRA is only suitable in case of relatively low probability of Msg1 collision, which will cause more wasted access occasions.

Mixed deployment:

Taking the points above, it does not seem to be possible to combine the deployments supporting 2 STEP and 3STEP CBRA in one deployment, which would result in not truly open interface between reader and the device. It will also require the device to support both types of CBRA by default increasing the complexity of devices.

Observation 4: Specifying 2 types of CBRA might decrease the acceptance of the standard by the markets.

Therefore, it is proposed to down-selects the simple and universal solution, i.e. 3step CBRA-only.

Proposal 1:	RAN2 to down-select 3step CBRA only between 2step and 3step RACH

## Conclusions

Observation 1: A-IoT device should only support one simple CBRA type.

Observation 2: 2step CBRA has its potential benefit only in case of high Msg1 transmission success rate.

Observation 3a: At the same uplink target coverage, Msg1 in 2step CBRA suffers larger transmission failure rate, compared to the Msg3 in 3step CBRA. Hence, 2step CBRA is only suitable in the better coverage case, i.e. only using 2step CBRA will reduce/limit the uplink coverage.

Observation 3b:  The 2step CBRA is only suitable in case of relatively low probability of Msg1 collision, which will cause more wasted access occasions.

Observation 4: Specifying 2 types of CBRA might decrease the acceptance of the standard by the markets.

Proposal 1:	RAN2 to down-select 3step CBRA only between 2step and 3step RACH

## References:

[1] R2-2408243	A-IoT random access procedure	Huawei, HiSilicon