3GPP TSG-RAN WG2 Meeting #127-bis	R2-2410410

Orlando, US, 18 – 22 November, 2024

Agenda item:	8.7.6

Source:	InterDigital

Title:	Discussion on UL congestion signaling

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The WID objective wrt. UL congestion signalling was captured as follows [RP-241771]:

- Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

Furthermore, RAN2#127-bis made the following agreements:

FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc. RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - Extend the Bit Rate field - Define a new bit rate table to provide sufficient granularity for XR traffic - Introduce new values for the bitRateMultiplier Send LS to SA4 asking about range/granularity which is required

While SA4 hasn’t yet replied about the required range/granularity, RAN2 can already further discuss on whether the indication should be per DRB or per QoS flow and which approach(es) to use for recommended bit rate values for XR applications.

# 2	Discussion

## 2.1	Indication granularity

In the previous RAN2#127-Bis meeting, it was discussed whether the UL congestion signaling granularity should be on a per DRB or per QoS flow level, while interworking with L4S (which works in QoS flow level) was to be considered as well. Since RAN plenary agreed the congestion signaling to be at the MAC layer in the RAN, it seems simplest to use the per DRB/logical channel (LCH) granularity which is what the MAC sees today. Considering in going to per QoS flow level, the signaling layer being the MAC could be easily questioned: MAC does not see QoS flows in neither the UE nor at the NW side; on the other hand, this would also require signaling support in F1 interface to make MAC layer aware of the congested QoS flow(s) in the NW.

Observation: MAC does not see QoS flow(s) in neither the UE nor the NW side, hence, supporting QoS flow level UL congestion signaling at MAC layer would require signaling support in F1 interface.

Furthermore, many XR applications and their implementations are capable of rate adaptation of their own flows. For example, based on the quality of the connection, an XR application can adjust its bit rate on a per flow basis to adapt to the quality of the connection. Also, from user experience point of view, it seems the application itself is best in adjusting the bit rate/etc. for each flow when the connection is poor/congested. Hence, it should be sufficient to enable the UL congestion indication on a per DRB/LCH basis, and the application(s) can then adapt the bit rate(s) of flows to come up to the recommended bit rate for the DRB/LCH indicated by the NW.

Proposal 1: The UL congestion indication is signaled on a per DRB/LCH basis.

## 2.2	UL congestion signaling options

The Recommended Bit Rate procedure has been supported since Rel-15, and it can support the requirements of the UL congestion signaling in all other means, but the bit rate values. Hence, the Recommended Bit Rate procedure can be used for the new congestion indication.

Proposal 2: The Recommended Bit Rate procedure is used for the UL congestion signaling.

The Recommended Bit Rate MAC CE is specified as follows (TS 38.321):

#### 6.1.3.20 Recommended bit rate MAC CE The Recommended bit rate MAC CE is identified by a MAC subheader with LCID as specified in Tables 6.2.1-1 and 6.2.1-2 for bit rate recommendation message from the gNB to the UE and bit rate recommendation query message from the UE to the gNB, respectively. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.20-1): - LCID: This field indicates the identity of the logical channel for which the recommended bit rate or the recommended bit rate query is applicable. The length of the field is 6 bits; - Uplink/Downlink (UL/DL): This field indicates whether the recommended bit rate or the recommended bit rate query applies to uplink or downlink. The length of the field is 1 bit. The UL/DL field set to 0 indicates downlink. The UL/DL field set to 1 indicates uplink; - Bit Rate: This field indicates an index to Table 6.1.3.20-1. The length of the field is 6 bits. For bit rate recommendation the value indicates the recommended bit rate. For bit rate recommendation query the value indicates the desired bit rate; - X: Bit rate multiplier. For UEs supporting recommended bit rate multiplier, when bitRateMultiplier is configured for the logical channel indicated by LCID field, X field set to 1 indicates the actual value of bit rate is the value corresponding to the index indicated by the Bit Rate field multiplied by bitRateMultiplier as specified in TS 38.331 [5]. - R: reserved bit, set to 0. Figure 6.1.3.20-1: Recommended bit rate MAC CE

Table 6.1.3.20-1: Values (kbit/s) for Bit Rate field Index; NR Recommended Bit Rate value [kbit/s]; Index; NR Recommended Bit Rate value [kbit/s]. 0; Note 1; 32; 700. 1; 0; 33; 800. 2; 9; 34; 900. 3; 11; 35; 1000. 4; 13; 36; 1100. 5; 17; 37; 1200. 6; 21; 38; 1300. 7; 25; 39; 1400. 8; 29; 40; 1500. 9; 32; 41; 1750. 10; 36; 42; 2000. 11; 40; 43; 2250. 12; 48; 44; 2500. 13; 56; 45; 2750. 14; 72; 46; 3000. 15; 88; 47; 3500. 16; 104; 48; 4000. 17; 120; 49; 4500. 18; 140; 50; 5000. 19; 160; 51; 5500. 20; 180; 52; 6000. 21; 200; 53; 6500. 22; 220; 54; 7000. 23; 240; 55; 7500. 24; 260; 56; 8000. 25; 280; 57; Reserved. 26; 300; 58; Reserved. 27; 350; 59; Reserved. 28; 400; 60; Reserved. 29; 450; 61; Reserved. 30; 500; 62; Reserved. 31; 600; 63; Reserved. Note 1: For bit rate recommendation message this index is used for indicating that no new recommendation on bit rate is given..

The following options were agreed to be further studied to provide bit rate values for XR applications:

-	Extend the Bit Rate field;

-	Define a new bit rate table to provide sufficient granularity for XR traffic;

-	Introduce new values for the bitRateMultiplier;

Since the X field follows the Bit Rate field, it seems not a good option to extend the Bit Rate field – this would mean that a new MAC CE would be required for the purpose. Hence, simplest approach is to define the new bit rate table and/or new values for the bitRateMultiplier – better granularity can be achieved by defining both.

Proposal 3: Define new bit rate table and values for the/new bitRateMultiplier.

It should further be discussed whether the old bit rate table values/bitRateMultiplier values should be able to be used concurrently with the new values defined to support the XR applications. If yes, some dynamic indication would be needed to indicate which values to use (e.g., by exploiting one of the Reserved bits in the MAC CE or define a new (e)LCID). Otherwise, the current MAC CE can be used as is by RRC configuration on which table and bitRateMultiplier values to be used. Since the bit rates to be indicated to XR applications would be generally smaller and with better granularity to the legacy values, the RRC configuration options seems sufficiently dynamic for the purpose if NW intends to use the new UL congestion signalling for the UE.

Proposal 4: RRC configures whether the new bit rate table and/or bitRateMultiplier value is used and the legacy Recommended Bit Rate MAC CE is used as is for the UL congestion signaling.

# 3	Conclusion

This document has made the following observations and proposals:

Observation: MAC does not see QoS flow(s) in neither the UE nor the NW side, hence, supporting QoS flow level UL congestion signaling at MAC layer would require signaling support in F1 interface.

Proposal 1: The UL congestion indication is signaled on a per DRB/LCH basis.

Proposal 2: The Recommended Bit Rate procedure is used for the UL congestion signaling.

Proposal 3: Define new bit rate table and values for the/new bitRateMultiplier.

Proposal 4: RRC configures whether the new bit rate table and/or bitRateMultiplier value is used and the legacy Recommended Bit Rate MAC CE is used as is for the UL congestion signaling.