3GPP TSG-RAN WG2 Meeting #128	R2-2410501

# Orlando, FL, USA, 18th – 22nd	November 2024

Agenda Item:	8.7.4.2

Source:	Ericsson

Title:	DSR enhancements

Document for:	Discussion, Decision

# 1	Introduction

At RAN#105 the XR WID [1] for Rel-19 was updated with the following objective on the scheduling enhancements related to DSR:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

In RAN2#127bis the enhanced DSR solution was further clarified that the buffered data is divided into multiple portions where the DSR indicates a buffer size and a shortest remaining time for each portion that has a buffer size larger than zero. Additionally, one bit will indicate if a further portion is reported for and no PSI information will be included in the DSR. Remaining topics to be agreed is the details on the DSR format, the triggering threshold configuration, how to handle inclusion of mixed data and the relation to Rel18 DSR. This paper will discuss these remaining aspects.

# 2 Discussion on DSR enhancements

## 2.1 New Rel19 DSR format

It was agreed in previous meetings that there can be multiple reporting thresholds configured for each LCG. For different traffic requirements and scheduler implementations there may be varying usefulness in the granularity of the reported information. Thus it should be up to network decision how many thresholds that are used for each LCG.

Network configures the number of thresholds and their values for each LCG.

It was also agreed that not all thresholds will be reported every time and that a bit in the DSR format will be used to convey if further thresholds for the LCG is present in the report. A straightforward and simple solution, that also doesn’t require any change to the structure introduced in the Rel18 DSR format, is to reuse the Rel18 reserved bit for this purpose. How this solution can work is shown in Figure 1. In this solution the n-bit has replaced the unused R-bit and is now used to indicate if a further threshold is being reported for the current LCG. In #127bis questions were raised if this solution would work to skip thresholds but as is shown in the example in Figure 1 that can be done easily. Since the remaining time field already provides an implicit indication for which threshold the information is being reported for (the remaining time value can only be between the upper and lower threshold values) there is no ambiguity in the reporting. For each LCG this solution enables to start at any threshold, jump over any threshold and report for any number of thresholds. If only one threshold is being reported for each LCG then the size of the MAC CE is not larger than the Rel18 DSR.

Figure 1: Mapping of data from the LCGs, with different remaining times and buffer sizes, to a new DSR format using multiple thresholds and a n-bit indicating if a further threshold is present for the current LCG.

For Rel19 DSR reuse the structure of the Rel18 DSR but change the R-bit to a n-bit to indicate if a further threshold is present for the current LCG.

Adopt the DSR format shown in Figure 1.

## 2.2 Reporting mixed data in the DSR

The whole purpose of the enhanced DSR feature in Rel19 is to give the network scheduler more precise information about the delay situation in the UE buffer. When remaining time of any data falls below the reporting threshold a new DSR will be triggered and the network will receive a report of the complete buffer situation inside the reporting thresholds, i.e. with the configured remaining time granularity. In this way the network can schedule more precisely the grant allocations compared to the Rel18 solution. However, for this to work the network should receive as correct information as possible about the buffer size associated with the remaining time reported. If this is not the case the scheduling decision may be inferior and leading to additional delays, e.g. too small allocations. A simple example of this is shown in Figure 2.

Figure 2: Issue with non-delay critical data being transparent in the DSR. The network schedules too small allocation since it doesn’t receive the information about the non-delay critical data being ahead in the queue.

The transmission order of the data impacts the scheduling decision in the network. In the example above with a buffer including both delay critical data and non-delay critical data there can be two scenarios:

- Delay critical data will be transmitted first and network only needs to provide a grant with the size of the delay critical data to accommodate all the delay critical data.

- Non-delay critical data will be transmitted first and network need to provide a grant with the size of both the non-delay critical data and the delay critical data to accommodate all the delay critical data.

It is important that this order is reflected in the DSR as this provides the means for the network to take the correct scheduling decision. The simplest solution is that this is done by multiplexing the buffer size of the data that will be transmitted before the delay critical data, as is shown in Figure 3. If the UEs transmission order is reflected in the DSR then the network can take this into account when performing the scheduling.

The DSR provides the information of the size that needs to be scheduled to accommodate the delay critical data.

Figure 3: UE that will transmit non-delay critical data before the delay critical data multiplex the size of the non-delay critical data into the buffer size of the delay critical data in the DSR.

## 2.3 Triggering of the DSR

It has been agreed to use a single triggering threshold. What is left to be decided is if the triggering threshold can be configurable or always should be the largest threshold (i.e. the threshold with the longest remaining time). On one hand having it configurable is not a problem, in the end the same information will be reported in the DSR as this only depends on the reporting thresholds. However for simplicity having the trigger as the largest threshold (i.e. threshold with longest remaining time) seems to have no apparent drawbacks. In fact, it is hard to come up with a reason why the network would want to have the DSR triggered at a later time and not when the data enter the first reporting threshold. After all the network has configured this threshold because it wants to receive information about data with remaining time in this interval. Unless a good reason is presented why the threshold should be configurable the largest threshold (i.e. the one with the longest remaining time) can be agreed to always be used as the triggering threshold. An example of the DSR triggering and reporting will work is shown in Figure 4.

The threshold with longest remaining time can be used as the trigger threshold.

Figure 4: Example of a DSR configuration with multiple reporting thresholds. PDU Set Delay Budget (PSDB) is 30ms and triggering is based on the largest threshold (Threshold 1 = 26ms) while the second threshold is only used for reporting with higher granularity (Threshold 2 = 12ms).

## 2.4 Relation to Rel18 DSR

It has been suggested that the UE needs to be able to use a mix of Rel18 DSR and Rel19 DSR at the same time. The main claim being that the Rel19 DSR may be too large to fit in the UL grant. However, with the format proposed in this paper the size of the Rel19 DSR can be kept in control. Furthermore, when a DSR is triggered it will have higher priority than data and thus not have a problem to fit the grant. Generally, for all buffer reporting the only case when data is added to the grant before the reports is when an opportunistic report is sent using any left-over padding bits. This is not the case with the DSR, as no padding DSR has been agreed. As such no truncated DSR was agreed in Rel18 and it is not foreseen that such is needed for Rel19 either. Nevertheless, if some sort of truncated DSR would be wanted it should never be the mix of Rel19 with Rel18 solutions, as this would create severe ambiguity for the scheduler. There are many scenarios were mixing the Rel18 DSR with the Rel19 DSR would create problems for the network scheduler. An example being that a Rel19 DSR with high granularity is transmitted first and then another opportunistic DSR is added to a later transmission, using the Rel18 low granularity format. In such situation there is no way for the scheduler to make any good assumption of the correct status of the UE buffer.

Using Rel19 DSR together with Rel18 DSR will create large problems of ambiguity to the network scheduler.

Rel19 DSR and Rel18 DSR should not be configured together.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	Using Rel19 DSR together with Rel18 DSR will create large problems of ambiguity to the network scheduler.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Network configures the number of thresholds and their values for each LCG.

Proposal 2	For Rel19 DSR reuse the structure of the Rel18 DSR but change the R-bit to a n-bit to indicate if a further threshold is present for the current LCG.

Proposal 3	Adopt the DSR format shown in Figure 1.

Proposal 4	The DSR provides the information of the size that needs to be scheduled to accommodate the delay critical data.

Proposal 5	The threshold with longest remaining time can be used as the trigger threshold.

Proposal 6	Rel19 DSR and Rel18 DSR should not be configured together.

# References

RP-241771 Revised WID: XR (eXtended Reality) for NR Phase 3, September 2024