3GPP TSG RAN WG2 Meeting #128	R2-2410515

Orlando, USA, 18 - 22 November 2024

Agenda Item:	7.2.3

Source:	Huawei, HiSilicon

Title:	Further MAC corrections

Document for: Discussion and Decision

# 1	Introduction

At RAN2#127bis, CR 1968r1 to TS 38.321 Rel-18 was agreed in principle. In this document, we consider additional corrections that can be considered.

# 2	Discussion

Section 5.8.2 describes the use of CG for RACH-less LTM cell switch as follows:

For an uplink grant configured for configured grant Type 1 for RACH-less LTM cell switch, when there is an ongoing RACH-less LTM cell switch procedure, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if an SSB corresponding to the configured UL grant has the same SSB index as the SSB associated with the TCI state indicated by the UL TCI state ID field, if present, or by the TCI state ID field otherwise, in the LTM Cell Switch Command MAC CE, as specified in clause 5.18.35:

2>	select the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE.

2>	indicate the SSB index to the lower layer;

2>	consider this configured uplink grant as valid.

1>	else:

2>	consider this configured uplink grant as not valid.

NOTE 1a:	When there is an ongoing RACH-less LTM cell switch, the configured grant Type 1 which is not specifically configured for LTM (see cg-LTM-Configuration in TS 38.331 [5]) is not used.

NOTE 1b:	After completion of LTM cell switch, the UE stops using the grant configured for RACH-less LTM cell switch (see cg-LTM-Configuration in TS 38.331 [5]).

According to the green highlighted text:

-	the UE shall check whether each grant from the CG is corresponding to the SSB associated with the TCI state indicated in the MAC CE;

-	if it is the case, the grant is valid, otherwise it is not valid.

This raises the question of the usefulness and effect of the yellow highlighted text, that comes in addition. 5.18.35 includes the following:

3>	indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

Accordingly, TCI state information is already passed to lower layers when the LTM cell switch MAC CE is indicated (in the recovery case, there is no MAC CE but RA is used, so the above text is not applicable).

In addition, the QCL RS in the indicated TCI state information may not be SSB, and there is no reason to ask the UE to "select an SSB" or indicate it to lower layers. So we understand that the yellow highlighted text is not applicable for LTM and should be removed.

For an uplink grant configured for configured grant Type 1 for RACH-less LTM cell switch, when there is an ongoing RACH-less LTM cell switch procedure, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if an SSB corresponding to the configured UL grant has the same SSB index as the SSB associated with the TCI state indicated by the UL TCI state ID field, if present, or by the TCI state ID field otherwise, in the LTM Cell Switch Command MAC CE, as specified in clause 5.18.35:

2>	consider this configured uplink grant as valid.

1>	else:

2>	consider this configured uplink grant as not valid.

NOTE 1a:	When there is an ongoing RACH-less LTM cell switch, the configured grant Type 1 which is not specifically configured for LTM (see cg-LTM-Configuration in TS 38.331 [5]) is not used.

NOTE 1b:	After completion of LTM cell switch, the UE stops using the grant configured for RACH-less LTM cell switch (see cg-LTM-Configuration in TS 38.331 [5]).

Proposal 1: Remove the text in 5.8.2 to select an SSB and indicate it to lower layers (highlighted in yellow before).

RAN1 has added the following description in TS 38.213 v18.4.0 clause 21.1:

# 21	L1/L2-triggered mobility procedures

[...]

## 21.1	Configured-grant PUSCH transmission in RACH-less LTM cell switch

[...]

If the UE is provided cg-LTM-Configuration, the UE performs configured grant Type 1 PUSCH transmission on the valid PUSCH occasions associated with the SS/PBCH block indexes same as the SS/PBCH block indexes provided by or associated with QCL RS of the CandidateTCI-State or CandidateTCI-UL-State indicated by the LTM Cell Switch Command MAC CE.

The text in clause 5.8.2 of TS 38.321 is similar but a little different:

For an uplink grant configured for configured grant Type 1 for RACH-less LTM cell switch, when there is an ongoing RACH-less LTM cell switch procedure, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if an SSB corresponding to the configured UL grant has the same SSB index as the SSB associated with the TCI state indicated by the UL TCI state ID field, if present, or by the TCI state ID field otherwise, in the LTM Cell Switch Command MAC CE, as specified in clause 5.18.35:

2>	select the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE.

2>	indicate the SSB index to the lower layer;

2>	consider this configured uplink grant as valid.

1>	else:

2>	consider this configured uplink grant as not valid.

NOTE 1a:	When there is an ongoing RACH-less LTM cell switch, the configured grant Type 1 which is not specifically configured for LTM (see cg-LTM-Configuration in TS 38.331 [5]) is not used.

NOTE 1b:	After completion of LTM cell switch, the UE stops using the grant configured for RACH-less LTM cell switch (see cg-LTM-Configuration in TS 38.331 [5]).

CandidateTCI-UL-State is defined as:

CandidateTCI-UL-State-r18 ::=           SEQUENCE {

tci-UL-StateId-r18                      TCI-UL-StateId-r17,

referenceSignal-r18                     CHOICE {

ssb-Index                               SSB-Index,

csi-RS-Index                            NZP-CSI-RS-ResourceId

},

pathlossReferenceRS-Id-r18              PathlossReferenceRS-Id-r17                                   OPTIONAL,   -- Need R

tag-Id-ptr-r18                          ENUMERATED {n0,n1}                                           OPTIONAL,   -- Cond 2TA

ul-PowerControl-r18                     Uplink-powerControlId-r17                                    OPTIONAL,   -- Need R

...

}

In our understanding, "provided by" refers to the case where the referenceSignal is an ssb-Index, while "associated with" refers to the case where it is a csi-RS-Index, in which case the UE needs to check the corresponding NZP-CSI-RS-Resource, i.e.:

NZP-CSI-RS-Resource ::=             SEQUENCE {

nzp-CSI-RS-ResourceId               NZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

powerControlOffset                  INTEGER (-8..15),

powerControlOffsetSS                ENUMERATED{db-3, db0, db3, db6}                 OPTIONAL,   -- Need R

scramblingID                        ScramblingId,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                OPTIONAL,   -- Cond PeriodicOrSemiPersistent

qcl-InfoPeriodicCSI-RS              TCI-StateId                                     OPTIONAL,   -- Cond Periodic

...,

[[

subcarrierSpacing-r18               SubcarrierSpacing                               OPTIONAL,   -- Cond LTM

absoluteFrequencyPointA-r18         ARFCN-ValueNR                                   OPTIONAL,   -- Cond LTM

cyclicPrefix-r18                    ENUMERATED {extended}                           OPTIONAL    -- Cond LTM

]]

}

This indicates a further candidate TCI state in qcl-InfoPeriodicCSI-RS. If the reference-Signal is an ssb-Index, the "associated SSB" is this ssb-Index. Otherwise, the UE needs to further look in the TCI state indicated by qcl-InfoPeriodicCSI-RS, and so on, until the UE finds an ssb-Index.

Effectively, the statements in TS 38.321 clause 5.8.2 is redundant with the statement in TS 38.213 clause 21.1, and to make specification maintenance easier, it could be good to remove it. However, it is important to indicate in TS 38.321 whether a grant is considered as valid for procedures in TS 38.321, which is difficult to capture in TS 38.213.

Therefore, we suggest trying to align the wording of TS 38.321 clause 5.8.2 with TS 38.213 clause 21.1, e.g.

For an uplink grant configured for configured grant Type 1 for RACH-less LTM cell switch, when there is an ongoing RACH-less LTM cell switch procedure, for each configured uplink grant valid according to TS 38.214 [7] for which the above formula is satisfied, the MAC entity shall:

1>	if an SSB corresponding to the configured UL grant has the same SSB index as the SSB provided by or associated with the TCI state indicated by the UL TCI state ID field, if present, or by the TCI state ID field otherwise, in the LTM Cell Switch Command MAC CE, as specified in clause 21.1 in TS 38.213 [6]:

2>	select the SSB associated with the TCI state indicated by LTM Cell Switch Command MAC CE.

1>	else:

2>	consider this configured uplink grant as not valid.

NOTE 1a:	When there is an ongoing RACH-less LTM cell switch, the configured grant Type 1 which is not specifically configured for LTM (see cg-LTM-Configuration in TS 38.331 [5]) is not used.

NOTE 1b:	After completion of LTM cell switch, the UE stops using the grant configured for RACH-less LTM cell switch (see cg-LTM-Configuration in TS 38.331 [5]).

Proposal 2: Align the wording on valid grant with the wording in TS 38.213 clause 21.1 (add "provided by or" before "associated with") and add a reference to TS 38.213 clause 21.1.

# 3	Conclusion

We made the following proposals:

Proposal 1: In TS 38.321, remove the text in 5.8.2 to select an SSB and indicate it to lower layers (highlighted in yellow before).

Proposal 2: In TS 38.321 clause 5.8.2, align the wording on valid grant with the wording in TS 38.213 clause 21.1 (add "provided by or" before "associated with") and add a reference to TS 38.213 clause 21.1.