3GPP TSG-RAN WG2 Meeting #128	R2-2410518

Orlando, USA, 18 - 22 November 2024

Agenda Item:	8.6.2

Source:	Huawei, HiSilicon

Title:	Inter-CU LTM

Document for: Discussion and Decision

# 1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18 for the intra-CU scenario. In Rel-19 [1], the scope is extended to inter-CU to support low latency and low interruption cell switch in broader areas. In this contribution, we discuss inter-CU LTM in standalone and NR-DC architectures.

# 2	Discussion

## 2.1	Inter-CU LTM in standalone

### 2.1.1	Security update

In subsequent inter-CU LTM, how to update the security key is one issue to be considered when UE switches among candidate cells. In RAN2 #125bis meeting, RAN2 provided four options and sent an LS [2] to SA3 to request SA3 to analyse those options.

Option 1: Use a new information in MAC CE to deliver the security information. Whether the UE uses horizontal or vertical key derivation is derived from this new information in MAC CE (which is currently, neither integrity protected nor ciphered). Option 1A: the NCC value to be used at inter-CU LTM execution is included in the LTM cell switch command MAC CE. Option 1B: the UE is preconfigured with a list of NCC values in a ciphered and integrity protected RRC message and the index of an NCC value in the list is included in the LTM cell switch command MAC CE. Option 2: Similar to Rel-18 S-CPAC key update mechanism, the UE is preconfigured from the source gNB with a list of NCC values per CU using RRC signalling (that is both integrity protected and ciphered). It is expected that the participating gNBs (CUs) would need to be aware of the list and how the UE applies the list during LTM cell switches: Option 2A: UE chooses the first unused NCC for the target CU upon inter-CU LTM execution. Option 2B: As an alternative to choosing the next unused NCC (as in option 2A), horizontal key derivation is used in this option if the LTM cell switch is between the same two CUs. Option 3: After the execution of inter-CU LTM cell switch, the participating gNBs are expected to be updated with new K-gNB* to be used for the next inter-CU LTM cell switch. The UE and CN are aware of how the UE would use the next NCC value. Option 3A: The UE determines the following NCC value to use by itself (e.g., increase by 1) after subsequent inter-CU LTM execution. Option 3B: UE is pre-configured by the CN (via source gNB RRC signalling) with a list of NCC values and the UE chooses the first unused NCC value as the next NCC value. Option 4: After every inter-CU LTM cell switch execution, the UE is provided via RRC signalling with the NCC value to be used by the UE for key derivation at the next inter-CU LTM cell switch.

After several SA3 meeting, a reply LS [3] was sent to RAN2:

SA WG3 thanks RAN WG2 for the LS on the security handling for inter-CU LTM in non-DC cases. SA WG3 has been considering the security mechanisms proposed for the inter-CU LTM feature. For this feature, SA WG3 want to preserve the principle that when there is a PDCP anchor change, then it is necessary to perform a key change (i.e. for solutions or handovers that do not require a PDCP anchor change, there is no need to provide rekeying). The rekeying method preferred by most companies in SA WG3 is that after every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used by the UE for key derivation at the next inter-CU LTM cell switch. This solution was preferred as it provides the NCC value using the protected RRC signalling and hence is considered to be secure. SA WG3 understands that, RAN WG2 is aiming to avoid RRC configuration/signalling between cell switches for subsequent LTM procedures. If it is necessary for the RAN WGs to avoid RRC signalling between cell switches, then SA WG3 can analyse further methods like, - Security threat of the unprotected MAC CE to deliver the NCC value. - Sending NCC value per candidate cell via RRC message during LTM preparation phase. SA WG3 will keep RAN WG2 and RAN WG3 informed as we make further progress on this topic.

According to the reply LS, most companies prefer Option 4: “After every inter-CU LTM cell switch execution, the UE is provided via RRC signalling with the NCC value to be used by the UE for key derivation at the next inter-CU LTM cell switch.” However, SA3 is not sure about whether RRC signalling is allowed in subsequent cell switch procedure, so RAN2 needs to discuss whether RRC signalling can still be transmitted. In SA3’s decision, if RRC signalling is not allowed, they will further analyse the method that the NCC value is delivered via MAC CE, which may have security threat.

Observation 1: SA3 prefers the solution which the network provides NCC value to the UE via RRC signalling after every inter-CU LTM cell switch. RAN2 needs to further discuss whether RRC signalling can be transmitted in subsequent LTM procedure.

The Rel-19 WID includes the following sentence: “Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM”. Some companies argued that no RRCReconfiguration message should be transmitted between subsequent cell switch. However, the intention of the sentence “aiming to avoid RRC configuration” is to avoid re-preparing candidate cell configurations after each cell switch involving heavy network-side signalling, and to avoid re-sending all candidate configurations to the UE after each cell switch, so avoid heavy RRC signalling, but there could be reconfigurations for other purposes (e.g., add CG resources for a new service, change the configuration for UE assistance information, etc.).

Observation 2: The expected benefits of subsequent LTM are to:
1) avoid re-preparing all candidate cell configurations after each cell switch, so avoid heavy network-side signalling 
2) avoid re-sending all candidate configurations to the UE after each cell switch, so avoid heavy RRC signalling.

In the RRC solution (option 4), when the UE switches from gNB1 to gNB2, upon completion of path switch, the new serving gNB (gNB2) sends to the UE an RRC message that includes the updated NCC value received from the AMF during the path switch procedure. The UE will use the NCC value for the next inter-CU LTM cell switch execution, e.g., to gNB3.

Observation 3: Sending to the UE an RRC message only with the NCC for the next inter-CU LTM cell switch does not change anything to the previously stored LTM candidate configurations, so subsequent LTM is supported with all its expected benefits.

In the MAC CE solution (option 1), the new serving gNB (gNB2) includes the NCC value in the next LTM cell switch command MAC CE indicating a candidate configuration whose PCell is controlled by another gNB e.g.,  gNB3. In this solution, there is also no need for the network to re-prepare candidate cell configurations or to re-send candidate configurations to the UE, so subsequent LTM is also supported with all its benefits.

Observation 4: Subsequent LTM can be supported with all its benefits with the RRC solution (option 4) or the MAC CE solution (option 1).

For both solutions, gNB2 needs to send an updated key to gNB3 so that the keys used by the UE and by gNB3 are aligned at the next inter-CU LTM cell switch. The keys could be delivered via Xn to all candidate gNBs before LTM cell switch is triggered or only to gNB3 when gNB2 has decided to trigger LTM cell switch to gNB3.

Proposal 1:	Support sending the keys for inter-CU LTM cell switch from the serving gNB to all candidate/ta gNBs before or when inter-CU LTM cell switch is triggered.

The NCC is an 8-bit value that can be included in an RRCReconfiguration message as an extension to LTM-Config-r18. If no other information needs to be transmitted to the UE in the same message, the message size is likely to be less than 10 bytes, while the message is transmitted while the radio quality should be very good because the path switch is executed immediately after handover. Therefore, the overhead is small, the UE can send the complete message quickly and then the UE is ready for any subsequent inter- or intra-CU LTM cell switch execution.

The LTM cell switch MAC CE is transmitted when handover is triggered to another cell, so presumably when the current serving cell quality is already degraded, so that even if the MAC CE is smaller than the RRC message, it may still use more resources and require HARQ retransmissions. From that perspective, there is no clear advantage to include the NCC in the LTM cell switch MAC CE as compared with sending it via RRC immediately after path switch.

According to the feedback from SA3, the MAC CE solution has more SA3 impacts, and SA3 needs to further discuss the security threat of the MAC CE solution. Therefore, we suggest adopting the RRC solution.

Observation 4: The MAC CE solution (option 1) does not have clear benefits compared with the RRC solution (option 4), and needs more discussion in SA3 about possible security issues.

Proposal 2:	Upon path switch completion, the new serving gNB sends the updated NCC value by RRC message to the UE for the next inter-CU LTM cell switch.

The procedure is shown in the figure below:

Figure 1. An example of the RRC solution for key update in subsequent LTM

### 2.1.2	Fast recovery

In RAN2 #127bis meeting, RAN2 FFS whether to support fast recovery when the selected cell by the UE is an inter-CU candidate cell. One concern is whether the key update method is applicable to the fast recovery case when the selected cell is an inter-CU candidate.

RAN2 #127bis 1. The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

According to the discussion above, before receiving an LTM cell switch MAC CE (e.g., from gNB1 to gNB2), the UE should already have an NCC value for the next inter-CU LTM cell switch. If an LTM cell switch fails and the UE selects an inter-CU candidate cell (e.g., in gNB3) for fast recovery, the UE can use this NCC to derive a new key. At the network side, there are two possible methods. One method is that the gNB1 sends new keys to all candidate gNBs when path switch is complete, including gNB2 and gNB3, so when the UE accesses a cell controlled by gNB3, gNB3 has the corresponding key. Another method is that gNB3 retrieves the new key from gNB1 upon receiving the msg3 from the UE, if gNB3 has not received the key from gNB1 already. By any of the methods, the UE can perform fast recovery to an inter-CU LTM candidate cell.

Proposal 3a:	Support recovery using an inter-CU LTM candidate cell.

Proposal 3b: The gNB controlling the by LTM candidate cell used for fast recovery retrieves the key from the source gNB if it has not received a new key.

## 2.2	Inter-CU LTM in NR-DC

### 2.2.1	Inter-CU MCG LTM

RAN2 agreed that the legacy Rel-15 RRC reconfiguration with sync procedure is applied. However, for subsequent inter-CU MCG LTM with SCG unchanged/modified, the legacy procedure for SN key delivery from the target MN to the SN cannot be re-used directly.

According to TS 37.340 “10.7 Inter-Master Node handover with/without Secondary Node change” section, the SN gets an updated SN key in the SN Addition Request message, as shown in the figure  below:

Figure 2. Part of MN initiated inter-MN handover with/without SN change procedure

Observation 5: In the legacy inter-Master Node handover without Secondary Node change, the SN receives the updated SN key in the SN Addition Request message.

However, for inter-CU MCG LTM with SN unchanged/modified, the SN Addition Request procedure is performed only in the preparation phase, and not performed again in the following subsequent MCG LTM procedure. In the preparation phase, the MN keys of candidate target MNs for subsequent inter-CU MCG LTM are not determined yet so it is not possible to use the SN Addition Request to deliver SN keys.

Observation 6: In inter-CU MCG LTM with SN unchanged scenario, it is not possible to use the SN Addition Request to deliver SN keys for subsequent inter-CU MCG LTM.

Assuming the RRC solution (option 4) is adopted for the standalone case, once the target MN gets a new MN key from the source MN, it can derive a new SN key and send the SN key to the SN. The procedure is shown in the figure below:

Figure 3. An example of SN key update in inter-CU MCG LTM with SCG unchanged/modified

Proposal 4:	In inter-CU MCG LTM with SN unchanged, the target MN sends the updated SN key to the SN after it receives a new MN key from the source MN.

### 2.2.2	Inter-CU SCG LTM

In SN-initiated inter-CU SCG LTM, the RRC configuration message is transmitted from the MN to the UE, which includes candidate SCG configurations. When SCG LTM is triggered, the UE applies the target SCG configuration and sends an RRCReconfigurationComplete message to the MN. Then, the MN transfers the SN RRCReconfigurationComplete message to the SN. In this case, if the SCG LTM cell switch is RACH-less, the UE may not have uplink data or signalling to transmit to the SCG. If the network configures UL grant skipping, the SCG may not receive any UE uplink transmission and may consider RACH-less cell switch to be failed.  To avoid this problem, the network should ensure that the UE does not skip the UL grant in RACH-less SCG LTM procedure, e.g., not configure enhancedSkipUplinkTxDynamic, enhancedSkipUplinkTxConfigured and skipUplinkTxDynamic.

Proposal 5: For inter-CU SCG LTM, the network ensures that the UE does not skip the UL grant for RACH-less cell switch.

In Rel-18, intra-CU SCG LTM cell switch does not affect the MN i.e., MCG operation is not affected. Since PDCP data recovery cannot be triggered for MN-terminated bearers and this would be required for MN-terminated bearers with an SCG RLC bearer, all MN-terminated bearers should be MCG bearers (otherwise, some data will be lost).

Observation 7: Rel-18 SCG LTM cell switch does not affect MCG operation at all and has common procedures with MCG LTM, but data can be lost for MN-terminated split and SCG bearers if any (because PDCP data recovery cannot be triggered on PDCP entities in the MN).

For inter-CU SCG LTM, the MN needs to be involved at least in the LTM preparation, as different SNs cannot communicate directly, and also for LTM cell switch notification, if the Xn signalling exchange is necessary at LTM cell switch. PDCP data recovery for MN terminated SCG/split bearers should be supported.

Proposal 6: For inter-CU SCG LTM, support to trigger PDCP data recovery in PDCP entities at the MN for MN terminated SCG/split bearers.

### 2.2.3	Coexistence of intra-MN/inter-MN MCG LTM and inter-SN/intra-SN SCG LTM

In previous RAN2 meetings, RAN2 had the following agreements related to LTM in NR-DC scenario:

RAN2 #126 Clarification of DC - An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells. RAN2 #127bis 11. RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen: - Inter-MN LTM and intra-SN LTM - Inter-SN LTM and intra-MN LTM

The co-existence of different MCG and SCG LTM configurations is summarized in the table below according to the WID:

|  | Intra-MN LTM | Inter-MN LTM | Intra-SN LTM | Inter-SN LTM |
| --- | --- | --- | --- | --- |
| Intra-MN LTM | / | / | / | / |
| Inter-MN LTM | Supported in Rel-19 | / | / | / |
| Intra-SN LTM | Supported in Rel-18 | Supported in Rel-19 | / | / |
| Inter-SN LTM | Supported in Rel-19 | Not supported | Supported in Rel-19 | / |

In order to make sure that the inter-CU LTM is not configured at the same time in MN and SN, inter-node interaction is needed. When the MN decides to configure inter-MN LTM candidates, the MN should send a notification to the SN to disable inter-SN LTM. When the SN decides to configure inter-SN LTM candidates, the SN should first send a request to the MN, and the MN can accept or reject the request.

Proposal 7a: When the MN configures inter-CU MCG LTM candidates, the MN sends a notification to the SN to disable inter-CU SCG LTM.

Proposal 7b: When the SN decides to configure inter-CU SCG LTM candidates, the SN sends a request to the MN, which may be accepted or rejected

In Rel-18, intra-CU SCG LTM is configured by the SN without MN involvement (i.e., candidate configurations are included in an SN message sent to the UE via SRB3 or forwarded by the MN via SRB1). If inter-CU MCG LTM is also configured, the target SCG configuration may be changed so that the intra-CU SCG LTM configuration may need to be changed or released. Since the target SCG configuration of inter-CU MCG LTM is prepared by the SN, the SN can include any modification or release of intra-CU SCG LTM configurations.

Proposal 8: In inter-CU MCG LTM with SN kept, the SCG part of the candidate configuration can include the addition, modification or release of intra-CU SCG LTM configurations (without MN involvement).

RAN2 agreed that subsequent CPAC is not simultaneously configured by the MN and by the SN. Subsequent CPAC is similar to SCG LTM, except that it is executed when the execution condition is met while SCG LTM is executed upon receiving an LTM cell switch MAC CE from the SCG.

Proposal 9: Discuss whether the UE can be simultaneously configured with candidate SCG LTM configurations in an MN message and SCG LTM configurations in an SN message. If this is not supported, when inter-CU SCG LTM is configured, intra-CU SCG LTM should also be configured in an MN message (this is not supported in Rel-18).

# 3	Conclusion

This contribution makes the following proposals:

Security

Proposal 1:	Support sending the keys for inter-CU LTM cell switch from the serving gNB to all candidate/ta gNBs before or when inter-CU LTM cell switch is triggered.

Proposal 2:	Upon path switch completion, the new serving gNB sends the updated NCC value by RRC message to the UE for the next inter-CU LTM cell switch.

Recovery

Proposal 3a:	Support recovery using an inter-CU LTM candidate cell.

Proposal 3b: The gNB controlling the by LTM candidate cell used for fast recovery retrieves the key from the source gNB if it has not received a new key.

MCG LTM in NR-DC

Proposal 4:	In inter-CU MCG LTM with SN unchanged, the target MN sends the updated SN key to the SN after it receives a new MN key from the source MN.

SCG LTM in NR-DC

Proposal 5: For inter-CU SCG LTM, the network ensures that the UE does not skip the UL grant for RACH-less cell switch.

Proposal 6: For inter-CU SCG LTM, support to trigger PDCP data recovery in PDCP entities at the MN for MN terminated SCG/split bearers.

Coexistence of different LTM configurations

Proposal 7a: When the MN configures inter-CU MCG LTM candidates, the MN sends a notification to the SN to disable inter-CU SCG LTM.

Proposal 7b: When the SN decides to configure inter-CU SCG LTM candidates, the SN sends a request to the MN, which may be accepted or rejected

Proposal 8: In inter-CU MCG LTM with SN kept, the SCG part of the candidate configuration can include the addition, modification or release of intra-CU SCG LTM configurations (without MN involvement).

Proposal 9: Discuss whether the UE can be simultaneously configured with candidate SCG LTM configurations in an MN message and SCG LTM configurations in an SN message. If this is not supported, when inter-CU SCG LTM is configured, intra-CU SCG LTM should also be configured in an MN message (this is not supported in Rel-18).

# 4	Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.

- R2-2404037, LS on security handling for inter-CU LTM in non-DC cases, RAN2.

- S3-244316, LS on security handling for inter-CU LTM in non-DC cases, SA3.