3GPP TSG-RAN WG2 Meeting #128	R2-2410519

Orlando, USA, 18 - 22 November 2024

Agenda Item:	8.6.4

Source:	Huawei, HiSilicon

Title:	Intra-CU conditional LTM

Document for: Discussion and Decision

# Introduction

In RAN #105 meeting, the WID of Rel-19 NR mobility enhancements Phase 4 was updated. According to the updated WID [1], conditional LTM (L1/L2 Triggered Mobility) should be specified for intra-CU case in Rel-19, and we should start with non-DC case discussion.

In RAN2 #127bis meeting, RAN2 began the discussion and made some agreements. In this contribution, we discuss the RACH-less procedures of intra-CU conditional LTM, and the event for triggering conditional LTM.

# 2	Discussion

## 2.1	RACH-less procedure

In RAN2 #127bis meeting, RAN2 agreed that RACH-less conditional LTM is supported. To achieve RACH-less cell switch, three aspects need to be considered: early TA acquisition, target cell UL grant acquisition, and beam alignment between the UE and the target cell.

### 2.1.1	Early TA acquisition

To enable RACH-less cell switch, the UE needs to acquire the target cell TA before cell switch. In Rel-18, both PDCCH-ordered early RACH and UE-based TA measurement are supported. In RAN2 #127bis meeting, RAN2 agreed both these two methods are supported for conditional LTM.

RAN2 #127bis: 7. UE based TA measurement mechanism is supported for conditional intra-CU LTM. 8. PDCCH ordered early TA acquisition is supported for conditional LTM.

In the UE-based TA measurement procedure, when the network enables the UE-based TA measurement by RRC, the UE measures the TA by the DL timing gap between the source cell and the target cell, and the exact TA measurement time is up to UE implementation. For conditional LTM, the UE is also able to perform UE-based TA measurement before the execution condition is met, so the UE can use the measured TA to send the first UL message to the target cell when the execution condition is met.

For PDCCH ordered early RACH, the source DU sends the PDCCH order to the UE, and the UE sends a preamble to an indicated candidate cell. The main differences between Rel-18 network triggered LTM and Rel-19 conditional LTM include how the UE acquires the TA, and whether the UE needs to maintain TA validity. In Rel-18 network triggered LTM, the UE gets the TA from LTM cell switch command MAC CE, and the UE does not check the TA validity since the network will ensure the TA is valid if the TA is included in the LTM cell switch command MAC CE. However, in Rel-19 conditional LTM, the UE triggers LTM cell switch by evaluating events, instead of by receiving LTM cell switch command MAC CE. Therefore, RAN2 needs to discuss how does the UE acquire the TA, and whether the UE needs to maintain TA validity.

Observation 1: For PDCCH ordered early RACH, the main differences between Rel-18 network triggered LTM and Rel-19 conditional LTM are how the UE acquires the TA, and whether the UE needs to maintain TA validity.

After the UE has sent a RA preamble to a candidate cell, there are two options to acquire the TA: 1) the candidate cell sends a RAR to the UE, 2) the candidate cell/DU sends the TA to the source cell, and the source cell sends the TA to the UE. In option 1, after the UE has sent a RA preamble, the UE needs to monitor RAR from the candidate cell, which may cause serving cell data transfer interruption/degradation for a period of time, due to receiving beam mismatch. To avoid this issue, we propose adopting option 2.

Proposal 1:	In PDCCH-ordered early RACH for conditional LTM, the UE receives the TA from the current serving cell.

After the UE has received the TA, the UE stores it, and can use a timer to determine its validity. If the TA validity timer is running when conditional LTM cell switch is triggered, the UE performs RACH-less LTM cell switch.

Proposal 2:	In PDCCH-ordered early RACH for conditional LTM, upon receiving the TA, the UE starts a timer and considers the TA as valid as long as the timer is running.

Another issue is when the source cell sends a PDCCH order for early RACH, and for which candidate cell. If the PDCCH order is sent to the UE too early, the acquired TA may be not valid anymore when cell switch is triggered. In addition, if the PDCCH order instructs the UE to send RA preambles to candidate cells that the UE will only access with very low probability, the early TA acquisition procedure may cause service interruption and extra UE power consumption for no benefit. Therefore, reports of L1 measurements results from the UE to the source cell are still needed to allow the source cell to decide to which cell to trigger PDCCH order. L1 measurement reports, either periodic, semi-persistent, aperiodic or event- triggered, or L3 measurement reports can be used for this purpose.

Proposal 3:	The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

### 2.1.2	UL grant acquisition

To achieve RACH-less cell switch, the UE needs a UL grant to send an UL message to the target cell. In Rel-18 network-triggered LTM, both CG-based and DG-based RACH-less cell switch are supported. In RAN2 #127bis meeting, RAN2 agreed to support CG-based RACH-less, but FFS on DG-based RACH-less.

RAN2 #127bis: 10. For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

In CG-based RACH-less, candidate cell needs to provide CG resources to a UE in advance, and the UE may use it at a late stage or even not use it at all. For some candidate cells, the UL resources could be limited, so it is difficult for those cells to allocate CG resources. Therefore, to achieve RACH-less cell switch, it is better to also support DG-based RACH-less cell switch.

Proposal 4:	Support DG-based RACH-less for conditional LTM.

In DG-based RACH-less procedure, the source cell/DU needs to send a request message to the candidate cell/DU, so the candidate cell/DU can begin DG scheduling. As we discussed above, periodic, semi-persistent, aperiodic and event triggered L1 reports can still be used in conditional LTM. In this way, the source cell/DU knows which cell the UE may switch to and which beam the UE may use, so the source cell/DU can send a notification to the candidate cell/DU, with beam information. Then, the candidate cell/DU can try scheduling the UE using the indicated beam. The exact scheduling time can be up to network implementation.

Proposal 5:	The source DU can send a notification message to a candidate DU (via CU) with cell and beam information, to indicate that the candidate DU may send dynamic grants to the UE on the indicated cell using the indicated beam. The exact dynamic grant transmission time is up to the candidate DU implementation.

When the condition for cell switch is met, the UE does not know whether the source cell/DU has already sent a notification to the target cell/DU for dynamic grant transmission. If the source cell/DU did not send any notification and the UE waits for DG from the target cell, the cell switch is likely to fail since the target cell/DU may not be able to successfully receive a SR from the UE due to beam mismatch. Then, after T304 has expired, the UE will initiate RRC re-establishment or fast recovery (if supported). Such a situation may cause a rather long interruption. To avoid this problem, the network should let the UE know whether the above notifications was provided to the target cell/DU or not. If not, the UE can directly initiate RACH to the target cell.

Proposal 6:	The source DU sends a notification to tell the UE the candidate cell(s) that will send DG. When the condition(s) for LTM cell switch to a candidate cell are met and CG is not configured for this candidate cell, the UE performs RACH-less LTM cell switch to that cell if the UE knows that the cell will send DG, otherwise the UE initiates RA.

The above procedure for DG-based RACH-less LTM cell switch is shown in the figure below:

Figure1: one example of DG-based RACH-less LTM cell switch procedure

### 2.1.3	Beam alignment

In the DG-based RACH-less LTM cell switch procedure described in Figure 1, in the “DG transmission request” message from the source cell to the target cell, beam information (e.g., TCI state) is included, so that the target DU knows which beam to use to schedule the UE. For the UE, the “DG transmission notification” message from the source cell to the UE can also include the beam information, and the UE uses the beam when switching to the target cell. By this method, the UE and the target cell can use aligned beams.

In the CG-based RACH-less LTM cell switch procedure, the CG occasions are associated with SSBs. If the target cell uses joint TCI mode, for a particular CG occasion, the target cell knows which UL beam to use to receive the UE UL message and also knows which DL beam to use to schedule the UE afterwards. If the target cell uses separate TCI mode, the network may not know the beam to use for DL transmissions since RAN2 agreed that the CG occasions are associated with UL TCI states when separate TCI mode is used. Therefore, it is also useful that the source cell sends beam information to the target cell ahead of LTM cell switch. In addition, the source cell could send the beam information to the UE, so that the beams used by the UE and by the target cell are aligned.

Observation 2: In CG-based RACH-less LTM cell switch, if joint TCI mode is used, the target cell/DU knows the UL receiving beam and DL transmission beam thanks to the association between CG occasions and SSBs. If separate TCI mode is used, the target cell/DU is not able to know the DL transmission beam that way, since the CG occasions are associated with UL TCI states.

Proposal 7:	In conditional LTM, for both CG-based RACH-less and DG-based RACH-less LTM cell switch, the source cell/DU sends joint TCI State, or DL and UL TCI States to the target cell/DU before conditional LTM triggering. In addition, the source cell/DU sends joint TCI State, or DL and UL TCI State to the UE before conditional LTM triggering.

It is possible that the indicated TCI state from the source cell/DU is not good enough when later conditional LTM cell switch is triggered, so the UE can evaluate the quality, and if the quality is below a threshold, the UE can initiate RA when accessing the target cell.

Proposal 8:	In conditional LTM, when LTM cell switch is triggered, if the target cell is configured for RACH-less LTM cell switch (with CG or DG), the UE has previously received TCI states and the quality of the RS associated with the indicated TCI states is above a threshold, the UE initiates RACH-less LTM cell switch. Otherwise, the UE initiates RA.

## 2.2	Cell switch completion

In Rel-18 network-triggered LTM, the source cell/DU may stop transmissions with the UE after sending the LTM cell switch MAC CE (or receiving the HARQ ACK for the LTM cell switch MAC CE). However, in conditional LTM, there is no LTM cell switch MAC CE and the source cell/DU is not aware that the UE is switching cell, especially in the inter-DU scenario.

After the UE has successfully switched to a target cell by conditional LTM, the target DU should send an Access Success message to the CU, then the CU should notify the source DU to stop transmissions with the UE. Subsequent LTM is supported for conditional LTM, so the source DU should not release the UE context if the source DU still provides candidate cells for the UE for subsequent conditional LTM. The detailed signalling design is up to RAN3.

Proposal 9:	At least for the inter-DU case, the CU notifies the source DU to stop transmissions with the UE after a successful conditional LTM cell switch. The source DU does not release the UE context if the source DU still provides candidate cells for the UE for subsequent conditional LTM. The detailed signalling design is up to RAN3.

## 2.3	Event configuration for conditional LTM

In RAN2 #127bis meeting, RAN2 made the following agreements for the event configuration for conditional LTM:

RAN2 #127bis: 1. Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. 2. Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. 3. Source cell and each candidate cell provides its own execution condition for conditional LTM. 4. It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

The Event LTM3 and LTM5 is:

RAN2 #126 - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. RAN2 #127 Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

In the LTM3 event-triggered L1 report for network-triggered LTM, in beam level evaluation, if one beam is offset better than the current serving beam for a certain time, the event is triggered and the report is transmitted. It is beneficial to send the report to let the network know the current channel conditions. The network can decide whether to trigger the LTM cell switch considering the beam measurement results included in the report i.e., the results of multiple beams. However, in the case of conditional LTM cell switch triggering, if only one beam of candidate cell is offset better than the current serving beam, it may not be reliable to directly trigger cell switch, because no other beam in the candidate cell is good enough so there is a risk of RLF if the UE switches to this cell and this beam becomes no more suitable. In addition, there may several suitable beams in the current serving cell, so that there is no need to trigger cell switch. Cell level evaluation for conditional LTM can make LTM cell switch decisions more reliable as more beams are taken into account.

The existing CondEventA3 and CondEventA5 are already specified for CHO and could be reused for conditional LTM with very low specification impact. In addition, as discussed for Rel-18, some UEs may support LTM but not support L1 measurement reports for LTM. In this case, the network needs to use L3 measurement reports to trigger PDCCH order and LTM cell switch. For conditional LTM in Rel-19, the same problem will exist and some UEs may be able to support CondEventA3 and CondEventA5 but not LTM3 or LTM5. Therefore, it should be possible to use CondEventA3 and CondEventA5 as execution conditions for LTM.

Proposal 10:	Support using CondEventA3 and CondEventA5 as execution conditions of conditional LTM (which can be used e.g., if the UE does not support L1 measurements for LTM).

# 3	Conclusion

This contribution makes the following proposals:

Early TA acquisition

Proposal 1:	In PDCCH-ordered early RACH for conditional LTM, the UE receives the TA from the current serving cell.

Proposal 2:	In PDCCH-ordered early RACH for conditional LTM, upon receiving the TA, the UE starts a timer and considers the TA as valid as long as the timer is running.

Proposal 3:	The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

UL grant acquisition for RACH-less LTM cell switch

Proposal 4:	Support DG-based RACH-less for conditional LTM.

Proposal 5:	The source DU can send a notification message to a candidate DU (via CU) with cell and beam information, to indicate that the candidate DU may send dynamic grants to the UE on the indicated cell using the indicated beam. The exact dynamic grant transmission time is up to the candidate DU implementation.

Proposal 6:	The source DU sends a notification to tell the UE the candidate cell(s) that will send DG. When the condition(s) for LTM cell switch to a candidate cell are met and CG is not configured for this candidate cell, the UE performs RACH-less LTM cell switch to that cell if the UE knows that the cell will send DG, otherwise the UE initiates RA.

Beam alignment

Proposal 7:	In conditional LTM, for both CG-based RACH-less and DG-based RACH-less LTM cell switch, the source cell/DU sends joint TCI State, or DL and UL TCI States to the target cell/DU before conditional LTM triggering. In addition, the source cell/DU sends joint TCI State, or DL and UL TCI State to the UE before conditional LTM triggering.

Proposal 8:	In conditional LTM, when LTM cell switch is triggered, if the target cell is configured for RACH-less LTM cell switch (with CG or DG), the UE has previously received TCI states and the quality of the RS associated with the indicated TCI states is above a threshold, the UE initiates RACH-less LTM cell switch. Otherwise, the UE initiates RA.

Cell switch completion

Proposal 9:	At least for the inter-DU case, the CU notifies the source DU to stop transmissions with the UE after a successful conditional LTM cell switch. The source DU does not release the UE context if the source DU still provides candidate cells for the UE for subsequent conditional LTM. The detailed signalling design is up to RAN3.

Execution conditions

Proposal 10:	Support using CondEventA3 and CondEventA5 as execution conditions of conditional LTM (which can be used e.g., if the UE does not support L1 measurements for LTM).

# 4	Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.