3GPP TSG-RAN WG2 Meeting #128	R2-2410520

Orlando, USA, 18 - 22 November 2024

Agenda Item:	 8.12.2

Source:	 Huawei, HiSilicon

Title:	 Discussion on Asymmetric DL sTRP/UL mTRP

Document for:  Discussion and Decision

# 1	Introduction

For Rel-19 MIMO, according to the WID as shown below [1] and recent RAN1 inputs [5], objectives relevant to UE-initiated/event-driven beam management (UEIBM) and asymmetric DL/UL TRP deployment are identified to have significant RAN2 impacts in terms of RRC configuration and MAC procedure, which are outlined in the WID.

RP-242394, Revised Work Item: NR MIMO Phase 5 The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this contribution, we present detailed analysis and provide our considerations on how RAN2 start work on asymmetric DL sTRP/UL mTRP, based on RAN1 agreements so far.

# 2	Asymmetric DL sTRP/UL mTRP

In Multiple Transmit/Receive Point (multi-TRP) operation, a serving cell can schedule the UE from two TRPs, providing better coverage, reliability and/or data rates for PDSCH, PDCCH, PUSCH, and PUCCH. In pre-Rel-19 mTRP scenarios, each TRP has downlink and may have uplink, as shown in figure 3 below. In Rel-19, a new scenario, asymmetric DL sTRP/UL mTRP is considered, in which one TRP has downlink and uplink while the other TRP only has uplink, as shown in Fig.4.

Figure 1: pre-Rel-19 multi-TRP scenario

Figure2:  Asymmetric DL sTRP/UL mTRP scenario

## 2.1	Pathloss

### 2.1.1	New MAC CE

In pre-Rel-19 mTRP scenarios, each TRP can transmit DL RS and the UE can measure the path loss directly based on each DL RS from the TRP. In Rel-19, a new scenario is considered, in which one TRP does not transmit any DL RS. For the TRP with DL RS transmission, the UE calculates the path loss based on the DL RS. However, for the TRP without DL RS transmission, a new mechanism is needed e.g., the path loss is calculated using the path loss to the TRP with DL transmission and an offset. RAN1 agreed that different path loss offset values can be configured for different TCI states of a TRP (as the TRP deployment is not visible to the UE):

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

Based on the above agreement, RAN2 should introduce in RRC a path loss offset (e.g. absolute value) for each joint and UL TCI state and a new MAC CE with eLCID for updating the path loss offset for TCI states after RRC configuration.

RRC reconfiguration supports delta signalling, in which the UE is required to keep fields that are absent. For example, if a number of TCI-UL-State were previously configured in BWP-UplinkDedicated for some UL BWP, and the network performs a reconfiguration of some DL parameters, or of some other UL BWP, the UE keeps the existing TCI-UL-State configured in this UL BWP.

If the path loss offset is included in TCI-UL-State, after the reconfiguration, does the UE use the path loss offsets previously configured by RRC, or does it use the path loss offsets modified by MAC CEs received until the reconfiguration?

Another scenario is BWP switching; For example, the UE is operating in BWP#1 and the path loss offset values are modified by a MAC CE. Then, the BWP is switched to BWP#2. Does the UE continue using the values used on BWP#1 or does the UE uses the values configured by RRC for BWP#2? Then, if the UE is switched back to BWP#1, does the UE uses the values that were previously used on BWP#1 i.e., as modified by MAC CEs when the UE was previously operating on BWP#1 or does the UE uses the values configured by RRC for BWP#1?

Proposal 1a: RRC configures an absolute PL offset for each TCI state configuration. A new MAC CE with eLCID and PL offset value indication is introduced to update PL offset for TCI states associated.

Proposal 1b: Discuss whether, after an RRC reconfiguration in which the path loss offset values are neither released nor modified by RRC, the UE continues using the values modified by MAC CEs, or reverts to the values configured by RRC.

Proposal 1c: Discuss whether, after BWP switching, the UE reverts to the RRC-configured path loss offset values for the new BWP or uses the last value that were used on this BWP i.e., including modifications by MAC CEs.

### 2.1.2	Offset value indication

On the PL offset value indication in new MAC CE, two options were discussed at last meeting.

- Option 1: absolute value

- Option 2: relative value

As discussed above, one question is, in case of an RRC reconfiguration that neither releases or modifies the previously configured values, whether the UE uses the values previously signalled by RRC or the values updated by the MAC CE. If the UE uses the values previously signalled by RRC, whether the MAC CE is an absolute a relative value makes no difference. If the UE uses the values as updated by the MAC CEs, in case of handover, the source node can provide these updated values as part of the UE AS context, so this also works.

A relative value can be smaller, so it could be encoded as less bits than an absolute value, which could make the MAC CE smaller. Therefore, we suggest using a relative value.

Proposal 2: The PL offset value in new MAC CE is a relative value to the absolute value configured in RRC.

### 2.1.3	TCI state indication

At last meeting, RAN2 discussed whether the PK offset is updated for the indicated TCI state or for the activated TCI state. After some checking on RAN1 agreement, it seems that the MAC CE can update the PL offset values of multiple TCI states:

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

Currently only one TCI state can be the indicated TCI state for a TRP so the agreement implies that the MAC CE update the PL offset value for the activated TCI states.

Proposal 3: The MAC CE includes PL offset values for the activated TCI state(s).

### 2.1.4	TCI mode

Based on the RAN1 agreement, both joint mode and separate TCI mode are supported for PL offset update. In our view, a single MAC CE would be sufficient for both modes e.g. in separate TCI mode, only UL TCI state ID is included without DL TCI state ID and in joint TCI mode, the TCI state ID is included. Also based on the agreement, they seem to agree with same MAC CE format.

Proposal 4: The same MAC CE format with PL offset indication is used for joint TCI mode and for separate TCI mode, e.g. in separate TCI mode, only UL TCI state ID is included with DL TCI state ID.

### 2.1.6	Other

To finalize the MAC CE format, it would be necessary to send RAN1 LS to ask some questions for details, for instance:

- whether this new MAC CE applies to CA and DC cases;

- which channel and signal this new MAC CE with PL offset update applies to.

Proposal 5: Send an LS to ask RAN1 about MAC CE format details:

- Whether this new MAC CE applies to CA and/or DC case.

- Which channel and signal this new MAC CE with PL offset update applies to.

## 2.2	PHR

### 2.2.1	Triggering

In TS 38.321, PHR triggering is defined as:

A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission; NOTE 1: The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5].

In the legacy mTRP scenario, different TRPs can transmit different DL RSs and the UE can measure the PLs separately based on each DL RS from the mTRP. In asymmetric DL sTRP/UL mTRP scenarios, the PL for one UL TRP is derived based on the DL TRP of the TRP and the PL for the other UL TRP is derived based on the DL TRP and PL offset as mentioned above. Though the PL calculation method of each mTRP has been modified, the PHR is still triggered according to the change of PL. Since the triggering mechanism has not been changed, such PL determination may not impact the PHR triggering.

Proposal 6: Discuss whether any change to PHR triggering is needed for asymmetric DL sTRP/UL mTRP scenarios.

### 3.2.1	Format

According to TS 38.213, PH is calculated by physical layer and indicated to the MAC layer. Then, the MAC layer includes it in the PHR MAC CE when UL resource are available after PHR is triggered. In Rel-19, RAN1 made the following agreement on PH calculation for asymmetric DL sTRP/UL mTRP scenarios:

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH

Based on the above agreement, RAN2 can assume that the physical layer indicates the calculated PH for asymmetric DL sTRP/UL mTRP scenarios to the MAC layer, and the MAC layer includes it in the existing mTRP PHR MAC CE, i.e. no new mTRP PHR MAC CE is introduced.

Proposal 7: RAN2 assumes no new mTRP PHR MAC CE is introduced i.e., the existing mTRP PHR MAC CEs can be reused.

# 4	Conclusion

From an initial analysis on RAN2 impacts for MIMO, we made the following proposals:

Proposal 1a: RRC configures an absolute PL offset for each TCI state configuration. A new MAC CE with eLCID and PL offset value indication is introduced to update PL offset for TCI states associated.

Proposal 1b: Discuss whether, after an RRC reconfiguration in which the path loss offset values are neither released nor modified by RRC, the UE continues using the values modified by MAC CEs, or reverts to the values configured by RRC.

Proposal 1c: Discuss whether, after BWP switching, the UE reverts to the RRC-configured path loss offset values for the new BWP or uses the last value that were used on this BWP i.e., including modifications by MAC CEs.

Proposal 2: The PL offset value in new MAC CE is a relative value to the absolute value configured in RRC.

Proposal 3: The MAC CE includes PL offset values for the activated TCI state(s).

Proposal 4: The same MAC CE format with PL offset indication is used for joint TCI mode and for separate TCI mode, e.g. in separate TCI mode, only UL TCI state ID is included with DL TCI state ID.

Proposal 5: Send an LS to ask RAN1 about MAC CE format details:

- Whether this new MAC CE applies to CA and/or DC case.

- Which channel and signal this new MAC CE with PL offset update applies to.

Proposal 6: Discuss whether any change to PHR triggering is needed for asymmetric DL sTRP/UL mTRP scenarios.

Proposal 7: RAN2 assumes no new mTRP PHR MAC CE is introduced i.e., the existing mTRP PHR MAC CEs can be reused.

# 5	Reference

- RP-242394, WID revision: NR MIMO Phase 5;

- RAN1 #108 chairman notes;

- TS 3GPP 38.331 V18.2.0

- TS 3GPP 38.321 V18.3.0

- R1-2407285 LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5