3GPP TSG-RAN WG2 Meeting #128	R2-2410521

Orlando, USA, 18 - 22 November 2024

Source: 	Huawei, HiSilicon, Turkcell

Title: 	Discussion on MBS broadcast over NTN

Agenda Item:	8.8.4

Document for:	Discussion and decision

# Introduction

One of the objectives for Rel-19 NTN WI [1] is the support of broadcast service over NTN:

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

During RAN2 #127bis meeting, the following agreements in [2] were made:

Agreements:

1.	For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

In this contribution, we provide our views on the remaining issues.

# Discussion

RAN2 narrowed down the methods for providing the intended service area and considered the following possibilities: SIB20, SIB21, or the MBSBroadcastConfiguration message. Each possibility has different implications, since SIB20, SIB21 and MBSBroadcastConfiguration take effect at different stages of the attempt to receive MBS service

- SIB20 is used to convey the MCCH/MTCH configuration information (the frequency resource for MCCH and MTCH, the time resource for MCCH). If the serving cell sends SIB20, the UE will receive the MCCH of this cell.

- SIB21 provides information related to service continuity for MBS reception, i.e., the mapping of MBS FSAI and the current/ neighbour frequencies. If the MBS capable UE is receiving or interested in receiving an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider the MBS frequency to be the highest priority during the cell selection procedure.

- the MBSBroadcastConfiguration message (provided via MCCH) is used to convey the control information for MBS (including the MRB configuration, PDSCH configuration for MTCH per MBS session, and neighbour cells which are also broadcasting this MBS session). The UE will apply the corresponding configuration of an MBS session, e.g., the scheduling information and the MRB configuration per MBS session. Besides, the UE also determines whether the MBS session of interest is provided in any neighbour cell or not.

Including the intended service area in SIB20 and in the MBSBroadcastConfiguration message can be used to direct the UE in the current serving cell to further acquire and apply the necessary parameter for the MBS session only if the UE is located in the intended service area. However, there are differences between these two methods.

If SIB20 provides the intended service area information, within which the cell provides MBS broadcast session (the detailed MBS session ID information is not in SIB20), the UE can determine not to monitor MCCH if it is not located in any of the service areas. This is also in line with the agreement “RAN2 understands that the expected UE behaviour is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH.” Some concerns were expressed during the previous meeting on whether SIB20 can accommodate the service areas (since the reference locations and radiuses would take a considerable number of bits if the number of service area is large), but we think that only the service area IDs needs to be provided in SIB20.

Proposal 1: Provide the intended service area ID(s) via SIB20 for the power saving purpose.

The last RAN2 meeting agreed to associate each MBS session with one or more intended service area IDs into MCCH. We notice that, the MCCH, i.e. MBSBroadcastConfiguration, provides the support status of MBS sessions for the serving cell and for neighbour cells. In addition to the MBS session ID list of the serving cell, it also indicates the MBS session ID list provided by the neighbour cells. Therefore, if MBSBroadcastConfiguration is responsible for associating an MBS session with an intended area, the MBS session specific intended area of current serving cell and of neighbour cells should be indicated. This allows the UE to request unicast reception of the service of interest after moving to a cell not providing the MBS session in the location area of UE.

Proposal 2:  Both current serving cell and neighbour cells MBS session specific intended area ID(s) should be indicated in the MCCH.

Proposal 3: Based on the neighbour cell MBS session specific intended area, the UE can request unicast reception of the service of interest if moving to a cell not providing the MBS session in UE location area.

SIB21 is used to guarantee the service continuity of MBS session when the UE initiates the cell reselection procedure. The UE could prioritize an MBS frequency on which the MBS session of interest is provided when the UE performs cell reselection. If the intended service area is provided per MBS frequency by SIB21, the UE knows the exact intended service area where a given frequency provides the MBS session of interest. Therefore, the UE can treat an MBS frequency as the highest priority based on its location and on the intended area per MBS frequency during cell reselection.

Proposal 4: The UE can decide to consider an MBS frequency as the highest priority frequency for cell reselection based on its location and on the intended area info/ID(s) provided per MBS frequency via SIB21.

In the previous discussion, we have shown that the intended area information is needed in multiple places for different usages, including SIB20 (for whether to obtain MCCH), MBSBroadcastConfiguration (for whether to monitor MTCH for MBS traffic), and SIB21 (for service continuity).

The next issue is which SIB or message should include the area information, i.e., a set of reference location and radius or by a set of polygon(s). We expect the size of such information to be significant so it should not be repeated in several places. From our point of view, the area information can be defined in a new SIB, together with an area ID for each area. By doing so, in the above mentioned places that require area information, the area ID can be used (which incurs much smaller signalling overhead compared with signalling the whole area definition).

Proposal 5: The intended service area (composed of one or multiple circles or polygons) is provided in a new SIB, along with an area ID for each area. The area ID is used in all places requiring service area info, as mentioned in previous proposals.

# Conclusion

In this document, we discussed the support of broadcast service via NR NTN, and proposed the following:

Proposal 1: Provide the intended service area ID(s) via SIB20 for the power saving purpose.

Proposal 2: Both current serving cell and neighbour cells MBS session specific intended area ID(s) should be indicated in the MCCH.

Proposal 3: Based on the neighbour cells MBS session specific intended area, the UE can request unicast reception of the service of interestif moving to a cell not providing the MBS session in the UE location area.

Proposal 4: The UE can decide to consider an MBS frequency as the highest priority frequency for cell reselection based on its location and on the intended area info/ID(s) provided per MBS frequency via SIB21 .

Proposal 5: The intended service area (composed of one or multiple circles or polygons) is provided in a new SIB, along with an area ID for each area. The area ID is used in all places requiring service area info, as mentioned in previous proposals.

# References

- RP-241789, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

- R2-2409213, Report from Break-Out Session on NR NTN and IoT NTN, Session chair (ZTE)