3GPP TSG-RAN WG2 Meeting #128	R2-2410527

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	7.4.2

Source:	Samsung

Title:	Discussion on some corrections

WID/SID:	NR_NTN_enh-Core

Document for:	Discussion and Decision

# Introduction

In the last meeting, there was some discussion on

- whether inter-gNB RACH-less HO can be supported

- TN to NTN mobility.

In this contribution, we continue to share our views on these issues and propose corresponding specification changes.

# Discussion

## Inter-gNB RACH-less HO in NTN

In the last meeting, inter-gNB RACH-less HO was discussed. Whether inter-gNB RACH-less HO can be supported without RAN3 impact was first discussed in main session for generalized RACH-less HO, see below.

Clarification of support for inter-gNB HO R2-2408233 Correction to RACH-less HO [RACH-lessHO] Huawei, HiSilicon, CATT, Apple, CMCC, Nokia, Nokia Shanghai Bell CR Rel-18 38.300 18.3.0 0905 - F NR_mobile_IAB-Core, NR_NTN_enh-Core, TEI18 - Samsung doesn’t think it should be removed as it applies to all UEs and we agreed that only the intra-gNB case could be supported in this release. There is an alternative to make the change only in NTN. - CATT thinks that simplest is to remove it and it can be up to gNB implementation when this is used. - Ericsson doesn’t agree as it would have RAN3 impacts and even questions if it is supported in NTN. Samsung explains that it was agreed for NTN. - Huawei is concerned that current text doesn’t work for mIAB. Samsung explains that this text is for all UEs. - Apple supports this CR as for mIAB and NTN this is supported. Qualcomm thinks that we can clarify this in NTN. - LG has same understanding as Ericsson. - Huawei thinks perhaps one compromise is to put in brackets (except for mIAB). Need to check if it is supported for NTN The CR is postponed

Then, in NTN session the discussion was continued for NTN inter-gNB RACH-less, see below.

R2-2408013 Correction on RACH-less HO in NR-NTN vivo, THALES CR Rel-18 38.300 18.3.0 0904 - F NR_NTN_enh-Core - Ericsson thinks we shouldn’t add features in maintenance, also this would have RAN3 impacts, e.g. for beam selection - After checking with RAN3 colleagues Nokia thinks that there would be no impact to RAN3 - Samsung thinks this is capturing existing agreements but prefers a different wording. QC agrees - Inmarsat and Thales think that there is no impact in RAN3 and RACH-less HO can be supported in the inter-gNB case by NW implementation Come back in the next meeting

Basically, the intention of majority is to capture in stage-2 that inter-gNB RACH-less HO can be supported at least for NTN in Rel-18. For NTN, we have agreed in early Rel-18 discussion (121bis) inter-gNB RACH-less HO can be supported.

- NTN RACH-less HO is supported for Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB; - NTN RACH-less HO can be supported for intra-satellite handover with different feeder links, i.e., with gateway/gNB switch, inter-satellite handover with gateway/gNB switch, and inter-satellite handover with same gateway/gNB.

The only concern is whether there is RAN3 impact to support inter-gNB RACH-less HO in NTN. We share our view on this issue.

For RACH-less HO, same as normal L3 HO ReconfigurationWithSync included in RRCReconfiguration message provides HO configuration, including information specific for RACH-less:

- N_TAoffset to determine the initial TA of the target cell;

- beam indication for DG-based RACH-less;

- CG configuration for CG-based RACH-less HO, included in configuredGrantConfig -> BWP-UplinkDedicated.

These three are all included in the RRCReconfiguration message for RACH-less HO. As normal L3 HO, the inter-gNB message HandoverCommand from the target gNB to source gNB contains the RRCReconfiguration message to be transmitted to the UE. We don’t think the source gNB and target gNB need additional information exchange to prepare the RRCReconfiguration message. The target gNB can provide the above three pieces of information using the existing mechanism and/or by implementation.

- The target gNB can determine N_TAoffset = source based on whether the target cell is co-located with the source cell;

- the beam indication can be determined by target gNB based on the measurement results included in the inter-gNB message HandoverPreparationInformation, which can include beam-level measurement results of the L3 measurement of the target cell;

- CG resources can be determined by the target gNB directly.

Observation 1: RRCReconfiguration message for RACH-less HO can be prepared by the target gNB using the existing mechanism and/or by implementation. Inter-gNB RACH-less HO in NTN can be supported without RAN3 impact.

As currently only intra-gNB RACH-less HO is mentioned in stage-2 description, we propose to clarify that inter-gNB RACH-less HO can be supported in NTN. And we think it’s better to capture the above RAN2 agreements for NTN specific scenarios, as proposed in CR R2-2409027.

Proposal 1: Add the following sentence in TS 38.300 clause 9.2.3.6 RACH-less handover.

“In NTN, RACH-less handover can be supported for intra/inter-satellite handover with same gateway/gNB and intra/inter-satellite handover with gateway/gNB switch.”

## TN to NTN cell reselection

In the last meeting, there was some discussion on the cell reselection from TN to NTN and relevant UE capability aspects. The following agreement was reached.

RAN2 understands that for now there are no RAN4 requirements regarding measurements on a NTN cell while the UE is camping or being connected to a TN cell and then it’s up to UE implementation whether to perform such measurements or not. RAN2 will not further work on enhancements for TN to NTN mobility in Rel-18.

However, after further checking, in TS 38.133 clause 4.2.2.12, RAN4 has already specified measurement requirements for TN-NTN cell reselection in both directions. A part of the clause is copied below.

#### 4.2.2.12		Measurements of inter-frequency NR cells with NTN carrier

This clause applies for the inter-frequency cell reselection for NTN carriers in FR1 NTN, and TN carriers if configured. The requirements in clause 4.2.2.12 apply provided that network provides SIB19 and UE is configured with one or more NTN carrier. UE is not required to ensure having a valid version of SIB19 and the exact time of reacquiring SIB19 is up to UE implementation.

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,  then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS38.304 [1] within Kcarrier_TN* Tdetect,NR_Inter_TN + i=1Kcarrier_NTNKmulti_SMTC,i*Tdetect,NR_Inter_NTN+T_GNSS if the UE does not support the feature for enhanced RRM requirements defined in TS38.306 [14]  or if the enhancedMeasurementLEO-r17 is not enabled, or within Kcarrier_TN* Tdetect,NR_Inter_TN + i=1Kcarrier_NTNKmulti_SMTC,i*Tdetect,NR_Inter_NTN_enh+T_GNSS if the UE supports the feature for enhanced RRM requirements defined in TS38.306 [14]  and the enhancedMeasurementLEO-r17 is enabled, if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least [5]dB in FR1 for reselections based on ranking or [6]dB in FR1 for SS-RSRP reselections based on absolute priorities or [4]dB in FR1 for SS-RSRQ reselections based on absolute priorities. The parameter Kcarrier_TN is the number of NR TN inter-frequency carriers and the parameter Kcarrier_NTN is the number of NR NTN inter-frequency carriers indicated by the serving cell.

<omitting>

Observation 2: In TS 38.133 clause 4.2.2.12, RAN4 has already specified measurement requirements for TN-NTN cell reselection in both directions.

We propose to correct the agreement made in the last meeting.

Proposal 2: RAN2 understands that there are RAN4 requirements regarding measurements on a NTN cell while the UE is camping or being connected to a TN cell. This reverts the agreement in the last meeting.

# Conclusion

In this contribution we discussed the following proposals.

Observation 1: RRCReconfiguration message for RACH-less HO can be prepared by the target gNB using the existing mechanism and/or by implementation. Inter-gNB RACH-less HO in NTN can be supported without RAN3 impact.

Proposal 1: Add the following sentence in TS 38.300 clause 9.2.3.6 RACH-less handover.

“In NTN, RACH-less handover can be supported for intra/inter-satellite handover with same gateway/gNB and intra/inter-satellite handover with gateway/gNB switch.”

Observation 2: In TS 38.133 clause 4.2.2.12, RAN4 has already specified measurement requirements for TN-NTN cell reselection in both directions.

Proposal 2: RAN2 understands that there are RAN4 requirements regarding measurements on a NTN cell while the UE is camping or being connected to a TN cell. This reverts the agreement in the last meeting.

# Reference

3GPP TS 38.133 V18.7.0, NR, Requirements for support of radio resource management.