3GPP TSG-RAN WG2 Meeting #128								R2-2410538

Orlando, USA, 18th – 22nd November, 2024

Title: 	Discussion on NW-sided data collection for training

Source: 	Huawei, HiSilicon

Agenda item:	8.1.3

Document for:	Discussion/Decision

# Introduction

In the last meeting, the following agreements are achieved for NW-sided data collection [1]:

Agreements

1	Periodic logging is supported for training data collection procedure in R19

2	Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

3	Periodic reporting of logged data is not supported.

4	On-demand reporting of the logged measurements will be specified

5	UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

6	The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

7	Low priority SRB will be used.    FFS new SRB or use of SRB4

8	For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

For UE power state discussion, there was a FFS from RAN2#127 meeting minutes [2].

Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

This paper analyzes the network-sided data collection topic further, with a focus on the FFS points mentioned above.

# Discussion

## Discussion on on-demand request reporting

Regarding the on-demand request, it has been agreed that the UE should send the data available indication to the NW when the collected data is available on the UE side. There is an FFS about the trigger condition for the data available indication.

We think data volume should be considered as the triggering condition for triggering available data indication. This is because data volume is the most important metric for data logging and the NW can use it to control how much data the UE should collect, e.g. depending on how much data the NW needs to receive from the UE to efficiently train a model. Furthermore, it is expected that the UE memory used for storing the collected measurements will be limited, so it is useful for the network to know when the UE’s memory is running out. Based on this the network can retrieve the data from the UE allowing it to collect more measurements, if needed. Thus, we think it is reasonable to introduce a configurable data volume threshold used by the UE to send a data available indication to the NW when the volume of collected data exceeds this threshold. It is noted that with this data volume threshold, the NW could be able to configure the threshold to be equal to the UE max buffer size, and thus it means that when the UE's buffer is full, the UE can send the data availability indication to the NW.

As for how the UE sends the data available condition, we think the UEAssistanceInformation message can be used.

Proposal 1: For the on-demand request data retrieval mechanism, the UE should send the data availability indication to the NW when the volume of the collected data reaches a threshold configured by the network.

Proposal 2: The UEAssistanceInformation message is used for sending data available indication.

A single UE may support data collection for multiple AI use cases, e.g. CSI prediction, beam management, and positioning. Thus, we think it should be possible for the network to configure one data collection configuration per use case. We currently do not see a need to have multiple configurations for the same use case simultaneously at a single UE.

Proposal 3: The NW can configure one data collection configuration per use case, i.e. multiple configurations for the same use case are not supported.

Another FFS is whether dynamic activation/deactivation of data collection should be supported. Since we do not believe multiple configurations for the same use case are needed, then also dynamic activation/deactivation is not required. Even when multiple configurations per use case would be supported, dynamic activation/deactivation is still not needed not needed in our view. Data collection for model training is normally a continuous process and it is unclear what the purpose would be of activating and deactivating it dynamically. If the network no longer requires more data from the UE for a certain data collection configuration, the network may simply reconfigure the UE using RRC signaling.

Proposal 4: Dynamic activation/deactivation of the data collection configuration is not supported (i.e. it is sufficient to rely on RRC signaling).

If multiple data collection configurations are supported, even if the UE has training data for different use cases, the NW may not need all the collected data from the UE at the same time. For example, the gNB may be only interested in BM-related training data collection for fine-tuning the BM model(s), and it may not need training data for other use cases. Thus, we think that the NW can be able to request some data from the UE.

Proposal 5: The NW should be able to request the UE to report the data only for a certain data logging configuration (e.g. corresponding to a specific use case).

## Discussion on the event-based data logging

In the last meeting, a radio condition-based event was agreed to be supported. The main rationale behind was to allow some data pre-filtering, e.g. allow the network to only collect the data when the radio conditions are bad. However, the details of the event were not discussed thus far. As we know, the existing radio condition based events are used for mobility, e.g., event A4 (Neighbour becomes better than threshold). Similarly, for logged MDT there is an event based on L1 RSRP so that the UE collects the data only in bad radio conditions. Such design of the existing MDT mechanism makes a lot of sense as the purpose of MDT is to gather measurements in order to optimize UE mobility in various RRC states. The mobility takes place in the cell edge areas, so obviously such filtering by design is useful as the measurements from the cell center are completely useless for inter-cell mobility purpose. However, the situation is completely different for AI aided beam management. The purpose of AI aided BM is to decrease the measurement effort at the UE and allow the NW to save an overhead related to reference signal transmission. This can equally take place in the cell center and in the cell edge.

Observation 1: AI aided beam management is equally useful in the cell center and at the cell edge, so the data from the whole cell area are useful for the NW, unlike the mobility related measurements specified within MDT thus far.

Secondly, the collected data will be sent to the NW and used for model training. Collecting the data only for bad or only for good radio conditions would allow the network to train the model either only for bad or only for good radio conditions. This again does not make any sense as for the AI aided BM feature to be useful, the trained models need to be at least somewhat general, e.g. applicable to a certain a cell as minimum.

Observation 2: It is unreasonable to assume that a network will train separate models only for good or only for bad radio conditions for the sake of AI aided beam management.

If the NW really wishes to use data from certain radio conditions, this can be easily achieved by filtering the collected radio data as L1-RSRP is one of training data for BM use case. What is more, the data can be filtered in any way an operator/NW sees useful, without the need of having any discussions on the triggering conditions in 3GPP. If we were to address this in 3GPP, a lot of factors would have to be discussed, such as a definition of the triggering condition, data collection event leaving conditions etc. Since this refers to BM use case, these discussions would have to additionally involve RAN1.

Observation 3: If needed, data filtering based on various factors, including radio conditions, can be easily performed by the NW after the data is collected, without having complex discussions in RAN1 and RAN2.

Thus, we think RAN2 should re-consider its decision to support the radio condition event based logging and propose the following.

Proposal 6: Event-triggered data logging based on radio condition is NOT supported.

## Discussion on data reporting for low power state UE

In the RAN2#127 meeting, RAN2 agreed that measurements and logging for data collection purpose can be controlled based on the power state of the UE. Whether the UE stops measurement autonomously and reports to the NW still needs to be discussed. As a baseline principle, the UE always needs to follow the network configuration, so it should not be the case that the UE stops the measurements autonomously. The network needs to fully control the NW-side data collection process, so we think the UE should notify the NW about the low power state. Based on such notification, the NW may further indicate the UE to stop/suspend the measurements and logging.

There may be two options for the NW command. The first is that the network removes the measurement and logging configuration on the UE side through the RRC reconfiguration message. The second one is that the NW indicates the UE to suspend the running data collection procedure and recover it when the UE exits low power state. In case the UE recovers from the low power state quickly, then the NW would not have to configure the data collection configuration again when the UE exits the low power state, so a process can be continued.

Proposal 7: When the UE enters a low-power state, it should notify the NW about this. Based on this notification, the NW may indicate the UE to stop/suspend the data collection procedure via RRC signalling, e.g. the NW could indicate UE either to release the measurement and logging configuration or to suspend the data collection procedure and recover it when UE exits low power state.

Another issue may happen if the NW tries to configure data collection configuration to the UE which is already in a low power state. In this case, the UE will have a preference not to collect the data. Thus, the UE that is already in a low power state should send low power state indication to the NW when it receives data collection configuration.

Proposal 8:  The UE that is in a low power state should send a low power state indication to the NW when it receives the data collection configuration (e.g. using the RRCReconfigurationComplete message).

## Discussion on SRB for data reporting

At RAN2#127bis meeting, it was agreed that low priority SRB will be used and whether to use new SRB or existing SRB4 will be further discussed.

The feature of NR QoE measurement reporting was introduced since Rel-17. RRC messages containing the QoE reports (generated in UE application layer) may be quite large, and then it may lead to frequent RRC message transmission in Uu interface. In LTE Rel-15, a separate SRB4 was introduced, and the network is able to configure SRB4 to a different logical channel group with the lowest priority among SRBs and DRBs. For NR QoE, SRB4 was also introduced for the same purpose.

For SRB4, currently it is mainly for QoE reporting. If SRB4 is also supported for NW-sided data reporting, our analysis is shown as below:

- Pros:  very few specification impacts

- Cons:  if both QoE reporting and NW-sided data reporting are enabled at the same time, there is still competition between the two types of reports, which results in more latency. Furthermore, there are some UE actions specified in relation to QoE based on whether SRB4 is configured or not, so this may cause some misinterpretations, e.g. when the NW wants to use SRB4 for AI, but the UE performs some actions related to QoE.

For new SRB approach, it can follow the principle of SRB4 where the LCH priority of the new SRB is configurable. Our analysis is shown as below:

- Pros:  only NW-sided data is transmitted via the new SRB, so it avoids competition with other reports. It also avoids potential issues with AIML/QoE feature interworking, as mentioned above.

- Cons: some specification impacts

In the future, there may be more use cases for NW-sided data reporting, and the requirements may be different from QoE reporting. In this case, we think new SRB approach may be better and it has better scalability and backwards/forwards compatibility.

Proposal 9: RAN2 to adopt a new SRB for NW-sided data reporting with a configurable priority.

# Conclusion

In this paper, we mainly address the FFSes left from RAN2#127-bis. We have the following observations and proposals:

On-demand request reporting

Proposal 1: For the on-demand request data retrieval mechanism, the UE should send the data availability indication to the NW when the volume of the collected data reaches a threshold configured by the network.

Proposal 2: The UEAssistanceInformation message is used for sending data available indication.

Proposal 3: The NW can configure one data collection configuration per use case, i.e. multiple configurations for the same use case are not supported.

Proposal 4: Dynamic activation/deactivation of the data collection configuration is not supported (i.e. it is sufficient to rely on RRC signaling).

Proposal 5: The NW should be able to request the UE to report the data only for a certain data logging configuration (e.g. corresponding to a specific use case).

Event-based data logging

Observation 1: AI aided beam management is equally useful in the cell center and at the cell edge, so the data from the whole cell area are useful for the NW, unlike the mobility related measurements specified within MDT thus far.

Observation 2: It is unreasonable to assume that a network will train separate models only for good or only for bad radio conditions for the sake of AI aided beam management.

Observation 3: If needed, data filtering based on various factors, including radio conditions, can be easily performed by the NW after the data is collected, without having complex discussions in RAN1 and RAN2.

Proposal 6: Event-triggered data logging based on radio condition is NOT supported.

Data reporting for low power state UE

Proposal 7: When the UE enters a low-power state, it should notify the NW about this. Based on this notification, the NW may indicate the UE to stop/suspend the data collection procedure via RRC signalling, e.g. the NW could indicate UE either to release the measurement and logging configuration or to suspend the data collection procedure and recover it when UE exits low power state.

Proposal 8:  The UE that is in a low power state should send a low power state indication to the NW when it receives the data collection configuration (e.g. using the RRCReconfigurationComplete message).

SRB for data reporting

Proposal 9: RAN2 to adopt a new SRB for NW-sided data reporting with a configurable priority.

# References

- [1] RAN2#127bis meeting minutes

- [2] RAN2#127 meeting minutes