3GPP TSG-RAN WG2 #128	R2-2410545

# Orlando, US, 18th – 22nd November 2024

Agenda Item:	8.6.3

Source:	Ericsson

Title:	Event definition and MAC CE content for L1 event-triggered measurements

Document for:	Discussion

# 1	Introduction

With the agreements from RAN2#127bis around how event based measurements are reported back to the network, there are some questions that need to be settled about the design of the events. This contribution discusses these issues and brings forth a number of proposals in this area as well as some baseline MAC CE designs.

# 2	Discussion

## 2.1	L1 events

The L1 event-triggered measurements objective is ongoing, and the following events have been agreed so far:

| Event | Description |
| --- | --- |
| Event LTM2 | Beam of serving cell becomes worse than absolute threshold; |
| Event LTM3 | Beam of candidate cell becomes amount of offset better than beam of serving cell; |
| Event LTM4 | Beam of candidate cell becomes better than absolute threshold; |
| Event LTM5 | Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. |

Additional use cases that we believe should be considered for conditional LTM are the “Release LTM Candidate Configuration” and “Early Sync deactivation of LTM candidate cells”. Managing the LTM candidate cells is always a difficult task for the network, and the early synchronization procedure are probably the most important feature that we have for LTM.

Managing the LTM candidate cells is always a difficult task for the network, and the early synchronization procedure is probably the most important feature that we have for LTM.

In a scenario where UE capability on the number of activated TCI states is limited (e.g., initial versions of the LTM, UE may support limited number of TCI states like 1 or 2), it may happen that after the TCI state of the LTM candidate cell is pre-activated, the radio conditions may change before the UE receives an LTM cell switch command and, without the measurement report from event LTM6, NW may not know the radio conditions change and NW may indicate the activated TCI which may not be the best beam to use after cell switch. We think LTM6 is quite useful to keep the preactivated TCI states update to best beams in the candidate cells.

We also would like to highlight that number of LTM candidates UE can support is a UE capability. We cannot rule out a possibility that some UEs may support only one LTM candidate cell. In that scenario, it is important for the NW to maintain LTM candidate cell as per the radio conditions change. If the beam of the candidate cell becomes worse than absolute threshold, then NW may release the LTM candidate cell and add a new LTM candidate cell (report upon LTM7 event entering condition will help achieving this).

Therefore, we believe that is important to have events which can help the network to address these use cases in the most efficient way. According to this, we would like to propose:

RAN2 to consider how to release the LTM candidate configuration and the triggering or updating of the early synchronization procedures when specifying L1 events.

The following events are supported:

  - Event LTM6: Beam of candidate cell becomes offset better than pre-activated beam of same candidate cell

  - Event LTM7: Beam of candidate cell becomes worse than absolute threshold

## 2.2	Time to Trigger

Based on the discussion in the last RAN2 meeting, one of the main questions in the area of Time To Trigger is how it should be treated from different perspectives:

- Should the TTT be applied per beam or per cell

- Should the TTT reset if what is referred to as the “current” beam changes, i.e. if UE evaluates another beam than previously (still fulfilling the event conditions).

It seems that these two questions A) and B) should be considered together, since the decision on B) depends on the decision on A). From a network perspective, the events will be used for triggering early synch and cell switch procedure for the case of LTM. For the cell switch, since we are taking a decision on a cell level, it might be enough with TTT per cell, i.e. as long as one beam in the cell fulfils the event condition, the TTT needs not to be reset. If there are beams that varies a lot in the candidate cell this can anyway be handled with beam management methods where we have even better channel estimations.

For the case of triggering presynch, one could argue that this is a beam level decision, and thus TTT per beam would be needed. However, for the case of TTT per cell, a presynch towards a candidate beam could still be fulfilled and updated if beam conditions changed.

TTT per cell seems is sufficient for the purpose of triggering cell switch.

TTT per beam seems natural for the purpose of using the measurements based on starting presynch towards a beam in a cell.

TTT per beam can be used by the network in a similar way as TTT per cell, to achieve cell switch decisions.

If the TTT per beam would be used, probably one of drawback is that it may be possible that network would receive a higher number of measurement reports, as the UE would need to keep one TTT for each beam that fulfill the entering condition. Further, even if the TTT could be tuned to avoid this issue, still would be particularly difficult for the network to decide on a reasonable value to configure.

It would basically mean a less filtered approach at the receiving network side, which is generally good for the network if it still can apply necessary filtering as needed. More measurements have to be weighed towards more accurate reports.

TTT per beam would for most cases result in a greater number of measurement reports.

Given that RAN2 has agreed that UE can select “any beam” of the candidate cell for performing the event evaluation, probably having a TTT per beam may trigger a measurement report for a beam which could not be the best at that point. This could be avoided in case the TTT would be per cell. According to this, we propose:

Networks configure a TTT value for each LTM candidate cell for which a L1 event is configured.

The TTT value is configured by the network within the LTM-CSI-ReportConfig, as part of the event configuration.

A further discussion is when the UE should start, stop, or restart the TTT. For this, it makes sense to follow the following principle for the handling of the TTT:

- The UE starts the TTT for an LTM candidate cell when at least one beam of the candidate cell fulfills the entering condition.

- The UE stops the TTT for an LTM candidate cell when all the beams of the candidate cell fulfill the leaving condition.

- The UE stops the TTT for an LTM candidate cell when none of the beams of the candidate cell fulfill the entering condition.

- The UE restarts the TTT for an LTM candidate cell if the indicated beam of the serving cell changes

Therefore, according to this we propose:

The UE handling the TTT for an LTM candidate cell according to the following principles:

  - The UE starts the TTT for an LTM candidate cell when at least one beam of the candidate cell fulfills the entering condition.

  - The UE stops the TTT for an LTM candidate cell when all the beams of the candidate cell fulfill the leaving condition.

  - The UE stops the TTT for an LTM candidate cell when none of the beams of the candidate cell fulfill the entering condition.

  - The UE restarts the TTT for an LTM candidate cell if the indicated beam of the serving cell changes.

## 2.4	MAC CE design

### 2.4.1 Reporting Content

Since some agreements has been done in RAN2#127bis there is now a general assumption on what to include in the MAC CE report to the network.

13. Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW.

A number N has been agreed that corresponds to the maximum number of SSBs for an LTM candidate configuration which is included in the report. In Rel-18, this is controlled by the network where a maximum of 4 SSBs per LTM candidate cell for a maximum of 4 LTM candidate cells can be included in the L1 report.

For Rel-18 LTM a maximum number of 4 beams for 4 candidates can be reported.

To align with what we have in Rel-18, we therefore propose:

The event-triggered L1 measurement report MAC CE can include up to 4 LTM candidate cells.

The event-triggered L1 measurement report MAC CE can include a maximum of 4 SSBs or CSI-RSs for each LTM candidate cell.

Another aspect to consider is that the MAC CE design should work for the case on when the reference signal that the UE measure is an SSB or a CSI-RS. According to this, since both the SSB ID space and the CSI-RS ID space have size 64 it would make sense to design the MAC CE to be agnostic to the reference signal that the UE measures. Therefore, we propose:

A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

In Rel-18, the L1 report over UCI is basically includes an index, which indicate to the network the i-th position of the lists which are included in the LTM-CSI-SSB-ResourceSet:

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

By reporting this index, which is called SSB resource index (SSBRI), the network knows the value of this index is at the i-th position in the list ltm-CSI-SSB-ResourceList and the i-th position in the field ltm-CandidateIdList. At first, this solution may look like some signalling is saved, as with one index we indicate two positions in two separate lists, but when re-using this same mechanism for the design of a MAC CE, this means that 9 bits shall be always reserved for every reported measurement, as the maximum size of the lists ltm-CSI-SSB-ResourceList and ltm-CandidateIdList accounts to 512 possible combinations.

Using the SSBRI in the event-triggered L1 measurement report MAC CE may not be efficient as a large number of bits are required which spans more than one octet.

As the MAC CE allows for more flexibility with respect to the UCI, it is possible to achieve the same results by including the reference signal ID and the LTM candidate configuration ID of the reported measurement quantity in an explicit way. Because of this, it would be good to use the combination of reference signal ID and LTM candidate configuration ID rather than the SSBRI or CRI. Thus, we propose:

The combination of reference signal ID plus the LTM candidate configuration ID is reported within the event-triggered L1 measurement report MAC CE report.

Since in the last RAN2#128 meeting it was agreed that a L1 measurement report can be triggered when the leaving condition of an event is met, a reasonable information to include in the report would be to tag the candidate cell which actually fulfilled the leaving condition. This is a useful information for the network because it can identify the candidate cell which is not performing good and take necessary actions. Thus, we propose:

If a L1 measurement report is triggered because of the leaving condition, the LTM candidate cell which fulfilled the leaving condition is tagged in the report.

### 2.4.2	Possible MAC CE formats

To have a baseline for discussions and illustrate our points discussed in the previous chapter we have drafted MAC CE designs for two different scenarios, one using SSBRI as beam identifier and one with candidate ID and beam index as a reference.

#### 2.4.2.1	MAC CE Format without using the SSBRI

This first MAC CE format assumes that no SSBRI is used for reporting the reference signal and the LTM candidate configuration identifier. In this case, one octet is used to indicate which of the eight LTM candidate configurations are part in the report (L1 – L8). Further, one octet is used to indicate how many beams that are reported per candidate, for a maximum number of 4 beams per candidate configuration. Here it is assumed that we keep the number of reported LTM candidates to a maximum of four, meaning that maximum 4 candidates with maximum 4 beams each can be reported. This brings the maximum size for this MAC CE to 280 bits, excluding the header. The remaining fields of the MAC CE are as follows:

- Event ID: The event ID which triggered the report

- Li: Corresponds to the i-th LTM candidate ID. If the field is set to 1 it means that the i-th LTM candidate ID is part of the report.

- Ni: Indicated the number of reference signals which are reported of the i-th LTM candidate ID indicated by the field Lj. The i-th LTM candidate ID is considered in the order of appearance (from right to left) of the field Ni.

- RS IDi,j: Corresponds to the j-th reference signal (SSB or CSI-RS) of the i-th LTM candidate ID.

- RSRPi,j: Correspond to the RSRP value measured on the j-th reference signal of the i-th LTM candidate ID.

- F: If the value is set to 1 this means that the RS IDi,j is the measured reference signal which has fulfilled the event criteria.

Figure 1. MAC CE format without using the SSBRI.

#### 2.4.2.2	MAC CE Format using the SSBRI

For this second option, instead of LTM Candidate ID and SSB index we are using SSBRI, which is already used to identify to identify LTM Candidate SSBs as per Rel-18 UCI report format. An SSBRI uniquely identifies both an LTM candidate cell and SSB ID in an LTM CSI resource configuration. SSBRI is encoded with log2(KSSSB) bits (see TS 38.212 Table 6.3.1.1.2-6), where KSSSB is the configured number of SS/PBCH blocks in the corresponding LTM-CSI-Resource configuration. This means that SSBRI can be up to 9 bits to indicate a list index of up to 512 (8 cells with 64 beams each).

One octets are used to indicate whether or not each reported SSBRI/RSRP pair triggered the report or not. Also, for this option four candidates with four beams each is considered, bringing the total size of the MAC CE to a maximum of 264 bits. However, this MAC CE would be difficult to extend (i.e., more octets need to be added) in case more information are needed to be reported (e.g., the cell which met the leaving condition or which of the reported LTM candidate cells is the serving cell). The remaining fields of the MAC CE are as follows:

- Event ID: The event ID which triggered the report

- Triggered quantity: Indicate the RSRI value of the reference signal (SSB or CSI-RS) which triggered the measurement report.

- RSRIi: Corresponds to the i-th SSBRI or CRI index according to received LTM CSI resource configuration.

- RSRPi: Correspond to the measured RSRP value of the i-th SSBRI or CRI index according to received LTM CSI resource configuration.

Figure 2. MAC CE format using the SSBRI

Since RAN2 has still to analysis which type of MAC CE format is more convenient for reporting the L1 event-triggered measurements, our propose would be for RAN2 to take the proposed MAC CE format as baseline for the time being.

RAN2 to consider the MAC CE formats in Figure 1 and Figure 2 as the baseline for the discussions on the event-triggered L1 measurement report MAC CE.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Managing the LTM candidate cells is always a difficult task for the network, and the early synchronization procedure is probably the most important feature that we have for LTM.

Observation 2	TTT per cell seems is sufficient for the purpose of triggering cell switch.

Observation 3	TTT per beam seems natural for the purpose of using the measurements based on starting presynch towards a beam in a cell.

Observation 4	TTT per beam can be used by the network in a similar way as TTT per cell, to achieve cell switch decisions.

Observation 5	TTT per beam would for most cases result in a greater number of measurement reports.

Observation 6	For Rel-18 LTM a maximum number of 4 beams for 4 candidates can be reported.

Observation 7	Using the SSBRI in the event-triggered L1 measurement report MAC CE may not be efficient as a large number of bits are required which spans more than one octet.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to consider how to release the LTM candidate configuration and the triggering or updating of the early synchronization procedures when specifying L1 events.

Proposal 2	The following events are supported:

a.	Event LTM6: Beam of candidate cell becomes offset better than pre-activated beam of same candidate cell

b.	Event LTM7: Beam of candidate cell becomes worse than absolute threshold

Proposal 3	Networks configure a TTT value for each LTM candidate cell for which a L1 event is configured.

Proposal 4	The TTT value is configured by the network within the LTM-CSI-ReportConfig, as part of the event configuration.

Proposal 5	The UE handling the TTT for an LTM candidate cell according to the following principles:

a.	The UE starts the TTT for an LTM candidate cell when at least one beam of the candidate cell fulfills the entering condition.

b.	The UE stops the TTT for an LTM candidate cell when all the beams of the candidate cell fulfill the leaving condition.

c.	The UE stops the TTT for an LTM candidate cell when none of the beams of the candidate cell fulfill the entering condition.

d.	The UE restarts the TTT for an LTM candidate cell if the indicated beam of the serving cell changes.

Proposal 6	The event-triggered L1 measurement report MAC CE can include up to 4 LTM candidate cells.

Proposal 7	The event-triggered L1 measurement report MAC CE can include a maximum of 4 SSBs or CSI-RSs for each LTM candidate cell.

Proposal 8	A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

Proposal 9	The combination of reference signal ID plus the LTM candidate configuration ID is reported within the event-triggered L1 measurement report MAC CE report.

Proposal 10	If a L1 measurement report is triggered because of the leaving condition, the LTM candidate cell which fulfilled the leaving condition is tagged in the report.

Proposal 11	RAN2 to consider the MAC CE formats in Figure 1 and Figure 2 as the baseline for the discussions on the event-triggered L1 measurement report MAC CE.

# 4	References

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #103, Maastricht, Netherlands, March 18-21, 2024.