3GPP RAN WG2 Meeting #128	R2-2410550

Orlando, USA, November 18th – 22nd 2024

Agenda Item:	8.1.2.2

Source:	InterDigital

Title:	LCM for UE-sided model for Beam Management use case

Document for:	Discussion, Decision

# Introduction

A RAN2 objective for AI/ML for Air Interface Work Item [1] is to design a general framework for one-sided AI/ML, including the signaling and protocol aspects of Life Cycle Management (LCM) to enable functionality selection, activation, deactivation, switching and fallback. In RAN2#127 [1] the following procedure for reporting applicable functionalities was agreed:

Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5. FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.

This contribution discusses the above procedure for applicable functionality determination and reporting, specifically remaining details on the applicable functionality reporting procedure and clarifications on previous agreements from RAN2#127bis [4] for UE-side data collection.

# Discussion

## UE-side data collection

Initial discussion regarding network control over initiation and configuration of data collection occurred in RAN2#127bis [4], resulting in the following high-level agreement:

Data collection initiation and configuration for data collection is under network control. FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)

From subsequent offline discussion in [POST127bis][020] [5], additional clarification to the above agreement may be useful. For example, the LS from SA2 mentions the following:

“some companies in SA2 understand that initiating (e.g., triggering), terminating collection of UE-side data and controlling data transfer may require NG-RAN involvement”

The above RAN2 agreement refers to the “configuration” and “initiation” of data collection, not the “transfer” of collected data. That is, the “data collection configuration” refers to configurations (e.g., reference signals, associated ID) necessary to collect training data, not the reporting or transfer of such collected data (which should be further discussed by RAN2). It is proposed that this be further clarified.

Proposal 1:	“Data collection configuration” refers to at least the configuration of reference signals and/or associated ID used for training data collection.

Other comments in [5] note that “network control” is not fully equivalent to “NG-RAN”, and the entity controlling (i.e., providing) the data collection configuration may be use-case dependent (e.g., for the positioning use case, the LMF is involved in PRS configuration used for data collection). However, for at least the beam management use case, the NG-RAN is the entity responsible for providing “data collection configurations” used to collect training data. This may also be further clarified.

Proposal 2:	For BM use case, “Data collection configuration” is provided by NG-RAN.

Regarding the FFS: “FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server)”: As noted by several companies in [5], which entity requests the initiation of data collection may depend on the data collection architecture option. For example, in addition to UE or UE side server, other entities which may request initiation of data collection include the NG-RAN, OAM or CN.

Observation 1:	Which entity requests the initiation of data collection may depend on the data collection architecture option and AIML use case.

Since data collection architecture options are actively under evaluation/discussion in SA2, it is proposed that conclusion on which entity requests the initiation of data collection should be postponed until further clarity is reached on which data collection architecture option(s) will be supported.

Proposal 3:	Postpone discussion on “how the NW determines whether data collection should be initiated” until further progress is made on the different data collection architecture options.

## Applicability reporting

A high-level framework for applicability reporting was agreed in RAN2#127 [1] and further refined in RAN2#127bis [4] based on the following agreements:

UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signalled. FFS on the applicability reporting content. UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality)

Regarding the content of the applicability report, in addition to reporting a functionality as “applicable”, it is FFS whether the network may determine which functionalities are “non-applicable” implicitly (e.g., if a supported functionality is not included in the applicable functionality report) or explicitly (e.g., a UE may explicitly report which functionality(ies) are not applicable).

Having the ability to explicitly indicate an AI/ML functionality as “non-applicable” would be useful, for example, if a previously reported functionality changes from “applicable” to “non-applicable”. In this case the UE may simply report the particular AI/ML functionality which is no longer applicable as “non-applicable”. The alternative (i.e., implicit indication) requires the UE re-send the entire list of applicable functionalities, which may cause significant overhead especially if functionality applicability changes dynamically (e.g., due to changing UE or NW side additional conditions).

Observation 2:	Explicitly reporting an AI/ML functionality as “non-applicable” is useful upon change of applicability (e.g., a previously reported applicable functionality is no longer applicable).

To minimize reporting overhead in the case of applicable functionality change, a UE should therefore be able to explicitly report which functionality(ies) are “non-applicable”.

Proposal 4: 	UE can explicitly report which AI/ML functionality(ies) are “non-applicable”.

A similar logic applies to whether an AI/ML functionality needs to be “active” to be reported as non-applicable. Requiring the UE to re-send the entire list of applicable functionalities to implicitly indicate which non-active functionality is no longer applicable can lead to extensive signalling overhead. If the UE is limited to only reporting an “active” functionality as non-applicable, then the UE may risk having to wait until a non-applicable functionality is activated until reporting it as non-applicable, which is inefficient and unnecessary. To enable a unified solution, it is therefore proposed that an AI/ML functionality does not need to be “active” to be reported as non-applicable.

Proposal 5: 	An AI/ML functionality does not need to be “active” to be reported as non-applicable.

Furthermore, a functionality may be non-applicable for many reasons outside of network control, such as model unavailability, model not applicable/trained for current UE side additional conditions (e.g., model not trained for the current UE speed), etc. However, if a functionality is not applicable due to missing information from the network (e.g., inference configuration), this may be easily corrected (e.g., via a UE request/response procedure, or via proactive reporting etc.). In addition to reporting a function as “applicable” or “non-applicable”, the UE should indicate the reason for non-applicability (e.g., model not available, not trained for current UE conditions, missing information, UE/NW side conditions etc.).

Proposal 6: 	UE indicates the reason an AI/ML functionality is non-applicable.

## Neighbour cell functionality applicability reporting

The currently defined applicability reporting framework is designed to report the applicable functionalities within a serving cell. However, there are advantages if the UE is also able to report the applicable functionalities for neighboring cell(s) as well. For example, this may support network decision on which target cell to hand the UE over to (e.g., the network may wish to prioritize cells in which the UE may continue AI/ML operation) and may also reduce service interruption to AI/ML operation upon mobility.

Observation 3:	Extending applicable functionality reporting to include neighbour/target cell(s) can support continuity of AI/ML operation during mobility.

However, there are several issues which may need resolution before confirming that applicable functionality reporting can be extended to include neighbor/target cells. For example, how the UE receives and validates the NW-side additional conditions for a neighbor/target cell or whether the UAI framework can be used to report the functionality applicability other than the serving cell. Before discussing these issues, it is proposed that RAN2 conclude on the baseline functionality reporting procedure (i.e., until RAN1 response is received).

Proposal 7: 	Postpone applicable functionality reporting for neighbour/target cells until the baseline applicable functionality reporting framework is concluded.

# Conclusion

In this contribution the following observations and proposals are made regarding applicable UE-side data collection and AI/ML functionality determination and reporting:

Observation 1:	Which entity requests the initiation of data collection may depend on the data collection architecture option and AIML use case.

Observation 2:	Explicitly reporting an AI/ML functionality as “non-applicable” is useful upon change of applicability (e.g., a previously reported applicable functionality is no longer applicable).

Observation 3:	Extending applicable functionality reporting to include neighbour/target cell(s) can support continuity of AI/ML operation during mobility.

Proposal 1:	“Data collection configuration” refers to at least the configuration of reference signals and/or associated ID used for training data collection.

Proposal 2:	For BM use case, “Data collection configuration” is provided by NG-RAN.

Proposal 3:	Postpone discussion on “how the NW determines whether data collection should be initiated” until further progress is made on the different data collection architecture options.

Proposal 4: 	UE can explicitly report which AI/ML functionality(ies) are “non-applicable”.

Proposal 5: 	An AI/ML functionality does not need to be “active” to be reported as non-applicable.

Proposal 6: 	UE indicates the reason an AI/ML functionality is non-applicable.

Proposal 7: 	Postpone applicable functionality reporting for neighbour/target cells until the baseline applicable functionality reporting framework is concluded.

# References

Draft-RAN2_127_Meeting_Report_v2

R2-2406202 - RAN2#126 chair notes

R2-2407848 LS on applicable functionality reporting for beam management UE-sided model – Intel

Draft-RAN2_127bis_Meeting_Report

R2-2410504, [POST127bis][020][AI PHY] Reply LS to SA2/SA5 (InterDigital, Nokia), RAN2-128, Orlando, USA, November, 2024