3GPP TSG-RAN WG2 Meeting #128	R2-2410590

Orlando, USA, 18th – 22th November, 2024

Agenda Item:	8.8.5

Source:	Huawei, HiSilicon, Turkcell

Title:	Discussion on Regenerative payload

Document for: Discussion and Decision

# 1	Introduction

In RAN2 #127 meeting, the setting of common TA and Kmac for regenerative payload was discussed, and the LS was sent to RAN1 and RAN4 asking whether to introduce negative value for common TA [1]. RAN1 replied in [2]that the majority don't want a negative common TA. The RAN1 response is as follows:

Response: A majority of companies in RAN1 think that it would not be a problem, depending on gNB implementation in a regenerative payload scenario, to stick to 0 as minimum possible value for TA common without introducing the negative value for ta-Common.

RAN4 replied in [3] that no consensus was reached in RAN4:

No consensus on whether there is an issue due to non-negative value for common TA or whether negative values should be introduced for ta-common. Note: No more discussion of this issue is expected in RAN4.

In this paper we provide our views on the TA parameters in the regenerative payload.

# 2	Discussion

In transparent scenario the UE is responsible for compensating the TA of UE-satellite-RP, and the TA of RP-gNB is compensated by the gNB. The UE calculates the UE-satellite TA based on location, and acquires satellite-RP TA, i.e., the common TA, broadcasted by the NW. If the UE’s calculation is not precise enough, e.g., due to the outdated GNSS position, the NW will indicate the extra value that the UE needs to compensate in Msg2 RAR.

In this mechanism, as shown in Fig 1, to avoid negative value in Msg2 RAR, the indicated common TA value to the UE may be set smaller than the ideal value, by implementation. The practical RP corresponding to the common TA value indicated to the UE is closer to the UE compared with the ideal RP.

Fig 1. Smaller common TA to avoid negative RAR in transparent payload

Following legacy principle, for the regenerative payload, the ideal RP should be just the same as the gNB. But considering the above analysis for the UE side TA calculation errors, the practical RP may be located closer to the UE, i.e., between the UE and the on-board gNB. In this condition, the common TA is not zero but a negative value.

But many companies in RAN1 thought the gNB implementation can tolerate such error, and were reluctant for introducing negative common TA value in spec [4]. Specifically, for TA accuracy error falls in [-Te, +Te] with Te smaller than CP length, gNB implementation can avoid the performance degradation. The minimum common TA indicated to UE can be zero, and if the UE’s calculated TA result is larger, the gNB implementation can solve the problem.

Observation 1: Neither RAN1 nor RAN4 reached any consensus on introducing negative values for common TA parameters.

According to RAN1 LS, RAN2 can specify for regenerative payload the RP is located at the satellite gNB, and the value of common TA and Kmac is zero. Hence we propose that

Proposal 1: Capture in TS 38.300 for regenerative payload, the RP is located at the satellite gNB and the value of common TA and Kmac can be configured as zero.

# 3	Conclusion

This contribution analyses the TA issue for regenerative payload, and proposes the following:

Observation 1: Neither RAN1 nor RAN4 reached any consensus on introducing negative values for common TA parameters.

Proposal 1: Capture in TS 38.300 for regenerative payload, the RP is located at the satellite gNB and the value of common TA and Kmac can be configured as zero.

# 4	References

- R2-2407623, LS on common TA in a regenerative payload scenario, RAN2 (contact company: CMCC)

- R1-2409258, Reply LS on common TA in a regenerative payload scenario, RAN1 (contact company: CMCC)

- R4-2416920, Reply LS on common TA in a regenerative payload scenario, RAN4 (contact company: CMCC)

- R1-2409256, Moderator’s summary on the discussion of common TA in a regenerative payload scenario, Moderator (CMCC), RAN1#118bis.