3GPP TSG-RAN WG2 #128	R2-2410610

Orlando, FL, USA, Nov 18th– 22nd, 2024

Agenda Item:	8.5.4

Source:	Ericsson

Title:	Adaptation of common signal/channel transmissions for NES

Document for:	Discussion

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised in [4]. The core part objective based on the revised WID is:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this paper, we address the third objective given above. In RAN2#125bis and RAN2#127, and RAN2#127bis meetings it was only possible to discuss the adaptation of paging occasions within the context of this third objective. For the RACH adaptation in spatial domain, it was agreed, during RAN2#126, that RAN2 will count on RAN1 evaluations and the conclusion. For reference regarding the progress of the discussion in RAN1 on the third objective, please see the list of RAN1 agreements in the Appendix.

# 2	Discussion

In this section, we consider the following aspects:

- adaptation of SSB signals,

- adaptation of PRACH occasions and related signaling, and

- adaptation of paging.

### 2.1 Adaptation of SSB signals

Currently, SSBs are specified as always-on transmissions whose periodicity can be changed semi-statically. A legacy NR UE may assume a default SSB periodicity of 20ms during initial cell search. Even though the frequency of most broadcast transmissions can be minimized (e.g., with a periodicity of 160ms or larger), SSB transmissions for UE initial cell search are required to be provided every 20ms regardless of whether there is light/no traffic in the cell. This leads to energy consumption in the network, which can be minimized by adapting the SSB periodicity or skipping SSB transmissions on certain occasions.

In TR 38.864 [2], the evaluation results for adaptation of SSB periodicity were captured. The results show that significant energy savings can be achieved on the network side with such adaptation. For example, Figure 1 shows how much energy saving can be achieved only by changing the periodicity of SSB transmissions from 20ms to 40ms. Note that the setup assumes that the other transmissions/receptions are minimized (e.g., SIB1/PRACH is 160ms) to show the full potential of such SSB adaptation.

Figure 1: NW energy saving (NES) when SSB periodicity is increased from 20ms to 40ms.

- Extending the periodicity of SSB transmissions in a cell can provide significant energy savings for the network.

In Rel-19, we think that UEs in RRC connected mode supporting the feature can be informed dynamically about the change in the periodicity of SSB transmissions. SSBs can, for example, be provided sparsely as a baseline, and upon need, such as prior to/during handover, positioning services (E-CID) or alike, additional SSBs can be provided on-demand. For UEs in RRC idle/inactive mode, having a longer SSB periodicity e.g., > 20ms as a baseline, and activating additional SSBs upon need can also be considered.

Support additional SSB transmissions that can be configured dynamically when needed, e.g., prior to/during handover, positioning services etc., on top of baseline SSB transmissions in RRC connected mode.

For UEs in RRC idle/inactive mode, a non-initial-access cell may have SSB periodicity of 160ms, as captured in existing specifications:

SSB-MTC2-LP-r16 ::= SEQUENCE {

pci-List SEQUENCE (SIZE (1..maxNrofPCIsPerSMTC)) OF PhysCellId OPTIONAL, -- Need R

periodicity ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1}

}

For example, if a capacity cell is overlapped by a coverage cell, the SSB transmission rate of the capacity cell may be every 160ms while the UE will be able to find the overlapping coverage cell for initial access as it is providing SSBs every 20ms. Then through neighbor cell information (SSB-MTC2-LP) provided by the coverage cell the UE may reselect to the low-rate SSB capacity cell.

Figure 2: Deployment according to legacy specifications in which a coverage cell providing initial access functionality provides SSB every 20ms, while the overlapped capacity cell (non-initial-access) provides SSBs every 160ms.

- In legacy specifications the periodicity of SSB can be 160ms for non-initial access cells.

However, in practice such low-rate SSB cells are not deployed. The main reason is that there may be UEs, for example UEs in poor coverage, in need of multiple SSBs (up to 3 SSBs) before such UEs can decode the paging message. This is visualized in Figure 3, in which case the UE needs to wake up more than 320ms ahead of the PO for processing the required number of SSBs. Note that PEI would not have helped in this scenario as the PEI occasion would still be 3 SSBs ahead and upon paging the UE would not be able to go back to deep sleep until the PO.

Figure 3: Timeline of RRC Idle/Inactive UE in need of multiple SSBs before the PO.

Instead, additional SSBs (in addition to a baseline SSB transmission rate) can be transmitted on demand prior to the POs for Rel-19 UEs in RRC Idle/Inactive. Through such a scheme, which is backwards compatible as legacy UEs can rely on the baseline SSB transmissions, it would then become practical to deploy such cells that are configured with low rate SSB transmissions. Hence, we think that adaptations to SSBs in capacity cells should be supported also in RRC Idle/inactive.

- Capacity cells are typically not configured with SSB transmissions with low periodicity, e.g., 160ms, since some UEs may need multiple SSB transmissions prior to PO/PEI decoding.

Figure 4: Timeline of RRC Idle/Inactive UE in need of multiple SSBs before the PO if SSB adaptations are done. The UE may choose to wake up closer to PO when additional SSBs are provided.

- Adapting SSBs for UEs in Idle/Inactive mode such that additional SSBs are added on top of low-periodicity SSBs prior to POs makes deployment of such cells with low-rate SSB transmissions practical.

In RRC Idle/Inactive mode, on top of baseline SSB configuration, additional SSBs can (dynamically or semi-statically) be provided on-demand, e.g., additional SSBs are transmitted prior to PO.

Dynamic SSB adaptations on PCell in RRC Connected mode are beneficial for multiple scenarios such as for fast moving UEs, during handover, positioning, etc. Similar as agreed for the objective 1 for OD-SSB provision on SCells via MAC-CE signalling, we think that a MAC-CE based mechanism could be used for PCell to adapt the SSB burst periodicity for the connected mode. We think that the values supported in current specifications (between 5ms and 160 ms) are sufficient for such adaptation. The 160ms periodicity ensures that a UE can operate in the network and gives the network the opportunity to enter deep sleep state. SSBs with 160ms could then be the baseline configuration. When needed, additional SSBs can be provided on top of the baseline configuration as often as up to every 5ms for optimal performance.

For connected mode, support dynamic adaptation of SSB in time domain based on MAC CE indication. The signaling should be common with Objective 1.

It shall be noted that one important scenario relevant for SSB-adaptations on PCell is for mobility, where additional SSBs from neighbour cells are transmitted when the UE approaches the cell border, or positioning, where additional SSBs from neighbour cells improve the measurement accuracy and reduce the measurement delay. When it comes to neighbour cells, a gNB can specify the neighbouring active (present) SSBs via the parameter ssb-ToMeasure and the associated rate/periodicity via the SSB Measurement Timing Configuration (SMTC), which defines the time window during which the UE measures the SSBs belonging to these neighbouring cells. SMTC of neighbour cells needs to cover the additional SSBs for a UE to benefit from the SSB adaptation on neighbour cells. That is, in addition to the two SSB configurations and the suggested signalling, the SMTC configuration for a UE needs to be dynamically adapted accordingly.

- Dynamic adaptation of SSB on neighbour cells need to be accompanied by corresponding dynamic adaptation of SMTC.

Introduce mechanism for dynamic SMTC adaptation.

### 2.2 Adaptation of PRACH occasions and the related signaling

Another important periodic/always-on activity for a cell that supports initial access is PRACH occasions and their periodicity. Similar to the discussion on SSB transmissions, PRACH occasions can be adapted dynamically rather than only semi-statically (via system information update) as specified today. These resources can be dynamically adjusted to accommodate changes in traffic load or save energy at the gNB. The PRACH reception on such cells must continue quite frequently even when the cell has no/light traffic. Dynamic adaptation of the PRACH occasions in the time domain, i.e., without using the system information update notification mechanism, allows energy saving on the network side while not compromising the initial access performance.

In TR 38.864 [2], the evaluation results for adaptation of the frequency of PRACH occasions were captured. The evaluations show that significant energy savings can be achieved by adapting the frequency of PRACH occasions. For example, Figure 5 shows how much energy savings can be achieved by changing the periodicity of PRACH occasions between 10ms and 40ms. Note that it is assumed in the setup that SSBs are broadcast every 20ms (as in legacy) and SIB1 is broadcast every 40ms.

Figure 5: 10ms PRACH occasion periodicity (left), versus another with 40ms PRACH occasion periodicity (right).

- Changing the periodicity of PRACH occasions in a cell can provide significant energy savings for the network.

In Rel-19, we think that UEs in RRC connected mode supporting the feature can be informed dynamically about the change in the periodicity of PRACH occasions. For UEs in RRC idle/inactive mode, changing the periodicity of PRACH occasions can also be considered. This can be realized by introduction of a second/additional configuration for PRACH occasions for Rel-19 UEs and dynamic signalling, such as DCI, can be used to activate these additional PRACH occasions per need basis. This would be backwards compatible as legacy UEs can continue to use such a carrier because the legacy PRACH configuration is still provided via system information broadcast and can be used. The only drawback is that the legacy UEs will be limited to use the less frequent PRACH configuration, i.e. they cannot use the frequent PRACH configuration.

Support activation of additional PRACH occasions dynamically for UEs in RRC Idle/Inactive and Connected modes

When it comes to signalling, the dynamic adaptation of additional PRACH resources can be achieved without requiring extra wake-up for UEs. For idle/inactive UEs, this adaptation can be communicated during their paging occasions (POs) (e.g., in paging DCI) and included as part of SIB1 (e.g., in SIB1 DCI), as paging and SIB1 reading are already part of the UEs’ regular idle/inactive mode activities. We believe that providing such an indication at POs and in the message that schedules SIB1 would be beneficial for a UE that would trigger network access after acquiring SIB1 or receiving a paging message.

It should be possible to activate additional PRACH resources for UEs in idle/inactive mode at paging occasions, and when SIB1 is broadcast.

UEs in connected mode monitor a paging occasion at least once per modification period, see from 38.331, section 5.2.2.2.2:

“UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.”

If paging DCI supports an adaptation mechanism for additional PRACH resources for both idle and connected UEs, this simplifies dynamic control of random-access resources and improves efficiency by using the same signaling for both scenarios. Furthermore, it does not require additional wakeup for UEs as they anyway receive this DCI.

- A unified signalling method for adapting additional PRACH resources for both idle/inactive and connected UEs simplifies the dynamic control and improves efficiency by utilizing the same signalling for both scenarios.

It should be possible to separately activate additional PRACH resources for UEs in connected mode at paging occasions.

There are two types of random access that can be configured, i.e., CBRA and CFRA. It is important to note that CBRA can be configured in all RRC states, i.e., RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED states, while CFRA can only be configured to UEs in RRC_CONNECTED state. In RAN1#116-bis, the following agreement was made:

Agreement

Support adaptation mechanisms of PRACH in time-domain for following:

- UE in idle/inactive mode

- UE in connected mode

Therefore, in order to support adaptation of PRACH occasions for UEs in idle/inactive as well as UEs in connected mode, we think that adaptation of PRACH occasions for CBRA should be prioritized over adaptation of PRACH occasions for CFRA. Furthermore, our opinion is that PRACH adaptations are necessary for CBRA depending on UL access load. For example, upon handover, in case of high load, a dedicated RACH configuration can be provided. Hence, CFRA-specific adaptations are not necessary in our view.

Adaptation of PRACH occasions is supported for CBRA.

Discuss whether there is a need to introduce adaptation of PRACH occasions for CFRA.

Another aspect that needs to be addressed is whether adaptation of PRACH occasions is supported for 2-step and 4-step RA. In our opinion, supporting 2-step RA should be optional as it is only applicable in good RSRP. Additionally, supporting 2-step RA may result in larger signaling overhead (than for 4-step RA). Finally, one needs to consider whether additional PUSCH resources would need to be dedicated in the case of optimizations for the 2-step RA. Therefore, we propose to prioritize the adaptation of PRACH occasions for 4-step RA.

Support adaptation of PRACH adaptations for 4-step RA as baseline.

### 2.3 Adaptation of paging

The adaptation of paging was discussed during RAN2#125bis and RAN2#127, and the following agreements were made:

Agreements on paging adaptation (RAN2#125bis):

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

Agreements on paging adaptation (RAN2#127):

- Option-a) is about the bundling of PF for R19 NES UEs.

- R2 observe that the option-a) and option-b) can be designed to configure the PO:s at same time position.

Agreements on paging adaptation (RAN2#127bis):

- Select option-b as baseline for R19 NES paging enhancement.

- R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values).

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Legacy UE is not barred.

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

The motivation for the existing framework of paging messages is to distribute the paging frames/occasions (PFs/POs) evenly across DRX cycles in idle/inactive modes. This is to balance the load not only from gNB perspective in a cell, but also from UE perspective to receive the paging message within a period intended based on the configured DRX cycle. From network energy saving perspective, such framework does not provide many opportunities for a gNB to sleep. It may become even more challenging to achieve energy savings when paging messages are broadcasted in all cells in a tracking area.

One of the aspects that needs to be addressed is how dynamic the paging adaptations should be. In our opinion, there is no need to introduce a dynamic adaptation mechanism due to its complexity. We think that semi-static configurations and updates via system information update notification, as in legacy, are good enough.

Support only semi-static configurations and updates via system information update notification, as in legacy, for paging adaptations.

Next, as highlighted in the list of agreements from RAN2#127bis meeting, option-b) is selected as baseline solution for supporting paging adaptations. According to the agreement from RAN2#125bis meeting, option-b) is about extending the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. On one hand, we observe that decreasing the number of PFs by extending the set of possible values for N would not be significantly beneficial in terms of NW energy savings unless legacy UEs are barred in the cell. This is because a cell supporting legacy UEs would still need to wake up for the sake of legacy PFs/POs that are configured according to the legacy value range of parameter N. On the other hand, it was agreed in RAN#127bis meeting that a legacy UE is not barred in the case of Rel-19 NES paging adaptations. Considering these two agreements from RAN#125bis and RAN2#127bis meetings, it would be desirable to clarify that option-b) is not about extending the values of N, but only about increasing the number of POs per PF (i.e., increasing the values of Ns).

For option b), a paging adaptation mechanism only based on increasing the values of Ns, i.e., increasing the possible number of POs per PF, would be sufficient.

One of the questions raised during RAN2#127 meeting and discussed further in the subsequent email discussion and RAN2#127bis meeting concerns the RRC signalling overhead in the case of option-b). Concerning this issue, RAN2 agreed in RAN2#127bis meeting that “R2 should aim at signalling overhead minimization” and there is an FFS on supporting Ns values larger than 8. Next, we discuss the means of minimizing the signalling overhead for option-b).

Regarding the maximum possible value for Ns for Rel-19 UEs, one needs to consider the paging capacity for different values of parameter Ns. To do so, let us recall that paging adaptations can provide network energy savings in scenarios where legacy PFs are sparsely configured. Here’s an assessment of the paging capacity for a typical scenario suitable for paging adaptations in which legacy PFs are configured as sparse as possible (i.e., N = T/16 according to the current specifications), SCS is set to 15 kHz, and Ns for legacy UEs is set to the highest possible value (i.e., Ns = 4) whereas Ns for Rel-19 UEs is set to 8, and the number of UEs that can be paged simultaneously is set to 32 (i.e., maxNrofPageRec INTEGER ::= 32 -- Maximum number of page records).

Then, we have the following for the number of UEs that can be paged per second:

- It is possible to page 6,25*(4 + 8) *32 UEs = 2400 UEs per second, where

  - 6,25 is the number of PFs per second (follows from the fact that for SCS = 15 kHz and N = T/16 there is a PF every 160ms).

Now, let us consider a legacy scenario with Ns set 4, SCS set 15 kHz, and considering that the number of UEs that can be paged simultaneously is set to 32. In this case, we have the following for the number of UEs that can be paged per second:

- For N = T/4, it is possible to page 25*4*32 UEs = 3200 UEs per second, where

  - 25 is the number of PFs per second (follows from the fact that for SCS = 15 kHz and N = T/4 there is a PF every 40ms).

- For N = T/8, it is possible to page 12,5*4*32 UEs = 1600 UEs per second, where

  - 12,5 is the number of PFs per second (follows from the fact that for SCS = 15 kHz and N = T/8 there is a PF every 80ms).

Based on the above calculations, we observe that the paging capacity that can be provided by setting the Ns for Rel-19 UEs to 8 is sufficient and comparable to the paging capacity that can be provided with legacy configurations. Therefore, we conclude that supporting higher values for parameter Ns for Rel-19 UEs would lead to unnecessary signalling overhead.

- It is sufficient to extend the value range of Ns for Rel-19 UEs with 8 considering the paging capacity that can be achieved with respect to legacy configurations.

No need to extend the value range of Ns for Rel-19 UEs beyond 8.

Furthermore, to allow for configuration flexibility, we propose to support 1, 2 and 4, similar to legacy, as possible values for parameter Ns for Rel-19 UEs.

The codepoints of Ns for Rel-19 UEs are 1, 2, 4 or 8.

Below, we evaluate the message size when Ns for Rel-19 UEs is set to 8 and compare the outcome with the message size of legacy configuration as baseline. In the following example, for the sake of evaluation, we assume a SIB1 configuration with hardly anything but the paging configuration in all scenarios.

- Legacy baseline scenario: T = 128, N = T/16, Ns = 4 (i.e., every 16th RF is a PF, and each PF is associated with 4 POs), and four instances of firstPDCCH-MonitoringOccasionOfPO parameter is configured for SCS = 15 kHz (i.e., corresponds to the case sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT from firstPDCCH-MonitoringOccasionOfPO). This scenario is illustrated in Figure 6.

Figure 6: Legacy baseline scenario (T = 128, N = T/16, Ns = 4, and SCS =15 kHz).

- Option b) scenario: T = 128, N = T/16, for legacy UEs Ns = 4 and for Rel-19 UEs Ns = 8 (i.e., every 16th RF is a PF, and each PF is associated with 4 POs for legacy UEs and 8 POs for Rel-19 UEs), SCS = 15 kHz (i.e., corresponds to the case sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT from firstPDCCH-MonitoringOccasionOfPO). This scenario is illustrated in Figure 7.

Figure 7:  Option b) scenario (T = 128, N = T/16, Ns for legacy = 4, Ns for Rel-19 UEs = 8 and SCS =15 kHz).

In Table 1, we provide the message sizes for the scenarios above. We assumed that the firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs are defined in the same way as in legacy (i.e., the range of firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs is given in symbols).

Table 1: Message sizes for different paging configurations

|  | Message size [octets] |
| --- | --- |
| Legacy (baseline) | 28 |
| Option b) with SCS = 15 kHz | 41 |
| Option b) with SCS = 480 kHz | 50 |

According to the results above in Table 1, option b) with SCS = 15 kHz and option b) with SCS = 480 kHz have 13 and 22 octets larger message size than the legacy (baseline) configuration, respectively. This accounts for approximately 46% and 79% increase in message size. One can observe that option b) introduces significant signalling overhead for the case where Ns for Rel-19 UEs is set to 8.

- Option b) introduces significant signalling overhead (~46% larger message size than the respective legacy configuration with T = 128, N = T/16, Ns for legacy = 4, SCS = 15 kHz) even foe the case where Ns for Rel-19 UEs is set to 8.

Based on the results in Table 1 above, we think that it would be beneficial to reduce the impact of option b) on the message size. This can be simply achieved by defining the range of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level instead of symbol level as currently defined for legacy UEs. Note that we are not proposing to change the meaning of firstPDCCH-MonitoringOccasionOfPO, but its resolution only  (i.e., the unit of firstPDCCH-MonitoringOccasionOfPO parameter would still be monitoring occasions).  This would allow us to define the range of the firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs to be the same as of the firstPDCCH-MonitoringOccasionOfPO parameters for legacy UEs but with fewer bits. In the Appendix, we present the details of how this modification could be captured in the context of the ASN.1 signalling.

In Table 2, we summarize the evaluation for message sizes for two scenarios with option b) in which firstPDCCH-MonitoringOccasionOfPO parameters for Rel-19 UEs are defined at the slot level.

Table 2: Message size for different paging configuration scenarios (slot level configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs).

|  | Message size [octets] |
| --- | --- |
| Option b) with SCS = 15 kHz (slot level configuration) | 37 |
| Option b) with SCS = 480 kHz (slot level configuration) | 46 |

Based on the results in Table 2 above, the configuration of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level results in 4 octets smaller message size than the configuration of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at symbol level, which corresponds to approximately 31% reduction of message size.

- The configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level results in 4 octets (~31%) smaller message size than the configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at symbol level.

Define values of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level.

# 3	Conclusion

In this contribution we discuss the objective on the adaptation of common signal/channel transmissions. Based on the discussion in the previous sections we made the following observations:

Observation 1	Extending the periodicity of SSB transmissions in a cell can provide significant energy savings for the network.

Observation 2	In legacy specifications the periodicity of SSB can be 160ms for non-initial access cells.

Observation 3	Capacity cells are typically not configured with SSB transmissions with low periodicity, e.g., 160ms, since some UEs may need multiple SSB transmissions prior to PO/PEI decoding.

Observation 4	Adapting SSBs for UEs in Idle/Inactive mode such that additional SSBs are added on top of low-periodicity SSBs prior to POs makes deployment of such cells with low-rate SSB transmissions practical.

Observation 5	Dynamic adaptation of SSB on neighbour cells need to be accompanied by corresponding dynamic adaptation of SMTC.

Observation 6	Changing the periodicity of PRACH occasions in a cell can provide significant energy savings for the network.

Observation 7	A unified signalling method for adapting additional PRACH resources for both idle/inactive and connected UEs simplifies the dynamic control and improves efficiency by utilizing the same signalling for both scenarios.

Observation 8	It is sufficient to extend the value range of Ns for Rel-19 UEs with 8 considering the paging capacity that can be achieved with respect to legacy configurations.

Observation 9	Option b) introduces significant signalling overhead (~46% larger message size than the respective legacy configuration with T = 128, N = T/16, Ns for legacy = 4, SCS = 15 kHz) even foe the case where Ns for Rel-19 UEs is set to 8.

Observation 10	The configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level results in 4 octets (~31%) smaller message size than the configuration of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at symbol level.

Based on the discussion in the previous section, we propose the following:

Proposal 1	Support additional SSB transmissions that can be configured dynamically when needed, e.g., prior to/during handover, positioning services etc., on top of baseline SSB transmissions in RRC connected mode.

Proposal 2	In RRC Idle/Inactive mode, on top of baseline SSB configuration, additional SSBs can (dynamically or semi-statically) be provided on-demand, e.g., additional SSBs are transmitted prior to PO.

Proposal 3	For connected mode, support dynamic adaptation of SSB in time domain based on MAC CE indication. The signaling should be common with Objective 1.

Proposal 4	Introduce mechanism for dynamic SMTC adaptation.

Proposal 5	Support activation of additional PRACH occasions dynamically for UEs in RRC Idle/Inactive and Connected modes

Proposal 6	It should be possible to activate additional PRACH resources for UEs in idle/inactive mode at paging occasions, and when SIB1 is broadcast.

Proposal 7	It should be possible to separately activate additional PRACH resources for UEs in connected mode at paging occasions.

Proposal 8	Adaptation of PRACH occasions is supported for CBRA.

Proposal 9	Discuss whether there is a need to introduce adaptation of PRACH occasions for CFRA.

Proposal 10	Support adaptation of PRACH adaptations for 4-step RA as baseline.

Proposal 11	Support only semi-static configurations and updates via system information update notification, as in legacy, for paging adaptations.

Proposal 12	For option b), a paging adaptation mechanism only based on increasing the values of Ns, i.e., increasing the possible number of POs per PF, would be sufficient.

Proposal 13	No need to extend the value range of Ns for Rel-19 UEs beyond 8.

Proposal 14	The codepoints of Ns for Rel-19 UEs are 1, 2, 4 or 8.

Proposal 15	Define values of firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs at slot level.

# References

RP-240170, “Revised WID: Enhancements of network energy savings for NR”, RAN103, Maastricht, Netherlands, March 2024.

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

3GPP TS 38.304 V18.1.0 User Equipment (UE) procedures in Idle mode and in RRC Inactive state

RP-242354, “Revised WID: Enhancements of network energy savings for NR”, RAN#105, October 2024.

# Appendix

### Examples of Rel-19 firstPDCCH-MonitoringOccasionOfPO configuration

Example 1 Symbol level configuration of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs.

Example 2 Slot level configuration of the firstPDCCH-MonitoringOccasionOfPO parameter for Rel-19 UEs.