3GPP TSG-WG2 Meeting #128	R2-2410628

# Orlando, U.S.A, 18th  - 22nd  NOV, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2009 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to 38.321 on RACH Initiation for RACH based LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the RACH based LTM, RACH shall be triggered if there is no valid TA of the target cell can be obtained by UE via either UE based TA measurement or LTM Cell switch MAC CE. But it is not explicitly defined in the specification due to the following procedure: One reason is shown as below: After Receiving the LTM Cell switch MAC CE, UE apply the RRCReconfiguration for the indicated candidate cell and generate the RRCReconfigurationComplete, according to the regular BSR triggering rules, the regular BSR would be triggered due to the new data arrival, and considering there is no valid PUSCH could be used for transmitting the BSR MAC CE due to the lack of valid TA value, then SR would be triggered and pending, then, due to the same reason, the RACH shall be triggered since there is no valid PUCCH can be used for transmitting pending SR. It can be seen that a RACH procedure for RACH based LTM is a very complex procedure which involves multiple Layer 2 procedures including (RRC, BSR, SR). However, among those procedures, UE processing time for those procedures is up to UE implementation, for example, the SR triggered by BSR MAC CE, from implementation perspective, SR will not be triggered as soon as the BSR is triggered but after a delay time, how long the delay time is up to UE implementation. Observation 1: The delay time between the reception of LTM Cell switch MAC CE and the initiation of the RACH is uncertain and it is up to UE implementation. However, from NW perspective, one important purpose of LTM is to reduce the cell switch delay caused by LTM, for achieving such purpose, NW send the LTM Cell switch MAC CE in a time instance which could be closed to the RACH resources as much as possible. Observation 2: In order to reduce the delay caused by LTM as much as possible, NW will send the LTM Cell switch MAC CE in a time instance which shall be closed to the RACH resources as much as possible, especially the CFRA resources is indicated in the LTM Cell switch MAC CE. However, considering the delay time between the reception of LTM Cell switch MAC CE and initiation of the RACH is uncertain, UE may miss the upcoming RACH resources and wait for the next round RACH resources if the time between the reception of LTM Cell switch MAC CE and RACH initiation left by NW is not sufficient. Observation 3: UE may miss the upcoming RACH resources if the time between the reception of LTM Cell switch MAC CE and RACH initiation is not sufficient, as a result ,the huge delay will be caused since the UE need wait for the next round RACH resources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1: clarify that in the subcaluse 5.18.35, the MAC entity shall initiate the RACH procedure if the TA value are received in the LTM cell swtich MAC CE is FFF and UE cannot get TA by using UE based TA measurement. Impact analysis Impacted 5G architecture options: NR-DC, SA, MR-DC Impacted functionality: LTM Interoperability: If the NW implements this CR but not the UE, UE may miss the upcoming RACH resources due to the uncertain delay caused by the complex procedure that triggers RACH if NW schedules the LTM Cell switch MAC CE transmission as much closed to the occasion of RACH resources as possible. If the UE implements this CR but not the NW, there is no any interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The considerable delay will be caused by waiting for the next round of the RACH resources. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.35 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.18.35	LTM Cell Switch Command

The network may instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE described in clause 6.1.3.75.

The MAC entity shall:

1>	if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell:

2>	indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE;

2>	if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers:

3>	if Timing Advance Command value (hexa-decimal) is not set as FFF; or

3> if the UE is configured with UE-based Timing Advance measurement as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the SpCell of the indicated LTM target configuration:

4>	process the obtained Timing Advance Command (see clause 5.2);

4>	consider the RACH-less LTM cell switch to be ongoing;

4>	if the MAC entity is associated with SCG:

5>	indicate to upper layers to skip the Random Access procedure for this LTM cell switch.

4> consider the SSB associated to the TCI state indicated by TCI state ID field as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell for RACH-less LTM cell switch (as in clause 5.8.2);

4> indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

3> else:

4> initiate the RACH for LTM cell switch (as in clause 5.1).