3GPP TSG RAN WG2 #128			R2-2410649

Orlando, US, November 18th – 22nd, 2024

Source: 	Fujitsu

Agena item:	8.8.5

Title:	Discussion on regenerative payload

Document for:	Discussion and decision

Background

In this contribution, we would like to discuss and provide our views on the Regenerative Payload from RAN2 perspective.

Discussion

Time-based measurement

t-Service

In Rel-17 NTN, only transparent payload case was considered. In such case, the feeder link switches may happen regardless of service link quality. To address this issue, the t-Service which indicates the time information on when the cell is going to stop serving the NTN UE due to feeder link switches and service link switches and the time-based measurement were introduced to initiate the measurement on NTN UE before t-Service, allowing for IDLE mode NTN UE to have enough time to reselect the best cell.

Observation 1: Time-based measurement can be initiated by the NTN UE based on the t-Service.

In Rel-19 NTN, satellites with regenerative payload are considered. Different from the transparent payload, feeder link switches may not affect the service link, because feeder link switches may not change the serving cell. In other words, the timing of the cell going to stop serving the NTN UE is not the same as that of feeder link switches.

Observation 2: In regenerative payload case, the timing of the cell going to stop serving the NTN UE is not the same as that of feeder link switches.

Meanwhile, in the current specification, the description of t-Service is defined as follows [1]:

t-Service

Indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering. This field applies for both service link switches in NTN quasi-Earth fixed cell and feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell.

Even in a case of a regenerative payload, service link switches will occur in earth-moving system, so the time-based measurement is useful for the NTN UE to indicate appropriate time for measurements, however, based on the above observations, t-Service may not be applicable for feeder link switches case.

Therefore, to support the time-based measurement in a regenerative payload, RAN2 clarifies the t-Service does not apply for feeder link switches case.

Proposal 1: In order to support time-based measurement in a regenerative payload, RAN2 clarifies the t-Service does not apply for feeder link switches case.

Conclusion

In this contribution, the following proposals and observations were made:

Observation 1: Time-based measurement can be initiated by the NTN UE based on the t-Service.

Observation 2: In regenerative payload case, the timing of the cell going to stop serving the NTN UE is not the same as that of feeder link switches.

Proposal 1: In order to support time-based measurement in a regenerative payload, RAN2 clarifies the t-Service does not apply for feeder link switches case.

References

- 3GPP TS38.331 V18.3.0, Radio Resource Control (RRC) protocol specification (Release 18)