3GPP TSG-RAN WG2#128		      					R2-2410660

Orlando, USA, November 18 – 22, 2024			     Revision of R2-2409000

Agenda item:	8.6.2

Source: 	Kyocera

Title: 	Potential issue on coexistence of inter-MN LTM and intra-SN LTM

Document for:	Discussion

Introduction

RAN#105 approved the revised WID for NR mobility enhancements Phase 4[1]. In the RAN2#127-bis meeting, the coexistence of Inter-MN LTM and Intra-SN LTM was discussed and the following cases were agreed [5]:

Agreements on inter-CU LTM

11. RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

In this contribution, the potential issue on coexistence of inter-MN LTM and intra-SN LTM is discussed.

Discussion

Potential issue on VarLTM-Config for SCG

In case of coexistence of inter-MN LTM and intra-SN LTM, the scenario of “Inter-MN MCG LTM with SN released” has a potential issue in handling the SCG LTM Config after MCG LTM. The scenario of Inter-MN MCG LTM with SN released is depicted in Figure 1, which shows the UE stores the intra-SN LTM configuration when inter-MN LTM is executed.

Figure 1 Inter-MN MCG LTM with SN released

According to the TS 38.331 [2] below, the UE maintains MCG and SCG LTM Configs in separate VarLTM-Configs and each LTM Config is configured from the MN and the SN respectively, i.e., it’s the problem that the MN cannot remove SCG LTM Config. Our understanding is shown in Figure 2.

The network configures the UE with one or more LTM candidate configurations within the LTM-Config IE. In NR-DC, the UE may receive two independent ltm-Config: - an ltm-Config associated with the MCG that is included within an RRCReconfiguration message received via SRB1; and - an ltm-Config associated with the SCG that is included within an RRCReconfiguration message either received via SRB3, or, alternatively, embedded in an RRCReconfiguration message received via SRB1.

Before “Inter-MN MCG LTM with SN released”, the UE is configured with two LTM Configs as follows:

- MCG LTM Config: A mixture of Intra-MN cells and Inter-MN cells can be configured.

- SCG LTM Config: Only Intra-SN cells can be configured.

Figure 2 LTM Config stored in UE before Inter-MN MCG LTM with SN released

According to the TS38.300 [3] below, for the subsequent LTM, the UE maintains LTM Configs until the NW explicitly indicates to remove it. Therefore, the UE maintains SCG LTM Config (i.e., for intra-SN LTM which is no longer valid) after inter-MN LTM execution. Note that the SCG is released upon inter-MN LTM in this scenario.

However, it’s unclear if the target MCG will use a new DC with the same SN (i.e., the source SN for the Intra-SN LTM Config which the UE stores), so the target MCG may not use the DC or may configure the UE with a new DC with the target SN different from the source SN. Since the SCG is released in the scenario of “Inter-MN MCG LTM with SN released”, the stored SCG LTM Config (for Intra-SN) is no longer necessary from the viewpoint of the target MCG. Note that keeping DC with the same SN after Inter-MN MCG LTM execution is considered in “Inter-MN MCG LTM with SN unchanged” scenario.

Subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without releasing other LTM candidate configurations after each LTM cell switch completion.

Since the SCG is released in the scenario of “Inter-MN MCG LTM with SN released”, the stored SCG LTM Config (for Intra-SN) is no longer necessary from the viewpoint of the target MCG.

According to the 38.300 [3] below, the UE should perform L1 measurement as long as LTM Config in the LTM preparation phase is applicable. Therefore, maintaining the unnecessary LTM Config increases the UE power consumption.

- The UE performs L1 measurements on the configured candidate cell(s) and transmits L1 measurement reports to the gNB. L1 measurement should be performed as long as RRC reconfiguration (step 2) is applicable.

If the UE has unnecessary SCG LTM Configurations, the UE needs to perform L1 measurements as long as LTM Config is applicable, which causes a problem that the power consumption of the UE increases.

To avoid this issue, the unnecessary SCG LTM Config should be removed upon execution of “Inter-MN MCG LTM with SN released”, which can be only done by the SN according to the current specification. However, this SN is already released when Inter-MN MCG LTM is executed. In the current specification, neither the UE nor the MN can remove SCG LTM Config. So, RAN2 should discuss how to remove the unnecessary SCG LTM Config, when no SN is configured to the UE.

The unnecessary SCG LTM Config for intra-SN LTM associated with the source MN, should be removed upon execution of “Inter-MN MCG LTM with SN released”.

The SCG LTM Config can only be done by the SN according to current specification, but this SN cannot remove the SCG LTM Config because this SN was already released when Inter-MN LTM was executed.

RAN2 should discuss the issue in “Inter-MN MCG LTM with SN released” scenario that the VarLTM-Config for SCG (for intra-SN LTM associated with the source MN) cannot be removed after inter-MN LTM since the SN is already released in the target gNB.

Potential solutions

There are two potential approaches to remove the SCG LTM Configuration.

The issue in the current specification is that the SCG LTM Config (i.e., VarLTM-Config for SCG) can be only added/modified/removed by the SN. Therefore, if the SCG LTM Config can be also removed by the MN only during the inter-MN LTM, it can be avoided that the UE keeps unnecessary SCG LTM configurations. This can be considered as an approach to resolve the issue.

Approach 1: The MN removes the SCG LTM Config (for Intra-SN) as long as inter-MN LTM.

RAN2#127 agreed to introduce the new Rel-19 ID as follows [4]:

Agreements on inter-CU LTM

RLC and PDCP re-establishment in inter-CU LTM:

6. If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

RAN2#127bis achieved the following agreements [5]:

Agreements on inter-CU LTM

2. For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

The agreements are summarized as follows:

- If the Rel-19 IDs are different for the source cell and the target cell,

    - The UE performs PDCP re-establishment and the Rel-18 L2 reset operation (i.e., RLC re-establishment and MAC reset), including security key update.

- Else if ltm-ServingCellNoResetID and ltm-NoResetID are different,

    - The UE performs the Rel-18 L2 reset operation (i.e., RLC re-establishment and MAC reset)

In other words, the Rel-19 ID is intended to distinguish between intra-MN LTM and inter-MN LTM. If SN released scenario and SN unchanged scenario are not considered separately, the Rel-19 ID can be used by the UE to remove the stored SCG LTM Config (i.e., for intra-SN LTM) in case of Inter-MN LTM.

Approach 2: If the Rel-19 IDs are different for the source cell and the target cell, the UE removes the SCG LTM Config (for intra-SN LTM) autonomously.

In our view both Approach 1 and Approach 2 can be used to remove the SCG LTM Config, although Approach 1 is currently applicable for both DC and non-DC scenarios while Approach 2 is only applicable to non-DC scenario.

RAN2 should discuss whether the SCG LTM Config (for intra-SN LTM associated with the source MN) is removed in case of inter-MN LTM, by the MN (Approach 1) or by the UE using the Rel-19 IDs (Approach 2).

Conclusion

In this contribution, the potential issue of inter-MN LTM with SN-released is discussed.  RAN2 is kindly asked to take into account the observations and proposal below:

Observation 1	Since the SCG is released in the scenario of “Inter-MN MCG LTM with SN released”, the stored SCG LTM Config (for Intra-SN) is no longer necessary from the viewpoint of the target MCG.

Observation 2	If the UE has unnecessary SCG LTM Configurations, the UE needs to perform L1 measurements as long as LTM Config is applicable, which causes a problem that the power consumption of the UE increases.

Observation 3	The unnecessary SCG LTM Config for intra-SN LTM associated with the source MN, should be removed upon execution of “Inter-MN MCG LTM with SN released”.

Observation 4	The SCG LTM Config can only be done by the SN according to current specification, but this SN cannot remove the SCG LTM Config because this SN was already released when Inter-MN LTM was executed.

Proposal 1	RAN2 should discuss the issue in “Inter-MN MCG LTM with SN released” scenario that the VarLTM-Config for SCG (for intra-SN LTM associated with the source MN) cannot be removed after inter-MN LTM since the SN is already released in the target gNB.

Proposal 2	RAN2 should discuss whether the SCG LTM Config (for intra-SN LTM associated with the source MN) is removed in case of inter-MN LTM, by the MN (Approach 1) or by the UE using the Rel-19 IDs (Approach 2).

References

- RP-242356 “Revised Work Item: NR mobility enhancements Phase 4”, Apple Inc, China Telecom, RAN#105

- TS38.331, “Radio Resource Control (RRC) protocol specification”, 3GPP

- TS38.300, “NR and NG-RAN Overall Description; Stage 2”, 3GPP

- R2-2407571 “Report from session on V2X/SL, R18/19 MOB, and R19 NES”, Vice Chairman (Samsung), RAN2#127

- R2-2409211 “Report from session on V2X/SL, R18/19 MOB, and R19 NES”, Vice Chairman (Samsung), RAN2#127bis