3GPP RAN WG2 Meeting #128                                                                       R2-2410682

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	8.2.5

Source:	HONOR

Title:	Consideration on resource configuration for Topology 2

Document for:	Discussion and Decision

1. Introduction

In RAN2#127-bis, the following aspects were agreed for Topology 2 [1]:

Agreements

1	Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

2	The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.  FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

3	The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

This contribution focuses on the resource configuration issue between gNB and the reader.

2 Discussion

2.1 Resource configuration for the A-IoT air interface

In RAN2#127-bis meeting, it is agreed that dedicated resource for A-IoT air interface in topology 2 (T2) is only configured to the UE reader via dedicated signalling, instead of the shared resource pool. Therefore, the CN or UE reader should provide information to assist resource configuration (/allocation) when there is A-IoT service request. RAN2 has achieved the consensus on the visible information which includes:

Agreements

1 At least the following information are considered useful to be visible to the reader from CN

- The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

- targeted for one or more than one devices;

- approximate number of target devices (if available).

FFS on mandatory/optional

2 RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the A-IOT interface as upper layer data.

As shown in RAN2#127 agreement [2], NW is in control of resources to be used by A-IoT air interface. Therefore, the above visible information could also be helpful for gNB to estimate and configure resource for A-IoT air interface between UE reader and A-IoT devices in T2. For example, the gNB will allocate resource according to service type or the number of target devices, e.g., more resources will be allocated for full inventory or a large number of devices.

Additionally, in the last meeting, there is much discussion on whether the message size reporting is needed. RAN2 has agreed that it is beneficial for the reader to know an estimate of expected D2R message size in T1. For T2, gNB could consider the message size for (expected or maximum) D2R and R2D transmissions as an important factor for resource configuration, especially when the upper layer packet size is not fixed or the packet sizes for different A-IoT services (e.g., inventory or read/write command) differ.

Proposal 1: The following information should be visible to the gNB for resource configuration:

-	The service type of A-IoT (e.g., inventory, command). FFS if more information on command type (e.g., read/write/disable) is useful;

-	targeted for one or more than one device;

-	approximate number of target devices (if available);

-       message size for D2R and R2D transmissions over A-IoT air interface.

SA2 has identified three types of solutions for T2. It is the UE reader who will get the visible information if upper layer control plane solution or upper layer user plane solution is adopted. The assistance information should be delivered to gNB when the UE request resource for A-IoT air interface. For RRC-based solution, it is expected that the gNB could be visible to the assistance information from CN and can configure the resource directly, e.g., via RRC signalling.

Proposal 2: The visible information could be delivered to gNB by either UE reader or CN.

2.2 Resource reconfiguration for the A-IoT air interface

In the last meeting, RAN2 has agreed that the A-IoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc). Resource reconfiguration may occur in the following scenarios. For example, when a UE reader switches to a new cell or suffers an RLF, the previously configured resources may become invalid, necessitating resource reconfiguration. For another example, when UE reader detects that the configured A-IoT resources cannot accommodate the realistic A-IoT procedure, UE reader could notify the gNB and request more resources. Moreover, if a timer is set to limit the resource usage time during the initial configuration, and now the timer is about to expire while the A-IoT-related procedure is still ongoing, then UE reader may request additional duration for the invalid resource from NW or ask NW to reconfigure the resources. In conclusion, there are two options to trigger resource reconfiguration:

-	Option-1: Resource reconfiguration triggered by NW (e.g., gNB makes the HO decision).

-	Option-2: Resource reconfiguration triggered by UE reader (e.g., due to RLF, dynamic resource requirement for A-IoT interface or timer expiration).

Proposal 3: RAN2 will consider the reconfiguration of A-IoT radio resources:

-	Option-1: Resource reconfiguration triggered by NW (e.g., HO).

-	Option-2: Resource reconfiguration triggered by UE reader (e.g., due to RLF, dynamic resource requirement for A-IoT interface or timer expiration).

In addition to the visible information outlined in Proposal 1, UE reader could also provide other assistance information, such as the information about device response results. This information may include the number of received device responses, and any buffered A-IoT data (if the UE is required to aggregate data). This is because, for instance, if an inventory procedure involved one hundred devices and the UE reader has already received responses from eighty devices, it is reasonable to expect that the reconfigured resources should be less than those previously configured to avoid resource wastage. Thus, the gNB should reconfigure appropriate resource to the UE reader for the remaining A-IoT service.

Proposal 4: The gNB should also acquire assistance information about the device response results to facilitate resource reconfiguration. This information may include the number of received device responses, and any buffered A-IoT data (if the UE is required to aggregate data).

During resource reconfiguration, there are three options for the gNB to obtain the information about current device response:

-	Option-1: from another gNB.

-	Option-2: from CN.

-	Option-3: from UE reader.

For option 1, when UE reader switches to a new gNB as it moves, the A-IoT task context (including the A-IoT service completion status) should be transferred from the source gNB to the new gNB (e.g., via Handover Request). For option 2, if UE reader has forwarded the aggregated A-IoT data (invisible to gNB) to CN, CN could provide the information (e.g., estimated amount of data for the remaining A-IoT service) to assist in resource reconfiguration. For option 3, UE reader could directly report the A-IoT service completion status, e.g., via RRC signalling or BSR report.

Proposal 5: Three options for capturing the information regarding current device response: 1) from another gNB, 2) from the CN and 3) from the UE reader.

3 Conclusions

In this contribution, we analyzed the possible resource configuration options and made the following proposals:

Proposal 1: The following information should be visible to the gNB for resource configuration:

-	The service type of A-IoT (e.g., inventory, command). FFS if more information on command type (e.g., read/write/disable) is useful;

-	targeted for one or more than one device;

-	approximate number of target devices (if available);

-       message size for D2R and R2D transmissions over A-IoT air interface.

Proposal 2: The visible information could be delivered to gNB by either UE reader or CN.

Proposal 3: RAN2 will consider the reconfiguration of A-IoT radio resources:

-	Option-1: Resource reconfiguration triggered by NW (e.g., HO).

-	Option-2: Resource reconfiguration triggered by UE reader (e.g., due to RLF, dynamic resource requirement for A-IoT interface or timer expiration).

Proposal 4: gNB should also acquire assistance information about the device response results to facilitate resource reconfiguration. This information may include the number of received device responses, and any buffered A-IoT data (if the UE is required to aggregate data).

Proposal 5: Three options for capturing the information regarding current device response: 1) from another gNB, 2) from the CN and 3) from the UE reader.

4. References

[1]	RAN2#127bis Chairman notes

[2]	RAN2#127 Chairman notes