3GPP TSG-RAN WG2 Meeting #128                                                                   R2-2410693

Orlando, USA, Nov.  18th – 22nd , 2024

Agenda item:	8.2.3

Source:	China Telecom

Title:	Further discussions on Ambient IoT Paging

Document for:	Discussion

- Introduction

In the RAN#102 meeting, a new SID on solutions for Ambient IoT (Internet of Things) in NR was approved [1].

In RAN2#127, it was agreed that the reader could trigger multiple/subsequent A-IoT paging for the same service request from CN. The A-IoT paging message will include a short and simple information to avoid duplicate response for the same service request. But how to avoid devices duplicate response for the same service request is FFS.

In the last RAN2#127bis meeting[1], the following agreements were made.

Agreements

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information the device determines whether to skip sending the response to paging.

- From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

- If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

Based on the above progress,this paper mainly analyses the details of subsequent A-IoT paging and paging contents.

- Discussion

2.1 Multiple/Subsequent A-IoT Paging

In general, the current agreements assume that paging always requires a response. This is clearly appropriate for the basic paging use cases: The reader needs to inventory one or multiple devices, receive data from device(s), or establish communication for data exchange.  However, readers usually cannot reach all target devices within the coverage area through a single round of paging. The reason may be that some devices are not sufficiently charged, so they cannot respond to paging. Or there are a large number of devices, and there is contention for uplink resources when multiple devices attempt to respond. Therefore, for each CN service request for paging, the reader can initiate multiple paging message.

A paging round is the duration of a series of paging messages triggered by the same CN request. Each paging message may correspond to a paging round. To reduce complexity, subsequent paging messages in the same paging round have no difference from the initial trigger message.

In the previous discussion, how to trigger multiple/subsequent A-IoT paging is FFS. There are three options for implementing multiple/subsequent A-IoT paging as follows:

- Upon receiving the CN request, the reader configures the time to trigger subsequent A-IoT paging and the number of paging repetitions.

- The service request from CN includes periodic A-IoT paging configuration(e.g., length/duration of paging round and the number of paging repetitions). Based on the configuration information in the upstream node, the reader configures the time to trigger subsequent A-IoT paging and the number of paging repetitions.

- CN will trigger the multiple/subsequent A-IoT paging one by one and reader will perform this transmission upon receiving the paging trigger from upstream node.

Option 1 may not be possible since the reader usually usually is not aware of the information about the devices in the coverage area. Compared to option3, option2 may reduce CN requests and corresponding overhead by configuring the reader to perform A-IoT paging periodically. However, according to SA2, reader-triggered periodic paging requires a stop request from the CN to quit paging repetition, and any reconfiguration requires an additional CN request. Therefore, considering the simplification of the A-IoT protocol, it is proposed to let the CN request A-IoT paging on demand instead of configuring periodic paging.

It is proposed to let the CN request A-IoT paging on demand instead of configuring periodic paging.

2.2 Contents of A-IoT Paging Message

RAN2 has made the agreement that the following cases for A-IoT paging message will be considered:

- a message containing an ID of a single A-IoT device.

- a message containing a group ID that maps to multiple A-IoT devices.

- a message that does not contain an ID, i.e., addressed for all devices that can receive the A-IoT message.

- a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion.

RAN2 did not reach consensus on the need for multiple IDs and suggested to confirm the need for this use case based on SA2 discussion. However, there was no relevant discussion or agreements at the recent SA2 meetings.

The use of multiple IDs actually exists from both CN/AF and A-IoT air interface perspectives. For CN/AF, it may need to inventory or read/write a small number of specific devices that do not belong to any configured group. For A-IoT air interfaces, after multiple paging rounds, the reader may need to inventory several remaining unresponsive devices with discontinuous device IDs. For example, Each paging message is targeted at the same batch of devices, but different devices may not have opportunities to response the paging in a certain round of paging. In the case that multiple device IDs with discrete/discontinuous ID numbers are in the service request message and further need to be included A-IoT air interface messages, it would be a waste to send paging messages with the same device IDs list repeatedly. Considering that some devices may have succeed in previous rounds of paging, if this part of the successful devices is always included in the subsequent paging messages, it is unreasonable. Instead, it makes more sense that all devices that has responded successfully can be eliminated from the paging message of the next round of paging.

While depending on TB size it may not be possible to include multiple device IDs, given the above need, details can be further discussed in WI stage.

It is proposed to support paging message containing multiple IDs of A-IoT devices. The multiple IDs can be used to indicate multiple devices with discontinuous device IDs.

In RAN2 #126, A-IoT paging message can be targeted to a single device, a group of devices, or all devices. Different paging functions may be defined for the above cases. At least a 2-step and a 3-step random access procedures should be supported for “inverntory only”, “inverntory only” and “inventory and command” use cases, as discussed in RAN2.

As A-IoT access procedures are triggered by the reader, the reader provides the access resource and determines the random access type, considering different service types. If the reader only wants to send commands to a small number of A-IoT devices, it may choose contention free access type. If the reader only needs a simple confirmation back from the device after executing the command, it can select 2-step A-IoT access. For cases where a large number of A-IoT devices are inventoried using limited resources, the reader may instruct the A-IoT device to perform a 3-step or 4-step A-IoT access. Thus, the reader should explicitly indicate the type of A-IoT access to the A-IoT device in the A-IoT paging messages.

The type of A-IoT access (e.g., 2-step, 3-step, CFRA) can be explicitly indicated to the device via A-IoT paging messages.

According to SA2, one A-IoT service may be transferred to multiple A-IoT readers with a same higher layer session ID. For example, inventory towards the same bunch of devices may be passed to readers within a local range. From the device side, it is of no use to repeatedly respond to all the paging procedures induced by the same service. Additionally, storing session ID and recording response status on the device helps to avoid duplicated response and reduce power consumption.

Apart from that, since the possibility of the access failure still exists, the reader may trigger multiple rounds of paging corresponding to the same service via air interface. In this sense, the paged devices need to know whether it has successfully been paged and found. That is, if the device receives the paging identified by the same identifier as it responded to before, it will know that the current paging is another round of the one it has responded and accessed; and there is no need to respond again. Thus, the lower layer session ID should be introduced to resolve such power consumption issue from the device point of view. To include a session ID indicating that paging is brought by a specific service, device can respond at will.

Regarding the paging cause, the paging cause “Inventory” and “command” can be included. Commands are instructions (e.g., Read, Write, Disable) sent by an AF to an A-IoT device and are protected end-to-end between AF and A-IoT Device. Therefore, the AMF may include one or more command information in the paging message. Then, the Command Information as string type can be also included in the paging message.

In conclusion, the following information can be included in the A-IoT paging message:

- Access type (e.g., 2-step, 3-step, CFRA)

- UE paging ID (an ID, a group ID or multiple ID)

- Paging cause (inventory or command)

- Session ID

- NAS data: e.g., Command

The following information can be included in A-IoT paging message:

- Access type (e.g., 2-step, 3-step, CFRA)

- UE paging ID  (an ID, a group ID or multiple ID)

- Paging cause (inventory or command)

- Session ID

- NAS data: e.g., Command

- Conclusions

Based on the discussion in the previous sections we propose the following:

- It is proposed to let the CN request A-IoT paging on demand instead of configuring periodic paging.

It is proposed to support paging message containing multiple IDs of A-IoT devices. The multiple IDs can be used to indicate multiple devices with discontinuous device IDs.

The type of A-IoT access (e.g., 2-step, 3-step, CFRA) can be explicitly indicated to the device via A-IoT paging messages.

the following information can be included in A-IoT paging message:

- Access type (e.g., 2-step, 3-step, CFRA)

- UE paging ID  (an ID, a group ID or multiple ID)

- Paging cause (inventory or command)

- Session ID

- NAS data: e.g., Command

- References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

- R2-24095xx, Draft Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China, 14 - 18 October, 2024

- TR 23.700-13	Study on Architecture support of Ambient power-enabled Internet of Things