3GPP TSG-RAN WG2 Meeting #128	 R2-2410697

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	8.3.5

Source:	KDDI Corporation

Title:	Discussion on simulation assumptions for RLF prediction

SID:	FS_NR_AIML_Mob – Release 19

Document for: 	Discussion and decision

- Introduction

In RAN#103 meeting, a revised Rel-19 SID of ‘AI/ML for mobility in NR’ was approved [1]. The detailed objectives are as follows. In this contribution, we mainly discuss the simulation assumptions for RLF prediction.

The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - NOTE 1: RAN1/3 work can be triggered via LS - NOTE 2: RAN4 scope/work can be defined and confirmed by RAN#105 after some RAN2 discussions (within the RAN4 pre-allocated TUs) NOTE 3: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 4: Two-sided model is not included

- Discussion

Simulation assumptions for RLF/HO failure prediction

Agreements on simulation assumption at RAN2 #126 To agree not consider Oxygen absorption (7.6.1), Time-varying Doppler shift (7.6.6), Explicit ground reflection model (7.6.8) and blockage (7.6.4) for channel modelling (38.901)

At the meeting, it was agreed that the Blockage model would not be considered as a simulation assumption for RRM measurement prediction evaluation[2]. The agenda of RAN2#127 was written that it should focus on discussing RLF specific methodology and simulation assumptions (addressing the differences or additional aspects from RRM prediction assumptions).

Draft summary of [POST127bis][022][AI mobility] Simulation Assumptions (OPPO) - 2.2 RLF event prediction 2.2.3 Simulation assumption Apart from baseline simulation assumptions, more parameters are proposed at RAN2@127meeting. Among the contributions, there are 5 kinds of simulation parameters: Case 1: parameters related to channel model e.g. channel blockage [1] [12] Case 2: parameters related to RLF procedure [1] [9] [12] [8][7] Case 3: parameters related to HO procedure [8] Case 4: parameters related to UE’s characteristics e.g. speed, trajectory etc. [10] Case 5: Interference related parameters [1] Parameters in case 3 can be saved because RAN2 agreed to simulate RLF without HO procedure. Parameters in case 4 can be also saved because company can report UE’s characteristics parameters, which is already in the template of the spreadsheet after [POST127bis][016][AI Mob] Simulation results (Mediatek). The intention for parameters in case 1 is to trigger RLF event easier. But now RAN2 agreed that UE will not change serving cell after it is dropped, triggering RLF event is not difficult any more. Considering company would like to reuse existing data set as much as possible, no change of such parameter for channel model is preferred. Observation 2: parameters in case 1 case 3 and case 4 can be saved.

We discuss controversial aspects of email discussion or on aspects not covered in email discussion related to simulation assumptions for RLF prediction.

As you see above draft summary, case1: parameters related to channel mode e.g. channel blockage is controversial aspect and not covered parameters of it. Some companies suggested to consider channel blockage in the channel model in order to collect more RLF event.

Observation 1: Case 1: parameters related to channel model e.g. channel blockage is a controversial aspect.

When evaluating the ability to predict RLF, special attention should be given to the phenomenon of blockage attenuation, particularly in the context of FR2. These higher frequencies are more susceptible to blockage by physical obstacles (e.g., buildings, trees, human bodies), leading to sudden drops in signal quality, known as blockage attenuation. Blockage model can be used in simulations of TS38.901[2].

The followings are the importance of considering blockage attenuation in simulations and evaluations for a couple of key reasons:

High Impact on FR2: Due to their higher frequency, FR2 signals have shorter wavelengths, making them more likely to be blocked by obstacles. This can lead to more frequent and severe signal degradation compared to lower frequency bands (FR1). Thus, accurately predicting when and where blockage attenuation might occur is crucial for maintaining network performance and reliability in FR2 deployments.

Enhancing Predictive Models with AI/ML: Incorporating blockage attenuation into AI/ML models allows these systems to better predict potential unintended events such as sudden signal losses or degradation. By understanding where blockage is likely to occur, the network can proactively adjust, for instance, by switching beams or rerouting connections to minimize the impact on users.

Improving User Experience and Network Efficiency: By accurately predicting and mitigating the effects of blockage attenuation, networks can provide more stable and reliable connections, especially in high-mobility scenarios. This leads to improved user experience through fewer dropped connections and higher data rates. Additionally, network resources can be utilized more efficiently, as the system can pre-emptively address potential issues before they affect users.

In conclusion, it was agreed that the channel model for RRM measurement prediction does not consider blockage. However, in RLF prediction simulations, particularly in FR2, physical obstacles (such as buildings, trees, and human bodies) can cause significant interference, leading to a sudden drop in signal quality known as blocking attenuation. Therefore, blockage is necessary to introduce abrupt changes in the radio link. Considering the evaluation workload, it is acceptable to allow the blockage model as an option.

Proposal 1: RAN2 should consider blockage in channel modelling to evaluate RLF for AI/ML aided mobility benefits particularly in FR2.

Proposal 2: RAN2 should allow the blockage model as an option considering the evaluation workload.

Proposal 3: If this option is acceptable, a check box for the blockage model will be added to the table summarizing each company's results.

Proposal 4: Case 1: parameters related to channel mode e.g. channel blockage can be saved to compare the simulation results of companies.

Two alternative model (model A and model B) for the blockage (7.6.4) for channel modelling (38.901)

should be chosen for simulation assumption for RLF to compare the simulation results from companies. Model A is applicable when a generic and computationally efficient blockage modelling is desired. Model B is applicable when a specific and more realistic blocking modelling is desired [3].

Proposal 5: Model A can be selected considering the workload. (Clause 7.6.4 of TR 38.901).

Proposal 6: For Blockage model A option of RLF prediction, unify required parameters of Blockage model A, or determine the rules for describing parameters.

Blockage model A has several configurable parameters. To unify the evaluation conditions, companies who plan to apply blockage model should provide their opinion on each parameter.

Proposal 7: For Blockage model A option of RLF prediction, companies input their opinion on the following configurable parameters.

    - The number of blocker K (the standard number is K = 4).

    - Scenarios for deciding non-self-blocking regions parameters given in Table 7.6.4.1-2 of TS 38.901.

    - Scenarios for deciding spatial correlation distance given in Table 7.6.4.1-4 of TS 38.901.

    - Whether the blocker has movement speed v.

    - Are there any other missing configurable parameters?

- Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1: Case 1: parameters related to channel mode e.g. channel blockage is a controversial aspect.

Proposal 1: RAN2 should consider blockage in channel modelling to evaluate RLF for AI/ML aided mobility benefits particularly in FR2.

Proposal 2: RAN2 should allow the blockage model as an option considering the evaluation workload.

Proposal 3: If this option is acceptable, a check box for the blockage model will be added to the table summarizing each company's results.

Proposal 4: Case 1: parameters related to channel mode e.g. channel blockage can be saved to compare the simulation results of companies.

Proposal 5: Model A can be selected considering the workload. (clause 7.6.4 of TR 38.901).

Proposal 6: For Blockage model A option of RLF prediction, unify required parameters of Blockage model A, or determine the rules for describing parameters.

Proposal 7: For Blockage model A option of RLF prediction, companies input their opinion on the following configurable parameters.

    - The number of blocker K (the standard number is K = 4).

    - Scenarios for deciding non-self-blocking regions parameters given in Table 7.6.4.1-2 of TS 38.901.

    - Scenarios for deciding spatial correlation distance given in Table 7.6.4.1-4 of TS 38.901.

    - Whether the blocker has movement speed v.

    - Are there any other missing configurable parameters?

References

[1] RP-240082 	Revised SID on AIML for mobility in NR (OPPO)

[2] R2_126_ChairNotes_24-05-24_13-10_eom

[3] TR 38.901 	Study on channel model for frequencies from 0.5 to 100 GHz.