3GPP TSG-RAN WG2 Meeting #128	R2-2410700

Orlando, USA, Nov 18th – 22th, 2024

Agenda item	:		8.8.4

Source	: 	Sharp

Title	: 	Discussion on UE behaviours within intended service area

Document for	:	Discussion and Decision

Introduction

In the last RAN2 #127bis meeting, RAN2 reached some agreements/decisions as follows [1];

Agreements: - For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

In this paper, we’d like to discuss UE behaviours for MBS related procedures.

Discussion

DL UE Behaviour inside the Intended Service Area

[MCCH re-acquisition considering intended service area]

In the RAN2#127bis meeting [1], it was agreed that RAN2 consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

If both the intended service area information and this association information is included in SIB, UE can consider whether UE is within intended service area or not without re-acquiring MCCH. However, it was agreed that the intended service area IDs are included in MCCH, and it means the association information between the MBS session and the intended service area would be included in MCCH. In this case, it is difficult for UE to consider whether UE is within intended service area or not without re-acquiring MCCH.

Therefore, in this clause, we'd like to analyse the UE behaviour for MCCH re-acquisition in a case the service area information is included in MCCH.

When the MBS session information is updated, the association information between the updated MBS session and the intended service area should also be updated. In addition, when the intended service area information is updated, the information that links the session information to the intended service area should also be updated. In other words, any changes of the MCCH information may occur when either the session information or the intended service area information is updated, e.g. in the following cases:

- Intended service area information is updated, for example, when intended service areas are out of cell range coverage especially in the case of Earth Moving Cells, or when some areas are no longer available due to the changed circumstances.

- The case of updating session information, for example, when an MBS service provider updates the area of the service.

Observation 1 : If the association information between the service area and the MBS session is included in the message carried by MCCH, since session information is associated with intended service area, MCCH modification is occurred by any changes of either intended service area or MBS session information.

In RAN2#127 meeting [2], it was agreed that the UE may not need to (re)acquire up-to-dated MCCH when the UE is not in any intended service area of its interested broadcast services.

In the current specification, if the NW sends an 'MCCH modification indication' to the UE when there are any changes for MCCH, the UE initiates the MCCH acquisition procedure based on this indication, and this is not considering whether UE is inside or outside the intended service area. This 'MCCH modification indication' related procedure is described in TS38.331[2] and TS38.212[3], and 2-bit bitmap information in DCI format 4_0 is used to indicate the change of the MCCH.

The 2-bit bitmap indicates the following information.

(bit1) the start of new MBS service(s)

(bit2) modification of MCCH information other than the change caused by start of new MBS service(s).

When applying the current mechanism to Rel-19 NTN MBS, UE which is not in any intended service area of its interested broadcast services can consider whether MCCH re-acquisition is needed or not based on this second bit (bit2), because this UE needs updated information not for session start but only for modification related to ISA.

Observation 2 : In the current specification, the UE initiates the MCCH acquisition procedure based on the MCCH modification indication regardless of whether UE is in any intended service areas or not.

Observation 3 : The MCCH modification indication can be used to avoid the unnecessary MCCH re-acquisition operations.

However, the second bit (bit2) indicates other causes than the change related to intended service area. Therefore, UE which is not in any intended service area of its interested broadcast services needs to acquire MCCH even if the change is not related to intended service area. So, we think RAN2 should discuss whether MCCH modification indication needs to be extended to differentiate the change related to intended service area from other causes to avoid unnecessary MCCH re-acquisition operations outside the intended service area.

Proposal 1 : RAN2 should discuss whether MCCH modification indication needs to be extended to differentiate the change related to intended service area from other causes to avoid unnecessary MCCH re-acquisition operations outside the intended service area.

[MCCH signalling]

At the RAN2#126 meeting [4], RAN2 agreed to support both EFC and EMC for MBS Broadcast. This means that solutions need to take into account the potential problems when SIBxx and/or MCCH-messages include intended service area information.

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

Observation 4 : Need to consider how to notify Intended Service Area, taking into account both EFC and EMC.

In the case of the EMC scenario, the cell coverage of the NTN-Cell is constantly moving across the earth's surface. At this time, there will be a change in the information on the intended service area supported by the NTN-Cell, which can be served, and there is also a change in the information on the intended service area where the NTN-Cell can provide the MBS broadcast service.

Observation 5 : In the case of an Earth-moving Cell, the information of the intended service area (number of areas, ID, allocation information, etc.) associated with each session broadcasted in that NTN-Cell can update frequently.

If the intended service area information is included in SIB and the association information between the MBS session and the intended service area is included in MCCH, the UE cannot know that there are any updates related to the service area without re-acquiring MCCH. Therefore, when the UE receives updated SIB including the intended service area information, UE also needs to re-acquire an MCCH-message to update the association information for the session based on the 'MCCH modification indication'.

Observation 6 : If the intended service area is included in the SIB, the UE also needs to re-acquire the MCCH-message to update the association information for the sessions.

In contrast, if ISA information is included in the MCCH message, i.e. in the MBSBroadcastConfiguration, changes in MCCH information can be flexibly indicated to UE by modification indications. This makes it possible to skip re-acquisition the MCCH message depending on the UE implementation. However, the current specification requires the full intended service area information to be acquired even if there are changes other than intended service area information, in the case of the intended service area information is included in the MBSBroadcastConfiguration.

Observation 7 : If intended service area is included in MBSBroadcastConfiguration, UE needs to receive the whole information even if the changes are other than the change of intended service area information.

The current MCCH structure is optimized for the legacy (i.e. TN-gNB) MBS service. In other words, the MCCH structure for UEs supporting NTN MBS Broadcast could be extended to support the intended service area. Then, to avoid the UE having to retrieve all the information every re-acquisition of MCCH-message, we think it could be considered to indicate the intended service area information as a separate MCCH message instead of including it in the MBSBroadcastConfiguration. It is reasonable from the perspective of the amount of data received.

Thus, we would like to propose as below :

Proposal 3 : To avoid the UE having to retrieve all the information every re-acquisition of MCCH-message, RAN2 consider to introduce a separate MCCH message than MBSBroadcastConfiguration for the intended service area information.

UL UE Behaviours considering the Intended Service Area

In RAN2#127 meeting, RAN2 agreed UE behaviours for MRB establishment/release.

- For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

- For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

There are some other remaining MBS related procedures (e.g. MII (MBS interest indication), cell reselection priorities handling). If these procedures are performed by UEs which are not intended to receive the service, there are negative impacts for NW operation, especially if UL resources are used by the procedure. For example, if UE is allowed to send MII from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the service. To prevent such negative impacts, the specification should also limit UE behaviours for these MBS related procedures. Therefore, if the MBS service is associated to the intended service area(s), MII should not be sent by UE located outside of the intended service area.

Observation 8 : If UE is allowed to send MBSInterestIndication message from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the MBS service.

Proposal 4 : MBSInterestIndication message should not be sent by UE located outside of the intended service area.

Conclusion

In this paper, we made the following observations and proposals;

[DL/MCCH re-acquisition without intended service area]

Observation 1 : If the association information between the service area and the MBS session is included in the message carried by MCCH, since session information is associated with intended service area, MCCH modification is occurred by any changes of either intended service area or MBS session information.

Observation 2 : In the current specification, the UE initiates the MCCH acquisition procedure based on the MCCH modification indication regardless of whether UE is in any intended service areas or not.

Observation 3 : The MCCH modification indication can be used to avoid the unnecessary MCCH re-acquisition operations.

Proposal 1 : RAN2 should discuss whether MCCH modification indication needs to be extended to differentiate the change related to intended service area from other causes to avoid unnecessary MCCH re-acquisition operations outside the intended service area.

[DL/MCCH signalling]

Observation 4 : Need to consider how to notify Intended Service Area, taking into account both EFC and EMC.

Observation 5 : In the case of an Earth-moving Cell, the information of the intended service area (number of areas, ID, allocation information, etc.) associated with each session broadcasted in that NTN-Cell can update frequently.

Observation 6 : If the intended service area is included in the SIB, the UE also needs to re-acquire the MCCH-message to update the association information for the sessions.

Observation 7 : If intended service area is included in MBSBroadcastConfiguration, UE needs to receive the whole information even if the changes are other than the change of intended service area information.

Proposal 3 : To avoid the UE having to retrieve all the information every re-acquisition of MCCH-message, RAN2 consider to introduce a separate MCCH message than MBSBroadcastConfiguration for the intended service area information.

[UL/ UE behaviour]
Observation 8 : If UE is allowed to send MBSInterestIndication message from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the MBS service.

Proposal 4 : MBSInterestIndication message should not be sent by UE located outside of the intended service area.

Reference

- Draft_RAN2_127b_Meeting_Report_v2

- R2-2407902, Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China

- 3GPP TS 38.331 V18.2.0 (2024-06)

- 3GPP TS 38.212 V18.3.0 (2024-06)

- R2-2406202, Report of 3GPP TSG RAN WG2 meeting#126, Fukuoka, Japan