3GPP TSG-RAN-WG2 Meeting #128	R2-2410701

Orlando, USA,18th– 22nd Nov, 2024

Agenda item	:		8.6.4

Source	: 	Sharp

Title	: 	Discussion on Conditional LTM

Document for	:	Discussion and Decision

Introduction

Revised WID [1] for enhancement mobility was approved and RAN2 started discussion on the WI. For conditional LTM, RAN2 starts discussion from this meeting;

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In the last meeting, we agreed below,

1.	Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

2.	Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

3.	Source cell and each candidate cell provides its own execution condition for conditional LTM.

4.	It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

5.	RACH-less Conditional intra-CU LTM is supported.

6.	RACH based conditional intra-CU LTM is supported.

7.	UE based TA measurement mechanism is supported for conditional intra-CU LTM.

8.	PDCCH ordered early TA acquisition is supported for conditional LTM.

9.	Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

10.	For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

11.	The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

In this paper, we discuss some issued related to conditional LTM.

Discussion

Preparation phase

In the last meeting, RAN2 agreed that EventLTM3-like and EventLTM5-like are used for evaluation for C-LTM.

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

In addition, it is needed to discuss whether CHO conditions (i.e. cell level conditions) are reused or not. In RAN#127, for event triggered measurement, it is agreed that evaluation is performed as beam level, not cell level. In our understanding, C-LTM should be based on these event triggered measurement mechanisms, so evaluation for C-LTM should be performed at the same level (i.e. beam level). In addition, beam level evaluation for event triggered measurement is performed in PHY/MAC layer and cell level evaluation is performed in RRC layer. However, if cell level evaluation for C-LTM in PHY/MAC layer is supported, system would become complicated. Therefore, we propose that CHO conditions are not reused for C-LTM.

Proposal 1. CHO conditions are not reused for C-LTM.

Early sync phase

Early DL sync

In the last meeting, RAN2 agreed that Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. Since gNB can configure UE to perform measurement report to determine which TCI states to activate, even if it is conditional LTM, TCI state activation/deactivation MAC CE should be used as with Rel-18 LTM.

- Proposal 2. TCI state activation/deactivation MAC CE in Rel-18 LTM can be reused for conditional LTM .

On the other hand, since UE will perform cell switch by itself according to the measurement result and the configured conditions, UE can perform early DL synchronization without waiting for the TCI state activation/deactivation MAC CE. Thus, UE can automatically determine/activate a TCI state for the target cell based on the measurement results.

Proposal 3. RAN2 considers autonomous TCI activation/deactivation performed by UE for conditional LTM.

Early UL sync

In the last meeting, RAN2 agreed that PDCCH order early TA acquisition and UE based TA measurement mechanism are supported to early UL synchronization. In addition, source gNB doesn’t need to send PDCCH order to UE if UE performs RA procedure for early synchronization based on the LTM configuration itself (e.g., UE selects preamble index/SSB index for candidate cells based on RRC configuration), i.e. RRC-based RA procedure for early synchronization. The main difference between RRC-based RA procedure and PDCCH order-based RA procedure is whether UE needs to receive PDCCH order or not. RRC-based RA procedure can benefit from low latency because the UE does not need to wait for the PDCCH order and performs early UL synchronization according to the LTM candidate configuration. Therefore, RRC-based RA procedure for early synchronization should be considered.

Proposal 4. RRC-based early TA acquisition mechanism for early synchronization can be considered for conditional LTM.

However, for both of PDCCH order-based and RRC-based RA procedure for early synchronization, how to receive TA information for conditional LTM should be further discussed since TA value is provided in LTM CSC MAC CE in Rel-18 LTM, and we assume that LTM CSC MAC CE is not used for conditional LTM cell switch. Therefore, UE needs to calculate TA value by itself or to be provided with TA value by gNB before cell switching. In Rel-18, RAN2 has discussed RAR-based TA acquisition option for a few meetings. In RAR-based TA acquisition, there are two ways to receive RAR from a source cell and from a candidate cell. Hence, we think RAR-based TA acquisition can be reconsidered for conditional LTM.

Proposal 5. For conditional LTM cell switch, LTM CSC MAC CE in Rel-18 LTM is not used.

Proposal 6. Support of RAR-based early TA acquisition for conditional LTM.

For early UL synchronization, NW can determine to send a PDCCH order to the UE based on the measurement report from that UE. However, in the measurement report, there may be more than one beams for multiple cells satisfying the events if it is an event-triggered report, NW cannot know the specific cell that the UE may switch to if UE can decide the target cell to switch to. Furthermore, if it is not an event-triggered report, NW cannot justify which cell the UE will switch to even though the candidate cell with the highest L1-RSRP value. Thus, in addition to based on the measurement report, a cell switch request (e.g., a cell ID) from UE can be transmitted to assist gNB to trigger PDCCH order for certain candidate cells, and thus gNB can have a specific target without providing PDCCH order to every candidate cell.

Proposal 7. A cell switch request from UE can be transmitted to assist gNB to trigger PDCCH order for certain candidate cells if UE can decide the target cell to switch to.

Evaluation phase

In Rel-18 LTM, UE starts L1 measurement and reporting when L1 measurement is configured in RRC reconfiguration. And gNB determines whether to trigger LTM cell switch for the UE based on the L1 measurement reports. In case of CHO, when UE is configured with CHO configuration, UE starts evaluating event condition of conditional reconfiguration. Therefore, in the same way, UE should start conditional LTM event evaluation upon UE is configured with C-LTM configuration.

Proposal 8. UE starts conditional LTM event evaluation once it is configured with C-LTM configuration.

And, just as the event evaluation mechanism for CHO should be aligned with event triggered L3 measurement report, the event evaluation mechanism for C-LTM should be aligned with event triggered L1 measurement report. This helps reduce the workload and keeps the specifications simple.

Proposal 9. The event evaluation mechanism for C-LTM should be same with the event evaluation mechanism for event-triggered L1 measurement reporting.

Execution phase

Upon LTM cell switch execution, UE can switch the serving cell with/without random access procedure. For RACH-based LTM cell switch, there are two type of RACH procedure, i.e. CFRA and CBRA. In Rel-18 LTM, source DU determines to trigger LTM cell switch and notifies it to target DU if source DU and target DU are not the same DUs. Hence, target gNB (target DU) can prepare dedicated resource for the UE when CFRA is needed. Using CFRA is useful for reducing communication interruption period. On the other hand, if CFRA is used for C-LTM, all of candidate cells need to reserve CFRA resources for the UE for the entire duration that C-LTM is configured. However, in case of CHO, both of CBRA and CFRA are supported. It means that the network can tolerate some waste of CFRA resources. So, in the same way, both CBRA and CFRA should be supported for conditional LTM.

Observation 1. In case of CHO, both of CBRA and CFRA are supported.

Proposal 10. Both CBRA and CFRA should be supported for conditional LTM.

For RACH-less cell switch, we assume that legacy mechanism will be used. It means that UE firstly sends RRCReconfigurationComplete message after the switch as with Rel-18 LTM when UE performs RACH-less LTM cell switch.

Proposal 11. When the UE executes a RACH-less LTM cell switch, it first transmits the RRCReconfigurationComplete message following the switch, as is customary in a Rel-18 LTM procedure.

Furthermore, there is an issue on how to handle UL grant to complete LTM cell switch procedure. In Rel-18 LTM, an UL grant is provided by configured grant or dynamic grant. In case of CG-based RACH-less cell switch, UE is configured with configured grant to be used for target cell by RRC signalling, and the UE sends complete message by using a beam which is indicated in the LTM CSC MAC CE. Target cell will prepare UL resource after source DU indicates the UE’s switch to the target DU. In case of C-LTM, if this mechanism is applied to UE, whose cell switch timing is unknown, the candidate cells must constantly reserve UL resources for the UE. In case of DG-based RACH-less cell switch, UE monitors PDCCH on the target cell and sends complete message upon receiving PDCCH for DG. Then, the target cell will prepare UL resource after source DU indicates the UE’s switch to the target DU. Similarly, in the case of DG-based, target cell will prepare UL resource and send PDCCH to the UE after source DU indicates the UE’s switch to the target DU. In case of C-LTM, if this mechanism is applied to UE, whose cell switch timing is unknown, the candidate cells must constantly reserve UL resources for the UE. In both cases configured grant and dynamic grant, it is not reasonable to consistently reserve UL grant for the UE, whose cell switch timing is unknown.

Observation 2. To perform RACH-less LTM cell switch, gNB needs to provide UL grant so as UE to send an RRCReconfigurationComplete message.

Observation 3. It is not reasonable to continue providing Dynamic/Configured grant for the UE, whose cell switch timing is unknown.

The key point is that target cell (DU) knows when UE switches the serving cell. If target cell (DU) knows when UE switch the serving cell, the target can reserve the UL grant depending on when the UE switches the serving cell (as mentioned above). Here, we propose two solutions. One of them is UE notifies execution of conditional LTM cell switch to source cell after deciding to execute the switch and before executing the switch. Another one is the target provides UL grant in early TA acquisition procedure (triggered by PDCCH order).

Figure 1. Notification of C-LTM cell switch (Alt1)

Figure 2. RAR with UL grant (Alt2)

For Alt1, when the event conditions are met, the UE can perform RACH-less LTM cell switch if the UE is DL/UL synchronized with target cell. UE needs UL grant towards the source for the notification, but it seems no problem based on the consideration of LTM use case i.e. LTM is the mobility mechanism of the UE where communication occurs continuously and it is desirable to have little interruption.

For Alt2, it needs RA preamble transmission and RAR reception after the event condition is met. It means that the UE performs early sync, cell switch and notification to the target simultaneously after UE receives PDCCH order. This scenario is feasible but it is not desirable because the RA procedure is required after the event condition is met.

Proposal 12. To support RACH-less LTM cell switch, RAN2 discusses which procedure should be considered as baseline of conditional LTM;(target DU may or may not be source DU)

Alternative 1. UE notifies that UE will perform LTM cell switch to the source DU just before the LTM cell switch, source DU notifies it to target DU, and target DU ensures UE has UL grant.

Alternative 2. UE performs event triggered measurement report to source DU for conditional LTM, source DU triggers early synchronization by PDCCH order, and target DU ensures UE has UL grant.

Configuration

Furthermore, legacy LTM configuration is not maintained as UE variables. On the other hand, configurations for conditional reconfiguration are managed as UE variable. If RAN2 follows CHO method, RAN2 needs to introduce a new UE variable for conditional LTM. If RAN2 introduce a new UE variable, UE/gNB needs to manage the entries for conditional LTM separately from the entries for the normal LTM, the interactions between the entries for conditional LTM and the entries for the normal LTM will be complicated and CSI measurement configuration will become more complicated. Therefore, the entries for conditional LTM are managed in ltm-Config which maintains Rel-18 LTM configuration.

Proposal 13. The entries for conditional LTM is managed in ltm-Config which maintains Rel-18 LTM configuration.

Subsequent C-LTM

The intention of Rel-18 subsequent LTM is to avoid frequent RRC configuration upon cell switch. The definition of subsequent LTM in Rel-18 is “Subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without releasing other LTM candidate configurations after each LTM cell switch completion” in [2]. Since Rel-18 only supports the subsequent intra-CU LTM and now Rel-19 subsequent C-LTM focuses only on intra-CU cases, we propose that the definition concept of Rel-18 subsequent LTM can be reused for Rel-19 subsequent intra-CU C-LTM with the revision as “Subsequent intra-CU C-LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without releasing other C-LTM candidate configurations after each C-LTM cell switch completion”.

Observation 4. The definition concept of Rel-18 subsequent LTM can be reused for Rel-19 subsequent intra-CU C-LTM since Rel-18 LTM focuses on intra-CU cases.

Proposal 14. The definition of Rel-19 subsequent intra-CU C-LTM is “Subsequent intra-CU C-LTM is done by repeating the early synchronization, C-LTM cell switch execution, and C-LTM cell switch completion steps without releasing other C-LTM candidate configurations after each C-LTM cell switch completion.”

Conclusion

Proposal 1. CHO conditions are not reused for C-LTM.

- Proposal 2. TCI state activation/deactivation MAC CE in Rel-18 LTM can be reused for conditional LTM .

Proposal 3. RAN2 considers autonomous TCI activation/determination performed by UE for conditional LTM.

Proposal 4. RRC-based early TA acquisition mechanism for early synchronization can be considered for conditional LTM.

Proposal 5. For conditional LTM cell switch, LTM CSC MAC CE in Rel-18 LTM is not used.

Proposal 6. Support of RAR-based early TA acquisition for conditional LTM.

Proposal 7. A cell switch request from UE can be transmitted to assist gNB to trigger PDCCH order for certain candidate cells if UE can decide the target cell to switch to.

Proposal 8. UE starts conditional LTM event evaluation once it is configured with C-LTM configuration.

Proposal 9. The event evaluation mechanism for C-LTM should be same with the event evaluation mechanism for event-triggered L1 measurement reporting.

Proposal 10. Both CBRA and CFRA should be supported for conditional LTM.

Proposal 11. When the UE executes a RACH-less LTM cell switch, it first transmits the RRCReconfigurationComplete message following the switch, as is customary in a Rel-18 LTM procedure.

Proposal 12. To support RACH-less LTM cell switch, RAN2 discusses which procedure should be considered as baseline of conditional LTM;(target DU may or may not be source DU)

Alternative 1. UE notifies that UE will perform LTM cell switch to the source DU just before the LTM cell switch, source DU notifies it to target DU, and target DU ensures UE has UL grant.

Alternative 2. UE performs event triggered measurement report to source DU for conditional LTM, source DU triggers early synchronization by PDCCH order, and target DU ensures UE has UL grant.

Proposal 13. The entries for conditional LTM is managed in ltm-Config which maintains Rel-18 LTM configuration.

Proposal 14. The definition of Rel-19 subsequent intra-CU C-LTM is “Subsequent intra-CU C-LTM is done by repeating the early synchronization, C-LTM cell switch execution, and C-LTM cell switch completion steps without releasing other C-LTM candidate configurations after each C-LTM cell switch completion.”

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- TS 38.300 V18.1.0 (2024-05), 5G; NR; NR and NG-RAN Overall description; Stage-2