3GPP TSG-RAN-WG2 Meeting #128	R2-2410706

Orlando, USA, 18th– 22nd Nov, 2024

Agenda item	:		8.13.4

Source	: 	Sharp

Title	: 	discussion on service continuity for multi-hop relay

Document for	:	Discussion and Decision

Introduction

In RAN#103 meeting, the new WID was approved [1];

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

And RAN2 starts discussion on service continuity for path switch of multi-hop relay

In this paper, we discuss some potential issues and solutions for it.

Discussion

According to the WID, RAN2 needs to discuss (A) framework of discovery and (re)selection, (C) SRAP and QoS handling, and (D) C-plane procedure. And RAN2 needs to consider forward compatibility while RAN2 discusses one additional hop relay. Considering future compatibility, the same term (e.g. intermediate relay UE) should be used for the new relays (both for first/second relay UEs), but for convenience, we will use the same term as the WID in this document.

In this document, we consider multi-hop systems as following the requirements specified in TS 22.261[2] (the list of KPIs is provided in the Appendix as an example). And we will consider the TR23.700-03[3] as baseline of the discussion.

Proposal 1. RAN2 will take into account the requirement specified in TS22.261 and the solutions and conclusion in TR23.700-03.

In the WID, it is assumed that the all UEs are in RRC_CONNECTED state before and after forming a multi-hop PC5 link. And candidate first relay UE is in RRC_CONNECTED state via indirect connection. Therefore, the candidate first relay UE is a remote UE in RRC_CONNECTED state.

Proposal 2. RAN2 considers the candidate first relay UE is a multi/single-hop U2N remote UE in RRC_CONNECTED state.

  - mechanism enhancement for Scenario A and B

Scenario A and B are below;

- Intra-gNB multi-hop indirect to direct path switching using existing framework

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

Firstly, we assume following procedure can be baseline of scenario A based on current procedure and SA2’s conclusions.

Figure 1. multi-hop to direct path switching procedure

First, RAN2 needs to decide an overall procedure to discuss details. So, we propose that figure1 should be considered as baseline for the M2D (multi-hop indirect to direct) path switching procedure.

Proposal 3. Figure1 should be considered as baseline for the M2D (multi-hop indirect to direct) path switching procedure (scenario A).

For the measurement reporting event, Event X1 (Serving L2 U2N Relay UE becomes worse than threshold1 and NR Cell becomes better than threshold2) can be used for this scenario since this path switching scenario is switching from source relay UE to target gNB. And event X2 (Serving L2 U2N Relay UE becomes worse than threshold) may also be used for it.

Proposal 4. Current measurement reporting event X1 and X2 can be reused for M2D (multi-hop indirect to direct) path switching procedure to report candidate relay UEs.

Secondary, we assume following procedure can be baseline of scenario B.

Figure2-a. Intra-gNB multi-hop indirect to single-hop indirect path switching by remote UE

Figure2-b. Intra-gNB multi-hop indirect to single-hop indirect path switching by first relay UE

For scenario B, we can consider two procedures, one is the procedure which the remote UE switches the path as shown in Fig.2-a, another one is the procedure which the first relay UE switches the path while keeping the path with the remote UE as shown in Fig.2-b. The procedure shown in Fig.2-a is used, for example, in case of public safety, and the procedure shown in Fig.2-b is used, for example, in case of wearable. Furthermore, most of the steps in the fig.2-b are supported by legacy I2D (indirect to direct) path switching procedure. Therefore, RAN2 should consider these two procedures for scenario B.

Proposal 5. Figure 2-a and Figure 2-b should be considered as baseline for the M2I (multi-hop indirect to single-hop indirect) path switching procedure (scenario B).

Another point of contention, in fig.2-a, is whether the target last relay UE and the source last relay UE can be the same UE. This type of case occurs when relay UEs are arranged in a straight line, and such a case is included in Ref. 4 as one of the use cases. Also, even if the target last relay UE were the source last relay UE, the gNB would know that based on the measurement report. Therefore, there is no problem at this time with the target last relay UE being the source last relay UE.

Proposal 6. The target last relay UE and the source last relay UE can be the same UE

We discuss measurement report event based on Fig.2-a since measurement report event for Fig.2-b is already supported in Rel-17. For the measurement reporting event, Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold) can be used for this scenario since this path switching scenario is switching to target relay UE. And event Z1 (Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2) can also be used for this switch. If the target last relay UE is source last relay UE, this event may not be used since the first relay UE may be closer than the last or other intermediate relay UE from remote UE’s perspective. For this case, Y2 should be configured. However, if the target last relay UE is different from source last relay UE Remote UE, this event (Z1) can be used for it since the reporting event having two thresholds is useful to avoid unnecessary reporting. It is up to gNB to decide how to configure Y2 and/or Z1. Also, event X1 can be used for this scenario.

Proposal 7. Current measurement reporting event X2, Y2 and Z1 can be reused for M2I path switching procedure to report candidate relay UEs.

  - . mechanism enhancement for Scenario C and D

Scenario C and D are below;

- Intra-gNB direct to multi-hop indirect path switching

- Intra-gNB single-hop indirect to multi-hop indirect path switching

The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

Following procedures are example of the switch procedures of scenario C.

Figure 3. Intra-gNB direct to multi-hop indirect path switching procedure

First, RAN2 needs to decide an overall procedure to discuss details. So, we propose that figure3 should be considered as baseline for the D2M (direct to multi-hop indirect) path switching procedure.

Proposal 8. Figure 3 should be considered as baseline for the D2M (direct to multi-hop indirect) path switching procedure (scenario C).

For the measurement reporting event, Event Y1 (PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2) and Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold) can be used for this scenario since this path switching scenario is switching to target relay UE from source PCell.

Proposal 9. Current measurement reporting event Y1 and Y2 can be reused for D2M path switching procedure to report candidate relay UEs.

For scenario D, following procedures can be a baseline.

Figure 4-a. Intra-gNB single-hop indirect to multi-hop indirect path switching procedure by remote UE

Figure 4-b. Intra-gNB single-hop indirect to multi-hop indirect path switching procedure by relay UE

To clarify the scenario D, we consider whether the source last relay UE can be the target first relay UE. In a U2N relay use case, the remote UE may be a low-power device, e.g. wearable. In this case, remote UE may move with the source last relay UE. Therefore, there exists a scenario where the source last relay UE can be the target first relay UE and also be a remote UE of the target last relay UE. Not covering this scenario would limit the use cases of Rel-19 multi-hop relay.

Figure 5. One possible case of scenario D

Proposal 10. RAN2 confirm that the source last relay UE can be the target first relay UE and also be a remote UE of the target last relay UE.

We illustrated the case that the source last relay UE is the target first relay UE in fig.4-b. In fig.4-b, most of the procedures are supported by legacy D2I (direct to indirect) path switching procedure. If P4 is confirmed, RAN2 should consider fig. 4-a/4-b as baseline of scenario D.

Proposal 11. Figure 4-a/4-b should be considered as baseline for the I2M (single-hop indirect to multi-hop indirect) path switching procedure (scenario D).

For the measurement reporting event, Event X2 (Serving L2 U2N Relay UE becomes worse than threshold), event Y2 (Candidate L2 U2N Relay UE becomes better than threshold) and event Z1 (Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2) can be used for this scenario since this path switching scenario is switching to target relay UE from source relay UE. This scenario is similar to I2I path switching as with M2I path switching procedure.

Proposal 12. Current measurement reporting event X2, Y2 and Z1 can be reused for I2M path switching procedure to report candidate relay UEs.

In R17, RAN2 introduced the Timer (T420) to manage whether the path switch is successful or not. We think T420 can be reused for MH U2N relay path switch. In U2U relay, the need for a Timer (T400) with double the existing value was recognized to consider the E2E delays caused by the increased number of hops. However, stop condition of T420 is sending RRCReconfigurationComplete message, i.e. when remote UE establishes PC5-RRC connection with relay UE and complete configuring PC5 RLC channel. It means that the increase in hop count has no relation to the value of T420.

Proposal 14. T420 can be reused for path switch to multi-hop indirect path without value expansion.

And for multi-hop indirect to direct path switching, RAN2 can reuse T304 as with legacy.

Proposal 14. T304 can be reused for path switch to direct path without value expansion.

  - . Contents of candidate relay UE(s) report

In Rel-18 U2N relay, (candidate) remote UE sends measurement report to send candidate relay UEs information to the gNB. The measurement report includes following contents;

SL-MeasResultListRelay-r17 ::= SEQUENCE (SIZE (1..maxNrofRelayMeas-r17)) OF SL-MeasResultRelay-r17

SL-MeasResultRelay-r17 ::= SEQUENCE {

cellIdentity-r17			CellAccessRelatedInfo,

sl-RelayUE-Identity-r17	SL-SourceIdentity-r17,

sl-MeasResult-r17	SL-MeasResult-r16,

...,

[[

sl-MeasQuantity-r18	ENUMERATED { sl-rsrp, sd-rsrp }	OPTIONAL,

sl-RelayIndicationMP-r18	SL-RelayIndicationMP-r18		OPTIONAL

]]

}

For the cell access information, the gNB needs to find whether the candidate first relay UE reaches the same gNB. In legacy, remote UE reports the serving cell information of the candidate relay UE to the gNB. And the remote UE considers the serving cell of the relay UE as PCell. Same as the legacy, in multi-hop system, remote UE should report the serving cell information of last relay UE connecting with the candidate first relay UE.

Observation 1. gNB can decide whether the reported first relay UE can reach the gNB since first relay UE contains cellAccessRelatedInfo of serving cell of last relay UE in RRC container included in discovery message.

Proposal 13. The cellIdentity in sl-MeasResultRelay without enhancement can be used for decision whether the candidate first relay UE is connecting with the same gNB.

These information except MP indication is the minimum required information. So, RAN2 can reuse them.

Proposal 14. Candidate UE’s cell info, Candidate relay UE’s source ID, Measurement result and information indicating whether the measurement result is SL-RSRP or SD-RSRP are report to the gNB as with legacy.

We also consider what information gNB needs to select the appropriate PC5 links. We assume the AS layer evaluation metrics that the gNB uses to select optimal PC5 links are PC5 link quality of each PC5 link, number of hops, and QoS profile for the (potential) remote UE. This allows the gNB to select a path with low latency and/or robustness (i.e., less likely to be disconnected) depending on the QoS and radio resource management of the remote UE. According to the current SA2 discussion, the discovery message may include hop counter, ID of last relay UE and accumulated QoS for the PC5 links (i.e. Achievable PDB) in addition to the legacy contents. Therefore, the remote UE knows these metrics. Then, the gNB knows that the reported first relay UE is connecting with the same gNB or not, the number of hops of the PC5 links to which the reported candidate first relay UE belongs, and the QoS profile for the (potential) remote UE. So, for selecting appropriate path (first relay UE), UE should send the information of PC5 link quality of the path the first relay UE belongs. Furthermore, the greater number of hops there are, the more the quality of each hop affects the delay and the efficiency of radio resource utilization. So, we consider that the UE needs to report to the gNB the PC5 link qualities of each hops of the path first relay UE belongs or accumulated QoS for the PC5 links (i.e. Achievable PDB).

Observation 2. In order to select appropriate path, from the perspective of QoS and radio resource management, gNB needs to know PC5 link quality of the PC5 links, number of hops, and QoS profile for the (potential) remote UE.

Observation 3. gNB already knows number of hops, and QoS profile for the (potential) remote UE.

Observation 4. The greater number of hops there are, the more the quality of each hop affects the delay and the efficiency of radio resource utilization.

Proposal 15. Potential remote UE reports to the gNB the PC5 link qualities of each hops of the path first relay UE belongs or accumulated QoS for the PC5 links (i.e. Achievable PDB).

  - . Lossless path switching

In Rel-18, RAN2 discussed how to avoid data stuck at relay UE during path switching. In the discussion, for I2I/I2D path switching, source relay UE sends all of UL data, which the source relay UE receives from remote UE, to the remote UE and source gNB sends all of DL data, which the remote UE haven’t received from the source relay UE, to the target gNB. Then, the source gNB configures the source relay UE to release a configuration for the remote UE afterwards. This scheme can be reused for multi-hop relay.

Proposal 16. RAN2 considers whether existing mechanisms for preventing data loss can be reused.

Conclusion

Proposal 1. RAN2 will take into account the requirement specified in TS22.261 and the solutions and conclusion in TR23.700-03.

Proposal 2. RAN2 considers the candidate first relay UE is a multi/single-hop U2N remote UE in RRC_CONNECTED state.

Proposal 3. Figure1 should be considered as baseline for the M2D (multi-hop indirect to direct) path switching procedure (scenario A).

Proposal 4. Current measurement reporting event X1 and X2 can be reused for M2D (multi-hop indirect to direct) path switching procedure to report candidate relay UEs.

Proposal 5. Figure 2-a and Figure 2-b should be considered as baseline for the M2I (multi-hop indirect to single-hop indirect) path switching procedure (scenario B).

Proposal 6. The target last relay UE and the source last relay UE can be the same UE

Proposal 7. Current measurement reporting event X2, Y2 and Z1 can be reused for M2I path switching procedure to report candidate relay UEs.

Proposal 8. Figure 3 should be considered as baseline for the D2M (direct to multi-hop indirect) path switching procedure (scenario C).

Proposal 9. Current measurement reporting event Y1 and Y2 can be reused for D2M path switching procedure to report candidate relay UEs.

Proposal 10. RAN2 confirm that the source last relay UE can be the target first relay UE and also be a remote UE of the target last relay UE.

Proposal 11. Figure 4-a/4-b should be considered as baseline for the I2M (single-hop indirect to multi-hop indirect) path switching procedure (scenario D).

Proposal 12. Current measurement reporting event X2, Y2 and Z1 can be reused for I2M path switching procedure to report candidate relay UEs.

Observation 1. gNB can decide whether the reported first relay UE can reach the gNB since first relay UE contains cellAccessRelatedInfo of serving cell of last relay UE in RRC container included in discovery message.

Proposal 13. The cellIdentity in sl-MeasResultRelay without enhancement can be used for decision whether the candidate first relay UE is connecting with the same gNB.

Proposal 14. Candidate UE’s cell info, Candidate relay UE’s source ID, Measurement result and information indicating whether the measurement result is SL-RSRP or SD-RSRP are report to the gNB as with legacy.

Observation 2. In order to select appropriate path, from the perspective of QoS and radio resource management, gNB needs to know PC5 link quality of the PC5 links, number of hops, and QoS profile for the (potential) remote UE.

Observation 3. gNB already knows number of hops, and QoS profile for the (potential) remote UE.

Observation 4. The greater number of hops there are, the more the quality of each hop affects the delay and the efficiency of radio resource utilization.

Proposal 15. Potential remote UE reports to the gNB the PC5 link qualities of each hops of the path first relay UE belongs or accumulated QoS for the PC5 links (i.e. Achievable PDB).

Proposal 16. RAN2 considers whether existing mechanisms for preventing data loss can be reused.

Reference

- RP-241609, New WID on NR sidelink multi-hop relay

- TS22.261, Service requirements for the 5G system

- TR23.700-03, Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 3

Appendix

- 7.7	KPIs for UE to network relaying in 5G system

In several scenarios, it can be beneficial to relay communication between one UE and the network via one or more other UEs. The functional requirements related to relaying can be found in clause 6.9.2. Performance requirements for relaying in different scenarios can be found in table 7.7-1.

Table 7.7-1: Key Performance for UE to network relaying

| Scenario | Max. data rate (DL) | Max. data rate (UL) | End-to-end latency (note 7) | Area traffic capacity (DL) | Area traffic capacity (UL) | Area user density | Area | Range of a single hop (note 8) | Estimated number of hops |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| InHome Scenario (note 1) | 1 Gbit/s | 500 Mbit/s | 10 ms | 5 Gbit/s/ home | 2 Gbit/s /home | 50 devices /house | 10 m x 10m – 3 floors | 10 m indoor | 2 to 3 |
| Factory Sensors (note 2) | 100 kbit/s | 5 Mbit/s | 50 ms to 1 s | 1 Gbit/s /factory | 50 Gbit/s /factory | 10000 devices /factory | 100 m x 100 m | 30 m indoor / metallic | 2 to 3 |
| Smart Metering (note 3) | 100 bytes / 15 mins | 100 bytes / 15 mins | 10 s | 200 x 100 bytes / 15 mins /hectare | 200 x 100 bytes / 15 mins /hectare | 200 devices /hectare | 100 m x 100 m | > 100 m indoor / deep indoor | 2 to 5 |
| Containers (note 4) | 100 bytes / 15 mins | 100 bytes / 15 mins | 10 s | 15000 x 100 bytes / 15 mins /ship | 15000 x 100 bytes / 15 mins /ship | 15000 containers /ship | 400 m x 60 m x 40 m | > 100 m indoor / outdoor / metallic | 3 to 9 |
| Freight Wagons | 100 bytes / 15 mins | 100 bytes / 15 mins | 10 s | 200 x 100 bytes / 15 mins /train | 200 x 100 bytes / 15 mins /train | 120 wagons /train | 1 km | > 100 m outdoor / tunnel | 10 to 15 |
| Public Safety (note 5) | 12 Mbit/s | 12 Mbit/s | 30 ms | 20 Mbit/s /building | 40 Mbit/s /building | 30 devices /building | 100 m x 100 m – 3 floors | > 50 m indoor (floor or stairwell) | 2 to 4 |
| Wearables (note 6) | 10 Mbit/s | 10 Mbit/s | 10 ms | 20 Mbit/s per 100 m2 | 20 Mbit/s per 100 m2 | 10 wearables per 100 m2 | 10 m x 10 m | 10 m indoor / outdoor | 1 to 2 |
| NOTE 1: Area traffic capacity is determined by high bandwidth consuming devices (e.g. ultra HD TVs, VR headsets), the number of devices has been calculated assuming a family of 4 members. NOTE 2: Highest data rate assumes audio sensors with sampling rate of 192 kHz and 24 bits sample size. NOTE 3: Three meters (gas, water, electricity) per house, medium density of 50 to 70 houses per hectare. NOTE 4: A large containership with a mix of 20 foot and 40 foot containers is assumed. NOTE 5: A mix of MCPTT, MCVideo, and MCData is assumed. Average 3 devices per firefighter / police officer, of which one video device. Area traffic based on 1080 p, 60 fps is 12 Mbit/s video, with an activity factor of 30% in uplink (30% of devices transmit simultaneously at high bitrate) and 15% in downlink. NOTE 6: Communication for wearables is relayed via a UE. This relay UE can use a further relay UE. NOTE 7: End-to-end latency implies that all hops are included. NOTE 8: 'Metallic' implies an environment with a lot of metal obstructions (e.g. machinery, containers). 'Deep indoor' implies that there can be concrete walls / floors between the devices. NOTE 9: All the values in this table are example values and not strict requirements. |  |  |  |  |  |  |  |  |  |