3GPP TSG-RAN WG2 Meeting #128	R2-2410713

Orlando, USA, Nov. 18th – 22th, 2024

Agenda item:		8.8.4 Support of broadcast service

Title:		Discussion on the support of broadcast service in NR-NTN

Source:		ETRI

Document for:		Discussion

# 1. Introduction

During the 102th meeting of RAN, a new WI on NTN for NR phase 3 was approved, including the following objective:

- Specify signalling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]. - Specify SIB signalling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary between CN and NG-RAN. [RAN3]

The first discussion regarding the new WI took place during 125bisth meeting of RAN WG2 and then, the following agreements were made for the support of broadcast service in NTN.

Agreements: 1. For MBS broadcast service we don’t restrict the work to any satellite constellation type 2. We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS 3. We will cover at least the case where the indicated intended service area covers a portion of a NTN cell 4. The intended service area can cover the area of more than one NTN cells (or portions thereof) 5. Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended service area) 6. At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - Circles (like for TN coverage) - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

During the 126th meeting of RAN WG2, the following agreements were made:

Agreements: 1. For MBS broadcast service, both EFC and EMC are supported. 2. RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. 3. MBS broadcast intended service area is provided via system information 4. For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS 5. When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

During the 127th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). 2. RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions 3. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) 4. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

During the 127bisth meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

In this paper, we would like to provide our views on the highlighted aspects as well as remaining issues on support of broadcast service in NTN.

# 2. Discussion

During the 127th bis meeting of RAN WG2, significant progress was made in supporting MBS broadcast services in NR-NTN, with an agreement to include one or more intended service area IDs in MCCH. The remaining issues that need to be addressed are as follows:

- Whether the list of the intended service areas (and related IDs) is also included in MCCH or provided into a new or existing SIB.

- The impact of the intended service area concept on legacy MBS service continuity mechanisms.

- How to associate the service area information with MBS sessions.

2.1	How to provide the list of the intended service areas: MCCH vs a new or existing SIB

During the 127th bis meeting, RAN WG2 considered two potential working assumptions for providing the list of intended service areas to UEs: using MCCH or a new/existing SIB.

- WA A: A new or existing SIB to provide the list of intended service areas associated with the service area IDs included in MCCH.

  - Pros: Directly allows the UE to skip MCCH re-acquisition when it is outside the intended service area of any relevant broadcast service.

  - Cons: Signal overhead due to duplicated signalling.

- WA B: MCCH to include both the intended service area IDs and the list of intended service areas.

  - Pros: Reduces signalling overhead.

  - Cons: Requires an additional mechanism to allow the UE to skip MCCH re-acquisition when it is outside the intended service area of any relevant broadcast service.

ETRI believes that both WA A and WA B can support the UE in skipping MCCH re-acquisition when it is outside the intended service area of any relevant broadcast service. If WA A is adopted, the UE could avoid unnecessary (re)acquisition of MBSBroadcastConfiguration messages when it is outside the intended service areas after receiving a new or existing SIB that conveys the list of intended service areas. Conversely, if WA B is adopted, the UE can store this information—potentially received through a previous MCCH or USD message—and then determine whether it needs to (re)acquire the latest MCCH information based on its current location and the stored intended service areas. Therefore, ETRI believes that the issue of skipping MCCH re-acquisition may not be a critical factor in deciding between these options.

Observation 1: Both WA A and WA B can enable the UE to skip MCCH re-acquisition when it is outside the intended service area of any relevant broadcast service.

However, if WA A is adopted, ETRI believes that SIB20 may not be suitable due to its limited payload size (e.g., 2976 bits), considering that a large number of service areas could be defined within a wide satellite beam coverage area and that each service area, formatted as circles or polygons, may require tens to hundreds of bits. Therefore, if WA A is adopted, ETRI prefers defining a new SIB to provide the list of intended service areas to the UE, similar to SIB25.

Observation 2: If WA A is adopted, an existing SIB (e.g., SIB20) may be unsuitable due to its limited payload size.

Proposal 1: If WA A is adopted, RAN2 should define a new SIB to provide the list of intended service areas to the UE.

2.2	Service Continuity

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled:

- 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20;

- 2) Either:

  - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.517 [20], or

  - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or

  - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service.

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service, the UE may consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session as specified in TS 38.300 [2], as long as SIB1 scheduling information of the cell contains SIB20 on the MBS frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell.

Considering that SIB21 contains the mapping relationship between the current and/or neighbouring carrier frequencies and MBS FSAI, this legacy mechanism for MBS service continuity assumes that the MBS service area is mapped onto one or more cells. However, in NTN systems, this may not work effectively because the UE may not consider that frequency as the highest priority if the MBS intended service area(s) of that frequency do not include the intended service area of the MBS session that the UE is receiving or interested in, even if the above conditions 1 and 2 are met.

Therefore, ETRI believes that enhancements to the legacy mechanism for MBS service continuity are necessary, including the following:

- To define a new mapping relationship between the current and/or neighbouring carrier frequencies and MBS intended service area IDs, and include the following information in SIB21:

  - Carrier frequency list (already existing in SIB21)

  - MBS intended service area ID list per each carrier frequency

- To update existing conditions for UE to perform MBS broadcast service cell reselection based on MBS broadcast intended service area information as followings.

  - If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the three following conditions are fulfilled:

    - 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20;

    - 2) Either:

      - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.517 [20], or

      - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or

      - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service.

    - 3) one or more MBS broadcast intended service area ID(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS broadcast intended service area ID(s) is also indicated for this MBS broadcast service session.

Observation 3: In NTN systems, the legacy MBS service continuity mechanism may not work effectively due to the lack of intended service area prioritization.

Proposal 2: RAN2 to consider the following enhancements to the legacy mechanism for MBS service continuity.

- To include the intended service areas of each MBS broadcast session for neighbouring cells in the MCCH or in a new/existing SIB that may convey the list of intended service areas.

- To define a new mapping relationship between carrier frequencies and MBS intended service area IDs in SIB21

- To update the UE's cell reselection behaviour based on MBS intended service area information.

2.3	Association between MBS broadcast intended service areas and MBS sessions

The location-dependent MBS service enables the distribution of different content data to different MBS service areas. For the support of location-dependent broadcast services, similar to the legacy MBS mechanism, the MBS broadcast intended service area ID can be used in combination with the MBS session ID to uniquely identify the service area-specific part of the MBS service within 5GC. Therefore, ETRI believes that for association with MBS sessions, the intended service area IDs should be associated with MBS session IDs with the following possibilities:

- Option 1: One intended service area ID mapped to multiple MBS session IDs.

- Option 2: Multiple intended service area IDs mapped to one MBS session ID.

Considering flexibility for the configuration of independent intended service area per each MBS session, in our view, Option 2 is preferred.

Observation 4: The intended service area ID should be associated with the MBS session ID to uniquely identify service areas within the 5GC.

Proposal 3: RAN2 to support N-to-1 mapping between multiple intended service area IDs and one MBS session ID.

# 3. Conclusions

Observation 1: Both WA A and WA B can enable the UE to skip MCCH re-acquisition when it is outside the intended service area of any relevant broadcast service.

Observation 2: If WA A is adopted, an existing SIB (e.g., SIB20) may be unsuitable due to its limited payload size.

Proposal 1: If WA A is adopted, RAN2 should define a new SIB to provide the list of intended service areas to the UE.

Observation 3: In NTN systems, the legacy MBS service continuity mechanism may not work effectively due to the lack of intended service area prioritization.

Proposal 2: RAN2 to consider the following enhancements to the legacy mechanism for MBS service continuity.

- To include the intended service areas of each MBS broadcast session for neighbouring cells in the MCCH or in a new/existing SIB that may convey the list of intended service areas.

- To define a new mapping relationship between carrier frequencies and MBS intended service area IDs in SIB21

- To update the UE's cell reselection behaviour based on MBS intended service area information.

Observation 4: The intended service area ID should be associated with the MBS session ID to uniquely identify service areas within the 5GC.

Proposal 3: RAN2 to support N-to-1 mapping between multiple intended service area IDs and one MBS session ID.